3GPP TSG-RAN WG2 Meeting #130		R2-2503975

St.Julians, Malta, May 19 – 23, 2025

Title: 	XR Rate control details

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.7.6

Document for:	Discussion and decision

# Introduction

At RAN2#129-bis it was agreed that the XR rate control mechanism uses the per QoS flow-based indication in the MAC CE. In this contribution we look at the remaining open issues for the rate control mechanism.

# Discussion

RAN2 made the following agreements for XR rate control:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Based on the above agreements, the main open issue is how to indicate the QoS-flow based recommendation in the MAC CE. It should be noted that the QoS flows are identified by the QFI and a given QFI is unique within a PDU session. So, in order to uniquely identify the QoS flow mapped to a given DRB, the MAC CE should uniquely identify the QoS flow within a given PDU session.

There are multiple options to achieve this.

Option 1: Indicate PDU session ID + QFI in the MAC CE: This will require a total of 14 bits (8 bits for PDU session ID and 6 bits for QFI) + 8 bits for the recommended bit rate – i.e. total of 3 octets.

Option 2: Indicate DRB ID + QFI in the MAC CE: This will uniquely identify the QFI since packets from different PDU sessions are anyway mapped to different DRBs. This will require a total of 11 bits (5 bit DRB ID + 6 bit QFI) and needs a 3 octet MAC CE payload – see below.

Option 3: Indicate just a QFI: This option assumes that at any given point in time, there is only one PDU session that has QoS flows that are subject to rate control. RRC signalling should indicate which PDU session has QoS flows subject to rate control (if there are more than 1 PDU sessions). This option requires just 6 bit QFI to be indicated and hence a 2 octet MAC CE – see below.

Options 1 and 2 are equally flexible but option 2 has lower overhead. So, between these two options, option 2 should hence be preferred clearly. Option 3 on the other hand has the least amount of overhead, although it comes with the restriction that only one PDU session can support QoS flows that are subject to rate control. This may be a reasonable assumption in reality though. So, both options 2 and 3 can be considered further. Between these two options, option 3 has the smallest overhead. So, we propose to adopt this option.

Based on the above, the following proposal is made:

Proposal 1: The MAC CE for XR rate control includes just the QFI indicator as shown in the figure below:

Proposal 2: RRC signalling indicates which PDU session contains the QoS flows subject to rate control (if there is more than one PDU session)

# Conclusion

The following proposal is made in this contribution:

Proposal 1 (MAC-05, 06, 07, 11, 12): The MAC CE for XR rate control includes just the QFI indicator as shown in the figure below (with the assumption that only one PDU session has QoS flows subject to rate control):

Proposal 2 (MAC-05, 06, 07, 09, 11, 12): RRC signalling indicates which PDU session contains the QoS flows subject to rate control (if there is more than one PDU session)

# References