3GPP TSG-RAN WG2 Meeting #130  	                            R2-2504055

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.4

Source: 	Sony

Title:	RA-RNTI issue and RACH adaptations

Document for:	Discussion

# Introduction

In RAN2#129 meeting, RAN2 reached the agreements on PRACH adaptations as following:

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

In this contribution, we discuss some remaining issues of RA-RNTI issue and RACH adaptation resources configurations.

# Discussion

2.1 RACH adaptation resources configuration

It was agreed in RAN1#116b that the time domain RACH adaptation is based on additional PRACH resources for NES -capable UEs in addition to PRACH resources for legacy UEs. RAN1 has further agreed to use DCI-based adaptation to trigger the additional PRACH resource. Then it is up to RAN2 to decide how the network will configure the additional PRACH resources and it is easy to conclude that the additional PRACH resources should be included in a SIB and further details e.g., to define a new SIB or to be included in SIB1 or any other existing SIB can be decided later.

Proposal 1: The additional PRACH resources for RACH adaptation should be included in a SIB.

2.1 RA-RNTI issue

For additional PRACH resources for NES -capable UEs, how to handle the case where the legacy RACH occasion (RO) overlap with additional ROs should be discussed RAN2 as RA-RNTI formula is in RAN2 spec.

The legacy RA-RNTI/MsgB-RNTI are calculated as follows (TS 38.321):

The RA-RNTI/ MSGB-RNTI associated with the PRACH occasion in which the Random-Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

From our perspective, it is necessary to have a standardized solution as network implementations cannot solve the RNTI issue without additional complexity and restrictions on the configurable RACH resources. For example, allocating different preamble list to the legacy RACH configuration and additional RACH configuration would limit the available resources, hence impacts the RACH capacity.

A simple solution would make a significant difference in terms of flexibility of configuring the RACH resources, reducing the complexity, increasing RACH resources and minimising the latency due to RNTI collisions. For example, for potential solution, there are a lot of unused RNTIs in the current equation based on the s_id depending on the RACH duration on each occasion. If there is a shift of s_id into another unused symbol index (0 ≤ s_id < 14), it will increase the number of available RNTIs as well as avoiding RNTI collisions, without increasing RNTI space. The shift/offset can be signalled in SIB1 as a part of PRACH resource configuration for NES -capable UEs. Note that fixed value (e.g., s_id =1) may not eliminate RNTI collisions in all cases, so to be future proof it may be desirable the network to configure the offset value.

Proposal 2: To avoid RNTI collision/confusion, simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of additional PRACH resources for RACH adaptation for NES -capable UEs.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: The additional PRACH resources for RACH adaptation should be included in a SIB.

Proposal 2: To avoid RNTI collision/confusion, simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of additional PRACH resources for RACH adaptation for NES -capable UEs.