3GPP TSG-RAN WG2 Meeting #130  	                            R2-2504061

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.2

Source: 	Sony

Title:	Remaining details on OD-SSB MAC CE

Document for:	Discussion

# Introduction

In RAN2#129bis meeting, on-demand SSB Scell MAC CE was discussed, and the agreements are as follows.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

In this contribution, we discuss the remaining issue on MAC CE design, i.e., how to support both explicit and implicit OD-SSB deactivations with single MAC CE format.

# Discussion

In RAN1#120bis meeting, RAN1 further agreed more details on deactivation of OD-SSB.

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

In RAN2#129bis meeting, it has agreed to design a single MAC CE for both implicit and explicit deactivation. The co-existence of implicit and explicit deactivation will indeed bring certain ambiguity from UE’s point of view, but a well-defined UE/Network behaviour would help without changing the already agreed MAC CE format e.g., no need to introduce any additional information in the MAC CE to further increase the signalling overhead. We foresee following three options as following:

Option 1: Network configures either implicit or explicit deactivation should be used.

The configuration could be per UE (applicable to all the Scells) or per Scell. From network point of view, if network wants to configure implicit deactivation, then it should configure od-ssb-nrofBurst for that specific Scell otherwise should not configure od-ssb-nrofBurst. From UE’s point of view, UE will follow the configuration of od-ssb-nrofBurst for the corresponding OD-SSB Scell. If od-ssb-nrofBurst is configured, UE will ignore any future MAC CE containing the deactivation/activation indication for the corresponding OD-SSB Scell and follow the implicit deactivation configuration after it is activated. If od-ssb-nrofBurst is not configured, UE will follow the deactivation indication in the MAC CE for the corresponding OD-SSB Scell after it is activated.

Option 2: Network configures both implicit and explicit deactivation can be used but with some pre-defined rules.

The rules for example can be: explicit deactivation always overwrites the implicit deactivation. Then with certain pre-defined UE behaviour, implicit and explicit deactivation can work simultaneously at the same time with single MAC CE. To be specific, if od-ssb-nrofBurst is configured for a certain OD-SSB Scell, after this OD-SSB Scell is activated, it will follow the implicit configuration until it receives a MAC CE with the deactivation indication is set for this OD-SSB Scell. If od-ssb-nrofBurst is not configured for a certain OD-SSB Scell, then only the MAC CE with the deactivation indication being set can deactivate the corresponding Scell.

Option 3: After network sends MAC CE to activate a certain Scell, network will not send any other MAC CEs if there is any implicit deactivation running for that UE.

The reason is that we assume that a good scheduler implementation should be able to plan ahead for this duration (in the worst case is about 5ms) and not cause conflicting configurations for the UE.

Therefore, we have the proposal as following:

Proposal 1: MAC-CE design shall address both implicit and explicit deactivation scenarios and suggest RAN2 to discuss and select one of the options as below.

Option 1: Network configures either implicit or explicit deactivation should be used.

Option 2: Network configures both implicit and explicit deactivation can be used but with some pre-defined rules.

Option 3: After network sends MAC CE to activate a certain SCell, network will not send any other MAC CEs to activate/deactivate SCells if there is any implicit deactivation running for that UE.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: MAC-CE design shall address both implicit and explicit deactivation scenarios and suggest RAN2 to discuss and select one of the options as below.

Option 1: Network configures either implicit or explicit deactivation should be used.

Option 2: Network configures both implicit and explicit deactivation can be used but with some pre-defined rules.

Option 3: After network sends MAC CE to activate a certain SCell, network will not send any other MAC CEs to activate/deactivate SCells if there is any implicit deactivation running for that UE.