3GPP TSG-RAN WG2 Meeting #130	R2-2504065

St. Julians, Malta, 19 - 23 May 2025

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon, Turkcell

Title:	Further consideration on UL capacity enhancement

Document for: Discussion and Decision

# 1	Introduction

According to the RAN plenary #105 meeting, the following objectives on IoT-NTN [1] were agreed:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In the last meeting, RAN2 has made the following agreements:

Part 1

- We tentatively refer to the new procedure as CB-msg3-EDT in the specs (can come back)

- Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

- We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

- For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

- We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

- We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

- If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

- The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

- The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

- RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

- For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

- For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

- We will not support TDD related parameters.

- For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

- For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

- For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

- For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

- For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

- For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Part 2:

- For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

- The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

- The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

- FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

Part 3:

- The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

- The Msg4 monitoring window configuration (e.g. length) is CE level specific

- RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

- RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

- RAN2 also intends to support CB-msg3-EDT for MT cases

- The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

- Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

- The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

- Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

- Backoff information could be included in CB-Msg4.

- L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

- HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In this contribution, we will further discuss the details of CB-Msg3 EDT mechanisms.

# 2	Discussion

## 2.1 CB-Msg3 EDT Fallback

In previous meetings, RAN2 has agreed:

- Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

One issue to discuss is how UE falls back when the maximum re-attempt number has been reached, i.e., whether the UE tries another CE level or the UE falls back to 4-step random access based EDT. In our understanding, falling back across different CE levels will make the whole mechanism very complex but may not have benefit at all. Because the contention resolution may still fail in a different CE level due to collision and the UE has to finally fall back to the legacy EDT.

Proposal 1: (MAC-15) When the max re-attempt number has been reached, the UE directly falls back to 4-step random access based EDT.

In addition, since the 4-step random access EDT is initiated by RRC layer, the MAC layer needs to notify the RRC layer when CB-msg3 EDT fails to perform the fallback operation.

Proposal 2: (MAC-15) When the max re-attempt number has been reached, the MAC layer needs to notify the RRC layer.

## 2.2 CB-RNTI calculation

To complete the contention resolution, the UE needs the RNTI to monitor the PDCCH for MSG4 scheduling. In the previous meetings, RAN2 had agreement to use CB-RNTI to scramble Msg3 and monitor Msg4. The CB-RNTI is derived from the transmit resource within the transmission window. However, the detail of calculation of CB-RNTI is still FFS.

In legacy RACH procedure, UE determines RA-RNTI according to selected PRACH resources.

36.321 5.1.4 Random Access Response reception The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI= 1 + t_id + 10*f_id where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10)) where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0. For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

RNTI used for Msg4 in CB-MSG3 EDT can use the similar mechanism as RA-RNTI. For CB-Msg3 EDT, we need to specify which PUSCH occasion to use for RNTI calculation within the transmission window. The simplest method is to just use the first PUSCH occasion.

Proposal 3: (MAC-2) The calculation of CB-RNTI is based on the first PUSCH occasion within the Msg3 transmission window and can reuse the RA-RNTI calculation formula.

## 2.3 HARQ feedback for MSG4

In last meeting, RAN2 has agreed that HARQ feedback is adopted to acknowledge Msg4. Since one CB-Msg4 can target multiple UEs simultaneously, there could be multiple HARQ feedbacks for one CB-Msg4. How the PUSCH resources are provided to support multiple HARQ feedbacks is FFS.

- HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In NR 2-step RA, successRAR for MSGB includes HARQ feedback resources. Similarly, for Msg4 multiplexing of CB-MSG3 EDT, HARQ feedback resources can be included in the Msg4 payload.

Proposal 4: (MAC-14) HARQ feedback resources can be included in the Msg4 payload.

## 2.4 MSG4 monitoring for CB-Msg3 EDT

In the last meeting, RAN2 has agreed that the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. It remains FFS on the support of NW configuration that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. This allows the early termination of the feedback monitoring and possible cancelation of replica transmission if the previous replica is already successfully transmitted.

RAN2#129bis agreement: - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not) - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

Some Companies think the issue is the Msg4 monitoring window overlaps with replica may cause conflict between UL and DL in Half Duplex. First of all, we think UL/DL collision can be avoided by NW and UE implementation:

NW implementation: When the DSA window configured by NW is less than the minimum UE-eNB RTT (as shown in Fig.1), uplink and downlink conflicts do not occur.

UE implementation: When the interval between first replica and last replica selected by the UE is less than the UE-eNB RTT (as shown in Fig.2), uplink and downlink conflicts do not occur.

Fig. 1 Msg4 monitoring window starts with each DSA window (NW implementation)

Fig. 2 Msg4 monitoring window starts with each DSA window (UE implementation)

In addition to the above cases, NW can always consider about the possible collision and avoid it when transmitting the feedback and UE can chooses the PUSCH occasions without any overlapping with the reception window by implementation.

Proposal 5: (MAC-10) RAN2 confirms that NW can configure that the Msg4 monitoring window starts/restarts in the subframe containing the last (N)PUSCH repetition of each replica plus UE-eNB RTT. The UL/DL collision can be avoided by implementation.

# 3	Conclusion

This contribution analyses the UL capacity enhancement, and proposes the following:

CB-Msg3 EDT Fallback

Proposal 1: (MAC-15) When the max re-attempt number has been reached, the UE directly falls back to 4-step random access based EDT.

Proposal 2: (MAC-15) When the max re-attempt number has been reached, the MAC layer needs to notify the RRC layer.

CB-RNTI calculation

Proposal 3: (MAC-2) The calculation of CB-RNTI is based on the first PUSCH occasion within the Msg3 transmission window and can reuse the RA-RNTI calculation formula.

HARQ feedback for MSG4

Proposal 4: (MAC-14) HARQ feedback resources can be included in the Msg4 payload.

MSG4 monitoring for CB-Msg3 EDT

Proposal 5: (MAC-10) RAN2 confirms that NW can configure that the Msg4 monitoring window starts/restarts in the subframe containing the last (N)PUSCH repetition of each replica plus UE-eNB RTT. The UL/DL collision can be avoided by implementation.

# 4	References

- RP-242397, WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3