3GPP TSG-RAN WG2 Meeting #130	R2-2504070

St. Julians, Malta, 19 - 23 May 2025

Agenda Item:	8.17

Source: 	Huawei, HiSilicon

Title: 	Discussion on RAN2 impacts of IoT-NTN TDD

Document for:	Discussion and Decision

- Introduction

In RAN#105, a new work item was approved to introduce NB-IoT TDD mode exclusively for 1616-1626.5 MHz Mobile Satellite Services MSS band [1]. During the RAN1#119 meeting, some agreements were made for the TDD pattern and the corresponding frame structure [2], as listed below.

Agreement： Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

The WID was revised in RAN#106 [3] to reflect RAN1 progress and narrow down the scope of the solutions.

In RAN2#129b meeting [4], the following agreements were reached for RAN2 impacts of this new WID.

Agreements: 1. Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. 2. In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell. 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes 4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe 6. For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

In this paper, we further discuss the RAN2 impacts of TDD operation for NB-IoT NTN.

- Discussion

  - Impacts on SI

In RAN1#120, the impacts on the system information other than SIB1-NB were left to RAN2 according to the following conclusion:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

In our understanding, SI messages (other than SIB1-NB which is separately discussed in RAN1) can only be transmitted on NB-IoT DL subframes, so that no SI messages need to be dropped due to the mapping on the non-DL subframes. This is beneficial for the system performance.

Proposal 1: NPDSCH for SI messages other than SIB1-NB should be mapped on NB-IoT DL subframes.

In legacy NB-IoT system, the SI-window of n th SI message configured in the SchedulingInfoList-NB-r13 of SIB1-NB starts at the subframe#0 in the radio frame satisfies the following equation.

(H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset

T is the periodicity corresponding to the SI message configured by si-Periodicity-r13 with the value range of {64, 128, 256, 512, 1024, 2048, 4096} radio frames. x=n-1∙w is related to the order of SI-message n and the SI-window length w configured by si-WindowLength-r13 with the value range of {160, 320, 480, 640, 960, 1280, 1600}ms. In the SI window, the TB of a SI-message is mapped to 2 or 8 consecutive valid NB-IoT DL subframes (provided by downlinkBitmap-r13) in the 1st radio frame of every {2, 4, 8, 16} radio frames (provided by si-RepetitionPattern) from the start of SI window. If the number of valid NB-IoT DL subframe is not enough, the TB can be extended the following radio frames not exceeding the SI-window length.

It is observed that the starting radio frame of each repetition are derived based on physical radio frame index. When the si-RepetitionPattern is configured with values other than “every16thRF”, the valid NB-IoT DL subframes of consecutive NPDSCH repetitions in a SI window may overlap due to small portion of valid NB-IoT DL subframes per 90ms. In such case, UE should skip the subsequent reception of SI NPDSCH repetitions.

Proposal 2: For the SI reception within an SI-window, UE should prioritize the previous SI reception in case there is collision between the previous SI repetitions and the subsequent SI reception.

  - Impact on Measurement

For a UE in NB-IoT NTN TDD cell, it cannot perform any service outside the available UL and DL subframes in the serving cell, due to the new frame structure. This period (outside the available UL and DL subframes) can be regarded as entering the dormant period, which is very beneficial for UE power saving. During this period, some operations such as GNSS measurement and inter-frequency neighboring cell measurement can be performed, so that normal data transmission during the serving cell’s available UL and DL subframes  is not affected.

Proposal 3: Measurement such as GNSS measurement and inter-frequency neighboring cell measurement can be performed during the period outside the serving cell's available UL and DL subframes to avoid interfering the normal data transmission during the serving cell’s available UL and DL subframes .

- Conclusion

The observations and proposals made in this contribution are summarized below:

Impacts on SI

Proposal 1: NPDSCH for SI messages other than SIB1-NB should be mapped on NB-IoT DL subframes.

Proposal 2: For the SI reception within an SI-window, UE should prioritize the previous SI reception in case there is collision between the previous SI repetitions and the subsequent SI reception.

Impact on Measurement

Proposal 3: Measurement such as GNSS measurement and inter-frequency neighboring cell measurement can be performed during the period outside the serving cell's available UL and DL subframes to avoid interfering the normal data transmission during the serving cell’s available UL and DL subframes.

- References

RP-242415, New WID on introduction of IoT-NTN TDD mode

Chair notes RAN1#119 eom1

RP‑243293, Revised WID on introduction of IoT-NTN TDD mode

R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN