3GPP TSG-RAN WG2 Meeting #130					           R2-2504108

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:		8.3.2

Source:			Huawei, HiSilicon

Title:			Discussion on functionality management for UE sided model

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#129bis meeting, agreements related to functionality management were reached as follows:

1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2. Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) 3. Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity 4. The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference 5. For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details 6. For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details 7. Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP. 8. The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework 9. For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message. 10. FFS whether association ID is required and should provide motivation on per use case bases 11. Model transfer will not be discussed in this study item

In this Tdoc, we provide our views on functionality management for UE sided model required to support the AIML aided mobility for the use case that were studied by RAN2 thus far. The document is divided into five parts which focus on configuration issues (section 2.1), reporting related issues (section 2.2), associated ID issues (section 2.3) and applicability issues (section 2.4).

# 2   Discussion

## 2.1 Configuration of different use cases and sub-use cases

Predicted Quantity

In RAN2#129bis meeting, it was agreed that the baseline quantity which is considered in AI aided mobility framework is RSRP. The decision was made mainly because only RSRP prediction has been studied by RAN2 in simulations. Our understanding is that RAN4 has also only considered RSRP quantity in their study. Considering other quantities may have further RAN4 impacts, and SINR is more related with RLF prediction which is out of Rel-19 scope, it makes sense to limit the scope of the future work to RSRP only. On the other hand, from RAN2 signalling point of view, introducing such limitation does not seem to bring any gains in terms of less workload, so whether to consider it in normative phase should be decided purely by RAN4.

Observation 1: RAN4 has not studied other quantities besides RSRP so far.

Observation 2: From RAN2 point of view, there is no additional work needed to support prediction for quantities other than RSRP.

Proposal 1: Whether to include SINR or RSRQ in normative phase should be decided by RAN4, from RAN2 signalling point of view, all quantities can be supported with no additional work.

Configuration vs. UE implementation

During the simulations, companies tested different kinds of models with different types of inputs etc. RAN2 needs to discuss whether such model related issues need to be specified or should be left up to UE implementation, taking several examples as below:

- Cluster-based prediction vs. cell-based prediction

- L1 filtering option

- Utilized RRM sub-use case

- OW length (for temporal domain prediction)

- Direct prediction vs. indirect prediction (only for measurement event prediction)

Companies had very differing views on which kind of model types, inputs etc. are best. From our perspective, what truly matters is the performance of the model. As long as proper prediction accuracy can be achieved, choices as above can be left to UE vendor discretion and we currently do not see the need to specify anything related to this.

Proposal 2: Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

On the other hand, some parameters are needed to be configured for UE performing prediction, as they are not only related to prediction accuracy but also related to NW performance/configuration. For example, different measurement objects (e.g., different cells) and different reporting configurations (e.g., different measurement events) may have different prediction accuracy requirements, so the NW should have the flexibility to control what measurement object(s) or what reporting configuration(s) can be predicted. Taking another example, PW/MRRT/MRRS/MG have impacts on both NW performance (e.g., resource allocation, HO performance) and prediction accuracy, thus such parameters should be configurable by the NW.

Proposal 3: The following items should be configurable by the gNB for prediction:

- Measurement framework parameters: measurement object, measurement report, measurement ID

- Prediction related parameters: PW length, MRRT, MRRS; FFS how exactly these are configured, e.g. explicitly or via other parameters (e.g. using SMTC for MRRT or set A/B beams for MRRS)

## 2.2 Reporting aspects for different use cases and sub-use cases

Prediction indication

Regarding the predicted RRM measurement and event reporting, one thing worth discussing is whether the UE needs to indicate whether the reported result is predicted or not. For RRM measurement prediction, one may argue that the gNB already knows implicitly whether a reported results are predicted since it configures the measurement configurations (e.g., measurement ID, measurement object, reporting configuration). However, this also depends on whether a UE can be configured with AI and non-AI RRM measurement configuration simultaneously and whether a UE can autonomously fallback to non-AI configuration in some cases (e.g. inapplicability or performance degradation). Another aspect is how the predicted results are supposed to be utilized by the network which is related to HO Option 1 and 2, as studied by RAN2. In Option 1, the predicted event is only used by the network to prepare the HO in advance, but the actual HO is executed only when the actual event occurs. In this case, it will require the UE to inform somehow the earlier event is predicted while the latter one is an actual event.

Observation 3: For RRM measurement prediction, whether an indication about the reported result being either an actual measurement or predicted measurement depends on whether UE can be configured with AI and non-AI RRM measurement configurations at the same time and also whether UE autonomous fallback is allowed.

Observation 4: For the event prediction, HO Option 1 requires the UE to report the predicted event to allow the network to prepare the HO in advance to ensure timely HO when the actual event occurs. In this case, it will require the UE to inform the NW that the earlier event is predicted while the latter is actual event.

Proposal 4: RAN2 to discuss whether the indication for whether the reported result (i.e. measurement or event) is a predicted or an actual measurement should be supported for different use cases.

What RRM measurements are reported

RAN2 agreed the following:

5. For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details 6. For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details

When it comes to periodic reporting of measurements, the situation seems rather clear, i.e. the UE reports predicted measurements according to the configured periodicity. When it comes to event triggered RRM measurement reporting, the question arises on which predicted measurements the UE should report. In legacy event triggered measurement reporting, the measurement result of only one moment is reported, i.e., the time when the TTT ends. However, for temporal domain prediction, the PW may be longer than TTT. So, predicted measurement result corresponding to the time when the PW ends may also be helpful for HO determination. Thus, it is suggested RAN2 to consider reporting of other measurement result(s), besides the one corresponding to the time when TTT ends, e.g., predicted measurement result corresponding to the time when PW ends. This is applicable to both event triggered reporting of predicted RRM measurements and measurement reporting during predicted event reporting.

Proposal 5: For both event triggered reporting of predicted RRM measurements and measurement reporting during predicted event reporting, it should be possible for the UE to report predicted measurement results corresponding to multiple time instances, e.g. on top of the result corresponding to TTT expiry time, the one related to PW end.

When to perform inference

In RAN2#129bis meeting, an FFS was captured on when the UE can perform inference once configured with predictions. During online discussion, some companies proposed that the UE can perform inference when the event entry condition is met based on actual measurement result, and some other companies proposed that the UE can perform inference when signal level drops below a certain threshold. We believe that when exactly the UE performs inference depends, e.g. on the use case which is configured:

- In case of RRM measurement prediction with periodic reporting, it seems obvious the UE needs to perform inference continuously.

- In case of event triggered predicted RRM measurement reporting, the UE only needs to perform inference once an event that triggers prediction reporting is met.

- In case of measurement event prediction, when to perform inference depends on the PW length and TTT, e.g. in case the PW length is longer than TTT, it seems the UE should perform inference continuously. On the other hand, in case PW length is shorter or equal to TTT, then the UE may only start to perform inference once the entering condition of the event is met for a certain time (e.g. PW length-TTT) to verify that the event needs to be reported.

However, different UE vendors may have different implementation logic and detailed solution can be up to UE implementation.

Observation 5: The exact time when the UE needs to perform inference depends on certain use case and reporting types.

Observation 6: UE vendors may have different strategies on when to perform inference.

Proposal 6:  The exact time of triggering inference can be up to UE implementation as long as the UE follows the inference configuration provided from the network, e.g. PW length, time instances to be reported etc.

## 2.3 Associated ID

Need of associated ID

In RAN2#129bis meeting, there is an FFS whether associated ID is required. One of the scenarios where it may be useful that was mentioned during the discussions was network energy saving use case. Namely, at times some of the cells in the network may be turned off for NW energy saving. A model trained while these cells are turned off may not be applicable to the times when these cells are turned on as the signals that UE detects and potentially uses for predictions are different. In this case, it indeed seems to make sense to apply associated ID to align training procedure and inference procedure. On the other hand, as shown in the generalization study, the performance of the model was not impacted by factors such as environment type (UMa vs. UMi), ISD, gNB transmit power etc. This suggests that the model may be generalizable over various cell configurations options. Hence, we believe that even though associated ID seems to be useful in some scenarios (such as NW energy saving), it is not always required (e.g. in case energy saving schemes are not deployed) and its configuration for training and inference should be optional.

Observation 7: Associated ID is needed for some scenarios, e.g., when the NW may turn some cell(s) off for NW energy saving.

Observation 8: The model prediction accuracy was very good in the studied generalization scenarios.

Proposal 7: Associated ID should be optionally configurable for training and/or inference (i.e. it is not mandatorily required for all training/inference configurations).

Granularity of associated ID

For BM, the associated ID is valid at cell-specific level, and there can be also different associated IDs depending on set A and set B configuration.

RAN1 agreements for associated ID of BM: - For BM it is only cell-specific, but there can be also different associated IDs depending on set A and set B configuration - Agreement from RAN1: Agreement For UE-sided model, for configuring the resource for data collection purpose, support • CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. o One CSI-ResourceConfigId is configured for Set A. o One CSI-ResourceConfigId is configured for Set B. o Note: UE performs measurement on all resources o One or two associated IDs can be configured in CSI-ReportConfig  When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,  Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B • FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

For BM use case, associated ID is related mainly to beam characteristics and that is why it can be configured differently for set A and set B beams. Furthermore, it was agreed that associated ID is cell-specific which allows to avoid the need of associated ID coordination between cells and also allows not to disclose details about the network implementation. In case of mobility use case, similar argumentation should be considered as the details of network implementation should still be hidden from the UE and the predictions may also be based on beams. On the other hand, some companies mentioned that since mobility involves multiple cells (i.e. at least a serving cell and one neighbouring cell), we may need to reconsider associated ID design. However, it should be noted that associated ID is supposed to be used as a way to abstract a certain network state for a certain cell which means it should still be cell-specific. On the other hand, model’s applicability may depend not only on a single associated ID, but on multiple associated ID’s, e.g. in case of inter-frequency prediction, the UE may need to verify associated ID of both the measured cell and predicted cell(s).

Based on the above, we believe the mechanism adopted for BM should be reused as a baseline for mobility, i.e., associated ID can be assumed to be cell-specific unless a requirement to revisit this assumption is identified.

Proposal 8: As a baseline, associated ID is cell-specific, similarly as for AIML for BM use case.

## 2.4 Applicability reporting

Figure 1: LCM procedure agreed in AIML interface.

The LCM procedure for BM is shown in Fig.1 above. For applicability reporting, details related to option A and option B are still under discussion in RAN2. Most likely, we will be able to reuse the mechanism to a large extent. Perhaps what needs to be understood at the moment regarding applicability reporting for mobility is the following:

- Should we allow the NW to configure the UE for inference before receiving applicability reporting?

- What parameters are needed for the UE to determine applicability for various use cases?

When it comes to the first point, there seem to be some use cases where the network may try to configure the UE with inference configuration blindly (i.e. before receiving applicability report), e.g. temporal domain Case A or inter-frequency measurements. On the other hand, what measurements need to be configured at a certain time for the UE depend on many factors, e.g. whether the UE is moving, what is the cell/frequency load, where in the cell the UE is located etc. Hence the network may want to know applicability of some extra functionalities in advance, without having to configure them for inference. This leads us to a conclusion that both option A and option B type of applicability reporting should be supported for mobility.

Proposal 9: Both option A and option B type of applicability reporting should be supported for mobility prediction configurations.

When it comes to parameters that are needed for the UE to determine applicability (related to option B), we think at least the following prediction related parameters have an impact on its determination (depending on the use case): PW length, MRRT, MRRS; Taking an example, the UE may have different applicability in different PW configuration. In addition, in some cases also associated ID may be required.

Proposal 10: For option B, at least the following parameters are needed for applicability determination (depending on the use case): PW length, MRRT, MRRS, and optionally associated ID.

Another question related to functionality applicability is the UE behavior once a certain functionality becomes inapplicable. This discussion is very similar to the case where the functionality does not meet minimum performance requirements which we discuss in detail in our paper on performance monitoring in [1]. We think the similar logic applies for inapplicability determination and therefore we propose the following:

Proposal 11: As a baseline, functionality management decisions are made at the network side based on the applicability reports received from the UE.

Proposal 12: RAN2 to discuss whether for temporal domain Case A prediction, UE can autonomously switch from predicted to regular measurements in case a certain functionality becomes inapplicable.

# 3   Summary

In the paper, the functionality management for UE-sided model is discussed, and the following observations and proposals are provided:

Configuration of different use cases and sub-use cases

Observation 1: RAN4 has not studied other quantities besides RSRP so far.

Observation 2: From RAN2 point of view, there is no additional work needed to support prediction for quantities other than RSRP.

Proposal 1: Whether to include SINR or RSRQ in normative phase should be decided by RAN4, from RAN2 signalling point of view, all quantities can be supported with no additional work.

Proposal 2: Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Proposal 3: The following items should be configurable by the gNB for prediction:

- Measurement framework parameters: measurement object, measurement report, measurement ID

- Prediction related parameters: PW length, MRRT, MRRS; FFS how exactly these are configured, e.g. explicitly or via other parameters (e.g. using SMTC for MRRT or set A/B beams for MRRS)

Reporting aspects for different use cases and sub-use cases

Observation 3: For RRM measurement prediction, whether an indication about the reported result being either an actual measurement or predicted measurement depends on whether UE can be configured with AI and non-AI RRM measurement configurations at the same time and also whether UE autonomous fallback is allowed.

Observation 4: For the event prediction, HO Option 1 requires the UE to report the predicted event to allow the network to prepare the HO in advance to ensure timely HO when the actual event occurs. In this case, it will require the UE to inform the NW that the earlier event is predicted while the latter is actual event.

Observation 5: The exact time when the UE needs to perform inference depends on certain use case and reporting types.

Observation 6: UE vendors may have different strategies on when to perform inference.

Proposal 4: RAN2 to discuss whether the indication for whether the reported result (i.e. measurement or event) is a predicted or an actual measurement should be supported for different use cases.

Proposal 5: For both event triggered reporting of predicted RRM measurements and measurement reporting during predicted event reporting, it should be possible for the UE to report predicted measurement results corresponding to multiple time instances, e.g. on top of the result corresponding to TTT expiry time, the one related to PW end.

Proposal 6:  The exact time of triggering inference can be up to UE implementation as long as the UE follows the inference configuration provided from the network, e.g. PW length, time instances to be reported etc.

Associated ID

Observation 7: Associated ID is needed for some scenarios, e.g., when the NW may turn some cell(s) off for NW energy saving.

Observation 8: The model prediction accuracy was very good in the studied generalization scenarios.

Proposal 7: Associated ID should be optionally configurable for training and/or inference (i.e. it is not mandatorily required for all training/inference configurations).

Proposal 8: As a baseline, associated ID is cell-specific, similarly as for AIML for BM use case.

Applicability reporting

Proposal 9: Both option A and option B type of applicability reporting should be supported for mobility prediction configurations.

Proposal 10: For option B, at least the following parameters are needed for applicability determination (depending on the use case): PW length, MRRT, MRRS, and optionally associated ID.

Proposal 11: As a baseline, functionality management decisions are made at the network side based on the applicability reports received from the UE.

Proposal 12: RAN2 to discuss whether for temporal domain Case A prediction, UE can autonomously switch from predicted to regular measurements in case a certain functionality becomes inapplicable.

# 4   References

- R2-2504110, Discussion on performance monitoring, Huawei, HiSilicon