3GPP TSG-RAN WG2 Meeting #130	R2-2504118

St Julien's, Malta, 19th – 23rd May, 2025

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will provide our views on unnecessary retransmission avoidance and timely RLC retransmission.

# 2. Discussion on timely RLC retransmission enhancement

An agreement on analysing solutions to ensure timely RLC retransmission has been reached in RAN2#129-bis [2] meeting, as given below:

- When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

## 2.1	Enhancement on autonomous retransmission

### 2.1.1	Preventing unnecessary retransmission

It was agreed that the discardTimer maintained in PDCP layer can be reused to determine whether the RLC SDU (segment) that has been submitted to the MAC entity can be considered for autonomous retransmission when the corresponding discardTimer of the RLC SDU becomes below a threshold before the associated feedback is received. To be more specific, when the remaining time of the running discardTimer of an PDCP SDU becomes below the configured threshold, the transmitting PDCP entity can send an indication to the Tx RLC entity for triggering autonomous retransmission. Upon reception of the indication, the Tx RLC entity can consider the RLC SDU (segment) for retransmission if it has been transmitted, but the associated feedback was not yet received.

Furthermore, although it was agreed that RLC SDU (segment) can be retransmitted at most once, the co-existence between the legacy NACK based retransmission and the newly introduced discardTimer based autonomous retransmission will inevitably introduce redundant retransmission and capacity loss. For example, there can be a case that the RLC SDU (segment) is just retransmitted, e.g. due to NACK, right before the autonomous retransmission is triggered. As shown in Figure 1, if an RLC SDU (segment) has been retransmitted because of the associated NACK, it may be retransmitted soon due to the autonomous retransmission, provided that its associated ACK has not been received before the reception of the retransmission indication.

Observation 1: Retransmission of an RLC SDU (segment) can be triggered by both associated NACK and the retransmission indication, and resultant redundant retransmission will lead to capacity loss.

We believe that one ARQ retransmission for RLC SDU (segment) can ensure the transmission reliability sufficiently, regardless of whether it was triggered by NACK or autonomous retransmission. Therefore, to avoid such repeated retransmission and resource waste, it is suggested that if the associated NACK of an RLC SDU (segment) was already received (meaning that the RLC SDU (segment) is/was already considered for retransmission), the autonomous retransmission will not be triggered even when the discard indication is received.

Figure 1 Autonomous retransmission after legacy retransmission

Proposal 1: Only RLC SDU for which no feedback was received is considered for autonomous retransmission by a Tx RLC entity.

The redundant retransmission may also occur when the associated NACK is received after the retransmission has been performed due to autonomous retransmission. As depicted in Figure 2, after the RLC SDU is submitted to the MAC entity, the Tx RLC entity retransmits the RLC SDU according to the remaining time, e.g. retransmission indication is received. Then when the associated NACK is received, the same RLC SDU (segment) will be retransmitted again, resulting in capacity loss.

Figure 2 Autonomous retransmission before legacy retransmission

Similarly, there also exists the case that the RLC SDU is initially transmitted just right before the autonomous retransmission. For example, due to the uncertainty of uplink resources allocation, the hundreds of RLC SDUs may be transmitted in various time points, and the unacknowledged late-transmitted RLC SDUs may be experiencing HARQ procedures. It is highly likely that retransmitting these RLC SDUs brings no benefit for the users` experience but inevitably deteriorates the system`s capacity. For example, assuming that the threshold for autonomous retransmission is 10ms and an RLC SDU is transmitted right before its remaining time reaching the threshold, e.g. at remaining time of 11ms, there is only 1ms for the Tx RLC entity to send Status Report or for the network to schedule the HARQ retransmission.

Therefore, considering the cases mentioned above, we believe that further conditions should be considered to effectively mitigate the capacity impact, e.g. by allowing the RLC SDU to experience a complete HARQ procedure.

To be more specific, if an RLC SDU has been submitted to lower layer, it should not be retransmitted within a period, leaving sufficient time for the network to perform HARQ retransmission. This can be achieved by introducing a prohibit timer, which can be started once an RLC SDU is submitted to the lower layers. The running prohibit timer can prevent the Tx RLC entity from triggering the redundant retransmission. For example, when the specific condition is met, e.g. NACK is received or the remaining time becomes below the threshold, if the prohibit timer is not running, the Tx RLC entity considers the RLC SDU (segment) for retransmission.

Observation 2: The capacity loss imposed by the autonomous retransmission can be mitigated if it allows for time for HARQ procedure and Status Report reception for each transmitted RLC SDU.

Proposal 2: Prohibit timer should be introduced for preventing redundant retransmission, which is started when the an RLC SDU is submitted to lower layers.

Figure 3 PDU set based autonomous retransmission

The XR evolution in Rel-18 introduced a requirement of PDU set integrated transmission, where if discardTimer of any PDU within a PDU set expires, the whole PDU set should be discarded. This considers the case that AL-FEC is not applied and loss of any data within the PDU set may deteriorate the decodability of the entire PDU set. Based on such assumption, retransmitting a PDU of the PDU set brings benefit only when all PDUs within the PDU set has been initially transmitted.

Therefore, to mitigate the capacity loss introduced by autonomous retransmission, we suggest that for the PDCP entity configured with pdu-SetDiscard, the autonomous retransmission is triggered for the PDU within the PDU set only when all PDUs within the PDU set have been submitted to lower layers.

Proposal 3: If pdu-SetDiscard is configured, the autonomous retransmission is triggered for the RLC SDU (segment) only when all RLC SDUs within the same PDU set have been submitted to lower layers.

The specification impacts of the above-mentioned proposals are provided in Appendix A.

Proposal 4: RAN adopts the TP in Appendix A for preventing unnecessary retransmission introduced by autonomous retransmission.

Moreover, according to RAN2 129-bis agreement that autonomous retransmission will not be triggered for the RLC SDU (segment) assuming that it has been pending for retransmission, where the associated specification impact is FFS. This aims to prevent Tx RLC entity from redundant retransmission for the same RLC SDU (segment) in the case that the remaining time of an RLC SDU (segment) that has been pending for retransmission due to NACK becomes below the threshold. We agree it is rational to prevent redundant retransmission for the same RLC SDU (segment), but it may not have specification impact according to legacy specification. Our understanding is that it is not a new issue and the same case can be found in legacy NACK based retransmission. For example, there exists the case that the Tx RLC entity receives NACK#2 for the RLC SDU that has been pending for retransmission due to last received NACK#1, which may occur when NACK#1 is received late due to HARQ procedure or when there is no resource since NACK#1 is received. Moreover, this scenario will happen more frequently if the new timer (e.g. t-RxDiscard) maintained at Rx RLC entity can trigger Status Report. However, we believe that if the RLC SDU (segment) pending for retransmission is triggered for retransmission again, it will not be repeatedly retransmitted in the same MAC PDU.

First of all, there is no benefit to have the duplicated RLC SDU (segment) within the same MAC PDU, as MAC entity will deliver the data multiplexed in MAC PDU only when the entire MAC PDU is successfully decoded, and we should assume that means UE is sufficiently smart to avoid such occasion.

Then, according to legacy specification shown below [3], RETX_COUNT will not be increased for the RLC SDU if it has been pending for the retransmission, when its associated negative acknowledgement is received.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Based on the analysis above, we believe that the legacy specification has already prevented Tx RLC entity from retransmitting the same RLC SDU (segment) redundantly and there is no specification impact to prevent redundant retransmission in case that an RLC SDU pending for retransmission is retransmitted repeatedly due to autonomous retransmission.

Observation 3: Legacy specification has already prevented Tx RLC entity from retransmitting the same RLC SDU (segment) redundantly in case of the same RLC SDU triggered for retransmission more than once.

Proposal 5: RAN2 confirms that there is no specification impact to meet the agreement “Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission”.

### 2.1.2	RLF increment due to autonomous retransmission

Moreover, when the threshold of retransmission is reached, e.g., RETX_COUNT = maxRetxThreshold, the Tx RLC entity can indicate to upper layers that max retransmission has been reached and hence RLF can be triggered accordingly. The RETX_COUNT maintained per RLC SDU is increased once the RLC SDU (segment) is retransmitted, e.g. NACK is received or t-PollRetransmit expires. Therefore, we believe RETX_COUNT should be increased if the RLC SDU (segment) is considered for retransmission triggered by autonomous retransmission. Additionally, as the autonomous retransmission will be performed at most once according to the agreement from RAN2 129, there is no harm to count it for RLF triggering.

Furthermore, when the Tx RLC entity is allowed to stop retransmitting the RLC SDU upon the reception of the discard indication, the RLC SDU will no longer be retransmitted by autonomous or NACK based retransmission. Consequently, the RETX_COUNT may not be reached and hence the RLF is not triggered by enhanced Tx RLC entity but naturally by other RLF mechanisms.

After checking the specification about RETX_COUNT shown below3, we believe that the current specification has already covered the case that RETX_COUNT is increased after autonomous retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Proposal 6: (RLC-7) RAN2 confirms that the RETX_COUNT is increased by autonomous retransmission and there is no additional specification modification needed.

## 2.2 Enhancement on polling triggering

RAN2 129-bis has agreed to reuse PDCP discardTimer to trigger polling, and further issue would be how to trigger polling based on remaining time of the discardTimer. This newly introduced polling triggering mechanism is different from the legacy polling triggering. To elaborate more, in legacy the polling will be triggered when the number of bytes submitted to lower layers reaches a threshold, and it is clear that only one poll will be sent in a specific RLC PDU. While in the discardTimer based polling triggering, as the PDCP discardTimer is maintained per SDU and the remaining times of numerous RLC SDUs may drop below the polling threshold simultaneously, multiple polling will be triggered.

As a simple solution, when there is an indication received from the upper layer for triggering polling for a specific RLC SDU that has been submitted to the lower layer, the Tx RLC entity can include a poll in a single AMD PDU to be transmitted. To elaborate more, even if the multiple indications for triggering polling are received, the poll indication can be sent just once by including it in a single AMD PDU.

Proposal 7: (RLC-8) RAN2 supports to send a poll with a single AMD PDU even if multiple indications for polling triggering are received.

Furthermore, there exists a case that no available RLC SDU can be transmitted when the polling is triggered based on the remaining time, e.g. both the transmission and retransmission buffer are empty. As this issue is relatively similar to that in t-PollRetransmit expiry, we believe that the similar operation can be reused for reference. For example, when a poll is triggered based on the remaining time, and if there is no available RLC SDU to be transmitted, the Tx RLC entity can consider a specific RLC SDU for retransmission to include the poll. Finally, a TP is provided in the Appendix B.

Proposal 8: (RLC-11) If there is no available RLC SDU to carry the poll when the poll is triggered based on the remaining time, RLC SDU delivered to the lower layer with highest SN or any not ACKed RLC SDU can be considered for retransmission, similar as in the legacy t-pollRetransmit expiry case as given in Appendix B.

# 3. Conclusion

This contribution makes the following proposals:

Autonomous retransmission

Proposal 1: Only RLC SDU for which no feedback was received is considered for autonomous retransmission by a Tx RLC entity.

Proposal 2: Prohibit timer should be introduced for preventing redundant retransmission, which is started when the an RLC SDU is submitted to lower layers.

Proposal 3: If pdu-SetDiscard is configured, the autonomous retransmission is triggered for the RLC SDU (segment) only when all RLC SDUs within the same PDU set have been submitted to lower layers.

Proposal 4: RAN adopts the TP in Appendix A for preventing unnecessary retransmission introduced by autonomous retransmission.

Proposal 5: RAN2 confirms that there is no specification impact to meet the agreement “Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission”.

Proposal 6: (RLC-7) RAN2 confirms that the RETX_COUNT is increased by autonomous retransmission and there is no additional specification modification.

Polling enhancements

Proposal 7: (RLC-8) RAN2 supports to send a poll with a single AMD PDU even if multiple indications for polling triggering are received.

Proposal 8: (RLC-11) If there is no available RLC SDU to carry the poll when the poll is triggered based on the remaining time, RLC SDU delivered to the lower layer with highest SN or any not ACKed RLC SDU can be considered for retransmission, similar as in the legacy t-pollRetransmit expiry case as given in Appendix B.

# 4. Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

[2] Chair note of RAN2#129, https://www.3gpp.org/ftp/Meetings_3GPP_SYNC/RAN2/Inbox/Chair_Notes/R2_129bis%20R19%20XR%20session%20notes%20(Dawid)%202025-04-10_EOM.docx

[3] TS38.322

- Appendix A

## 5.2	Data transfer procedures

### 5.2.3	AM data transfer

#### 5.2.3.1	Transmit operations

##### 5.2.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows:

-	a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size;

-	a SN falls outside of the transmitting window otherwise.

The transmitting side of an AM RLC entity shall not submit to lower layer any AMD PDU whose SN falls outside of the transmitting window.

For each RLC SDU received from the upper layer, the AM RLC entity shall:

-	associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next;

-	increment TX_Next by one.

When submitting an AMD PDU that contains a segment of an RLC SDU, to lower layer, the transmitting side of an AM RLC entity shall:

-	set the SN of the AMD PDU to the SN of the corresponding RLC SDU;

- if the AMD PDU includes the last byte of an RLC SDU:

  - start the t-retransmissionProhibit for the corresponding RLC SDU, if configured.

The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an RLC SDU by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

-	send an indication to the upper layers of successful delivery of the RLC SDU;

-	set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet;

- if the t-retransmissionProhibit for the RLC SDU is running:

  - stop the t-retransmissionProhibit for the RLC SDU.

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

        - STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: and

- if the t-retransmissionProhibit, if configured, is not running:

  - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

Upon reception of the indication from upper layer to trigger the retransmission for a particular RLC SDU:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; and

- if the t-retransmissionProhibit, if configured, is not running:

  - if pdu-setDiscard is configured for the associated DRB and any AMD PDU belonging to the PDU Set including the associated RLC SDU has been submitted to lower layer: or

  - if pdu-setDiscard is not configured for the associated DRB:

    - consider the RLC SDU or the RLC SDU segment for which any acknowledgement was not received for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

  - set the RETX_COUNT associated with the RLC SDU to zero.

- else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

  - increment the RETX_COUNT.

- if RETX_COUNT = maxRetxThreshold:

  - indicate to upper layers that max retransmission has been reached.

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) t-retransmissionProhibit

This parameter is used by the transmitting side of each AM RLC entity to prevent frequent retransmission (see clause 5.3.2).

- Appendix B

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.3	Polling

#### 5.3.3.1	General

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

#### 5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

- increment PDU_WITHOUT_POLL by one;

- increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

- if PDU_WITHOUT_POLL >= pollPDU; or

- if BYTE_WITHOUT_POLL >= pollByte:

  - include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

- if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

- if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

  - include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

Upon reception of the indication from upper layer (e.g. PDCP) to trigger the polling for a particular RLC SDU:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer

  - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

  - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling)

    - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

    - consider any RLC SDU which has not been positively acknowledged for retransmission

  - include a poll in an AMD PDU as described below.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

- set the P field of the AMD PDU to "1";

- set PDU_WITHOUT_POLL to 0;

- set BYTE_WITHOUT_POLL to 0.

Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall:

- set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer;

- if t-PollRetransmit is not running:

  - start t-PollRetransmit.

- else:

  - restart t-PollRetransmit.

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) enhancedPollingThreshold

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for the RLC SDUs with remaining time below the threshold (see clause 5.3.3).