3GPP TSG-RAN WG2 #130	R2-2504131

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.2 Functionality management

Source:	Ericsson

Title:	Inference and report configuration of UE sided model

Document for:	Discussion

# 1	Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

This contribution discusses the aspects related to the inference and report configuration for the UE sided model, as part of the LCM framework.

# 2	Discussion

## 2.1 RRM measurement prediction configuration

In RAN2#129bis meeting the following was agreed:

The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

Based on this agreement, the MeasConfig IE, including the triplet MeasId, MeasObjectNR and ReportConfigNR IEs can be used for the inference configuration of the AI/ML model for mobility purpose.

MeasConfig IE, including the triplet MeasId, MeasObjectNR and ReportConfigNR IEs, is used for the inference configuration.

A first step for inference configuration is to define the Set A (the set of cells/beams the UE is going to predict) and the Set B (the set of measurements the UE provides as input to the AI/ML model).

Considering that the MeasObjectNR IE is currently used to configure the resources applicable for intra/inter-frequency measurements, this IE can be used to define the Set A and Set B as well.

The Set A and Set B is configured within the MeasObjectNR.

The Set A and Set B configuration depends on the type of predictions:

- Temporal domain prediction: Set A is the same of Set B, or it can be a subset of Set B

- Spatial domain prediction: Set A and Set B are two different set of cells/beams in the same frequency

- Frequency domain prediction: Set A and Set B are two different set of cells/beams in different frequencies

Based on this understanding and since MeasObjectNR IE configures the resources for a single frequency, the following is applicable:

- For temporal and spatial domain prediction, the same MeasObjectNR is used to configure both Set A and Set B.

- For frequency prediction, Set A and Set B are configured using two (or more) different MeasObjectNR IEs, one per each frequency.

For temporal and spatial domain predictions, the same MeasObjectNR is used to configure both Set A and Set B. For frequency prediction, Set A and Set B are configured using two (or more) different MeasObjectNR IEs, one per each frequency.

Besides the Set A and Set B configuration, additional configuration is needed to allow the UE to perform correct inference. These additional configuration parameters depend on the prediction type. Here is a list of initially proposed parameters:

- Temporal prediction (Case A): prediction window, prediction samples interval

- Temporal prediction (Case B): MRRT, measurement instances skip pattern

We think that for spatial and frequency prediction the definition of Set A and Set B is enough to allow the UE to perform inference and no additional configuration is needed.

The inference configuration for temporal domain prediction (Case A) includes at least: prediction window and prediction samples interval.

The inference configuration for temporal domain prediction (Case B) includes at least: MRRT and measurement instances skip pattern.

No additional configuration is needed for spatial and frequency domain prediction, besides Set A and Set B definition.

## 2.2 Report Configuration

In RAN2#129bis meeting the following agreement have been taken:

For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

Based on this agreement and since the RRM measurement framework is used for inference configuration and reporting, the simplest solution is to use the ReportConfigNR IE to configure the inference reporting. In existing specification, both periodical and event triggered reporting is supported and both options should be supported for predictions too.

The ReportConfigNR IE is used to configure the inference reporting.

Both periodical and event triggered reporting are supported.

The reporting information (and so the relevant configuration in ReportConfigNR IE), depends on the type of predictions:

- Temporal prediction (Case A): number of predictions, list of predictions for each prediction sample

- Temporal prediction (Case B): list of predictions (besides the RRM measurements)

- Spatial prediction: cell ID, list of predictions

- Frequency prediction: frequency, cell ID, list of predictions

The reporting for temporal domain prediction Case A includes: number of predictions, list of predictions for each prediction sample.

The reporting for temporal domain prediction Case B includes: list of predictions (besides the RRM measurements).

The reporting for spatial domain prediction includes: cell ID, list of predictions.

The reporting for frequency domain prediction includes: frequency, cell ID, list of predictions.

## 2.3 Event prediction configuration

In RAN2#129bis meeting, the following agreements have been taken:

For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Currently, the events used for RRM measurements and mobility procedures are defined in the ReportConfigNR IE, included in the MeasConfig. Considering that the predicted events can be defined in a similar way as the mobility events, it is proposed to use ReportConfigNR IE to configure the predicted events (e.g. pA1, pA2, …, pA6), in fields specific to the event predictions, to distinguish them from the RRM events.

The predicted events (e.g., pA1, pA2,…pA6) are configured in the ReportConfigNR IE in separate fields specific to event predictions.

In order to predict events using the indirect approach, the UE uses the predicted RSRP values (temporal, spatial or frequency domain predictions) as input to the entry condition (whose parameters are related to pA1, pA2,…pA6 event defined in the ReportConfigNR IE) instead of the actual measurements. This means that there needs to be an association between the MeasObjectNR defining the predictions and the ReportConfigNR defining the predicted event. This is possible via the triplet: measId, measObjectId, reportConfigId

ReportConfigNR containing the predicted event configuration is associated with the MeasObjectNR including the prediction configuration, via the triplet: measId, measObjectId, reportConfigId.

The agreement from RAN2#129bis meeting indicates as baseline the RSRP quantity. Although most of the simulations done during the Study Item were using RSRP as predicted quantity, we do not see any reason to limit the study to the RSRP quantity only since there is no indication that the results would be significantly different for other quantities than for RSRP. Therefore, other measurement quantities (e.g., RSRQ and SINR) can be considered as input/output quantity to the AI/ML model as well. From a RAN2 point of view, it may even cause additional specification impact to exclude these quantities, as they are already part of the configuration.

RSRQ and SINR can be considered as baseline quantities in the event configuration, besides RSRP.

## 2.4 Applicability

In RAN2#129bis meeting, the following agreement were taken:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

There are two cases that need to be considered for the applicability. First, there is the initial inference configuration, e.g. received at connection setup, or when the network initially configures the predictions. Second, there is the case when the applicability changes during an ongoing connection.

For the initial inference configuration, if the received configuration is applicable, the UE can consider the configuration as activated and can immediately start performing the inference. If the received configuration is not applicable, the UE can consider the inference configuration as deactivated. It is preferred that the configuration is not released as the network would like the predictions to be performed since it provided the configuration, but the UE should not perform any inference.

In general, for the initial configuration, the UE can start performing the inference as soon as it receives an applicable configuration.

For the initial inference configuration, if the inference configuration is applicable: the UE considers the configuration to be “activated” and starts performing inference.

For the initial inference configuration, if the inference configuration is not applicable: the UE considers the configuration to be “deactivated” but it does not autonomously release the configuration.

After the initial configuration, the applicability may change from applicable to non-applicable or vice versa. As the UE activates/deactivates the inference configuration upon initial configuration depending on the applicability, it is most consistent to have the same behavior also at subsequent applicability changes. That means that the UE can autonomously activate the inference configuration if it becomes applicable and autonomously deactivate the inference configuration if it becomes inapplicable. If the network does not want the UE to autonomously activate the inference configuration, it always has the option of removing the inference configuration from the UE. The UE needs to inform the network if it has activated or deactivated the inference configuration though, so that the network is aware of the status.

The UE autonomously activates a configured inference configuration if it becomes applicable. The UE notifies the network of the activation using an RRC message

The UE autonomously deactivates a configured inference configuration if it becomes inapplicable. The UE notifies the network of the deactivation using an RRC message

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	MeasConfig IE, including the triplet MeasId, MeasObjectNR and ReportConfigNR IEs, is used for the inference configuration.

Proposal 2	The Set A and Set B is configured within the MeasObjectNR.

Proposal 3	For temporal and spatial domain predictions, the same MeasObjectNR is used to configure both Set A and Set B. For frequency prediction, Set A and Set B are configured using two (or more) different MeasObjectNR IEs, one per each frequency.

Proposal 4	The inference configuration for temporal domain prediction (Case A) includes at least: prediction window and prediction samples interval.

Proposal 5	The inference configuration for temporal domain prediction (Case B) includes at least: MRRT and measurement instances skip pattern.

Proposal 6	No additional configuration is needed for spatial and frequency domain prediction, besides Set A and Set B definition.

Proposal 7	The ReportConfigNR IE is used to configure the inference reporting.

Proposal 8	Both periodical and event triggered reporting are supported.

Proposal 9	The reporting for temporal domain prediction Case A includes: number of predictions, list of predictions for each prediction sample.

Proposal 10	The reporting for temporal domain prediction Case B includes: list of predictions (besides the RRM measurements).

Proposal 11	The reporting for spatial domain prediction includes: cell ID, list of predictions.

Proposal 12	The reporting for frequency domain prediction includes: frequency, cell ID, list of predictions.

Proposal 13	The predicted events (e.g., pA1, pA2,…pA6) are configured in the ReportConfigNR IE in separate fields specific to event predictions.

Proposal 14	ReportConfigNR containing the predicted event configuration is associated with the MeasObjectNR including the prediction configuration, via the triplet: measId, measObjectId, reportConfigId.

Proposal 15	RSRQ and SINR can be considered as baseline quantities in the event configuration, besides RSRP.

Proposal 16	For the initial inference configuration, if the inference configuration is applicable: the UE considers the configuration to be “activated” and starts performing inference.

Proposal 17	For the initial inference configuration, if the inference configuration is not applicable: the UE considers the configuration to be “deactivated” but it does not autonomously release the configuration.

Proposal 18	The UE autonomously activates a configured inference configuration if it becomes applicable. The UE notifies the network of the activation using an RRC message

Proposal 19	The UE autonomously deactivates a configured inference configuration if it becomes inapplicable. The UE notifies the network of the deactivation using an RRC message

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR