3GPP TSG-RAN WG2 Meeting #130	R2-2504144

St. Julian's, Malta 19 - 23 May 2025

Agenda Item:	7.0.2.22

Source:	Huawei, HiSilicon

Title:	Handling of radio and RLC bearers in LTM with NR-DC

Document for:	Discussion and Decision

# 1	Introduction

In this document, we discuss the handling of radio and RLC bearers in LTM with NR-DC.

# 2	MCG and SCG LTM in NR-DC

The handling of radio bearers the LTM cell switch procedure is specified as follows:

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall: [...] 1> for each SRB/DRB in the current UE configuration: 2> if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or 2> if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key: 3> keep the associated PDCP and SDAP entities, their state variables, buffers and timers; 3> release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity; [...] 1> if the LTM cell switch is triggered by an indication from lower layers: 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3; 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3; [...] 1> release the radio bearer(s) and the logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete).

According to the yellow highlighted text:

-	at MCG LTM cell switch, the UE shall release all currently established SN-terminated bearers and SCG RLC bearers whose configuration is neither in the LTM candidate configuration nor in the LTM reference configuration;

-	at SCG LTM cell switch, the UE shall release all currently established MN-terminated bearers and MCG RLC bearers whose configuration is neither in the LTM candidate configuration nor in the LTM reference configuration.

At SCG LTM cell switch, SRB1/SRB2 should not be released, no MN-terminated DRB should be released and no MCG RLC bearer should be released. Therefore, the SCG LTM configuration (candidate or reference, if supported by the UE and configured) needs to include the configuration of all MN-terminated bearers and of all MCG RLC bearers.

Observation 1: According to current procedure text, the SCG LTM candidate or reference configuration must include the configuration of all MN-terminated radio bearers and of all MCG RLC bearers (otherwise, the UE will release them).

One solution would be that the MN provides the configuration of MN-terminated bearers to the SN and then the SN includes it in the LTM reference configuration, or in each SCG LTM candidate configuration.

Currently, the CG-ConfigInfo inter-node message includes the following:

CG-ConfigInfo-IEs ::=       SEQUENCE {

ue-CapabilityInfo           OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)

OPTIONAL,-- Cond SN-AddMod

candidateCellInfoListMN     MeasResultList2NR                            OPTIONAL,

candidateCellInfoListSN     OCTET STRING (CONTAINING MeasResultList2NR)  OPTIONAL,

measResultCellListSFTD-NR   MeasResultCellListSFTD-NR                    OPTIONAL,

scgFailureInfo              SEQUENCE {

failureType               ENUMERATED {t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, synchReconfigFailure-SCG,

scg-reconfigFailure, srb3-IntegrityFailure},

measResultSCG               OCTET STRING (CONTAINING MeasResultSCG-Failure)

}                                                                        OPTIONAL,

configRestrictInfo          ConfigRestrictInfoSCG                        OPTIONAL,

drx-InfoMCG                 DRX-Info                                     OPTIONAL,

measConfigMN                MeasConfigMN                                 OPTIONAL,

sourceConfigSCG             OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL,

scg-RB-Config               OCTET STRING (CONTAINING RadioBearerConfig)  OPTIONAL,

mcg-RB-Config               OCTET STRING (CONTAINING RadioBearerConfig)  OPTIONAL,

mrdc-AssistanceInfo         MRDC-AssistanceInfo                          OPTIONAL,

nonCriticalExtension        CG-ConfigInfo-v1540-IEs                      OPTIONAL

}

mcg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in MN, used by the SN to support delta configuration to UE (i.e. when MN does not use full configuration option), for bearer type change between MN terminated bearer with NR PDCP to SN terminated bearer. It is also used to indicate the PDCP duplication related information for MN terminated split bearer (whether duplication is configured and if so, whether it is initially activated) in SN Addition/Modification procedure. Otherwise, this field is absent.

This field could be included in the RRCReconfiguration message of the SCG LTM candidate configuration:

RRCReconfiguration-v1560-IEs ::=  SEQUENCE {

mrdc-SecondaryCellGroupConfig

SetupRelease { MRDC-SecondaryCellGroupConfig }  OPTIONAL, -- Need M

radioBearerConfig2   OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL, -- Need M

sk-Counter           SK-Counter                                      OPTIONAL, -- Need N

nonCriticalExtension RRCReconfiguration-v1610-IEs                    OPTIONAL

However, the current description of mcg-RB-Config indicates that this field is only used for bearer type change from MN-terminated to SN-terminated bearer, or to indicate the PDCP duplication related information for MN-terminated split bearer, but it is not used in other circumstances.

While RAN2 agreed some MN-SN coordination for LTM UE capability, if the MN indicates some capabilities that the SN is allowed to use, the SN may or may not want to use LTM (it may or may not support it), and it is not defined how the SN informs the MN and how the MN understands that it needs to provide its RB configuration to the MN.

In addition, there is currently no field in inter-node RRC messages that allows the MN to provide the configuration of MCG RLC bearers to the SN, to that they can be included in the SCG LTM reference or candidate configurations.

Observation 2: Providing the configuration of all MN-terminated bearers and MCG RLC bearers in SCG LTM candidate configurations has no UE impact but requires modifications on Xn (e.g., the SN requests the MN to provide its radio bearer and RLC bearers configuration and the MN provides it and provides updates later, until the SN indicates that it is no longer needed).

Besides, if the MN decides to modify its radio bearer configuration and informs the MN, the SN needs to update all its SCG LTM candidate configurations or the references configuration.

Observation 3: If the SCG LTM candidate configurations include the MN-terminated bearers and MCG RLC bearers, they must be updated every time the MN modifies any of its radio bearer or RLC bearer configurations.

Alternatively, it would be possible to modify the UE procedure not to release any MN-terminated bearer and MCG RLC bearer at SCG LTM cell switch

Observation 4: Alternatively, the UE procedure could be modified so that the UE keeps MN-terminated bearers and MCG RLC bearers unchanged at SCG LTM cell switch.

For MCG LTM in NR-DC, there is a similar situation: the UE will release the SN-terminated bearers such as SRB3 or SN-terminated DRBs and of SCG RLC bearers, unless the MN provides the SN RB configuration and SCG RLC bearer configuration in the MCG LTM candidate (or reference) configurations.

While the MN can at any time obtain the SN RB and SCG configuration, the SN may change it without informing the MN, so it is necessary to indicate to the SN that it must provide its full SN RB configuration at every change.

Observation 5: For MCG LTM in NR-DC, if the UE procedure is not modified, the MN needs to include all SN-terminated bearers and all SCG RLC bearers in the MCG LTM reference or candidate configurations.

Proposal 1: Discuss how to handle RBs for LTM in NR-DC

a)	option 1: modify the UE behaviour: at MCG LTM cell switch execution, the UE may only release MCG RLC bearers (unless the SCG is explicitly released) and radio bearers using the master key, and at SCG LTM cell switch execution, the UE may only release SCG RLC bearers and radio bearers using the secondary key

b)	option 2: modify MN-SN signalling for:

-	the SN to ask the MN to always provide the full MN RB + MCG configuration to the SN for SCG LTM, and the MN to provide them to the SN

-	the MN to ask the SN to always provide the full SN RB + SCG configuration to the MN for MCG LTM, and the SN to provide them to the MN

and assume that the MN/SN includes/updates them in the LTM reference or candidate configurations.

Proposal 2: If option 1 is selected, agree the draft CR in annex.

# 3	Security

If a DRB ID is present in candidate LTM configuration #1 but not in LTM configuration #2, and the UE moves from configuration #1 to configuration #2 and back to configuration #1, the same DRB is established with PDCP COUNT starting from 0.

To avoid keystream reuse, it is necessary that the network either modifies the DRB ID in configuration #1 or changes the master key by L3 signalling.

If the UE is configured to perform LTM recovery, the UE in configuration #2 may move to configuration #1 at any time, and the network may not have the time to perform L3 handover.

Proposal 3: When a DRB is only in a subset of LTM candidate configurations, the network ensures that keystream reuse cannot happen by changing the key before triggering LTM cell switch that will establish this bearer, and by not configuring the UE to perform LTM recovery.

# 4	Conclusion

We have the following proposals for LTM.

Proposal 1: Discuss how to handle RBs for LTM in NR-DC

a)	option 1: modify the UE behaviour i.e., at MCG LTM cell switch execution, the UE may only release MCG RLC bearers (unless the SCG is explicitly released) and radio bearers using the master key, and at SCG LTM cell switch execution, the UE may only release SCG RLC bearers and radio bearers using the secondary

b)	option 2: modify MN-SN signalling for:

-	the SN to ask the MN to always provide the full MN RB configuration to the SN for SCG LTM

-	the MN to ask the SN to always provide the full SN RB configuration to the MN for MCG LTM

and assume that the MN/SN includes/updates them in the LTM reference or candidate configurations.

Proposal 2: If option 1 is selected, agree the draft CR in annex.

Proposal 3: When a DRB is only in a subset of LTM candidate configurations, the network ensures that keystream reuse cannot happen by changing the key before triggering LTM cell switch that will establish this bearer, and by not configuring the UE to perform LTM recovery.

# 5	Draft CR

3GPP TSG-RAN WG2 Meeting #130	R2-25xxxxx

# Saint Julians, Malta, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | xxxx | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of radio and RLC bearers at LTM cell switch execution in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2 |  |  |  |  |  | Date: |  |  | 2025-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to clause 5.3.5.18.6, the UE shall release any radio bearer and RLC bearer that is in the current UE configuration but is not included in the reference configuration or in the candidate configuration to which the UE is executing the LTM cell switch. However, MCG LTM cell switch is supposed not to involve the SN, to the MN may not have up-to-date SN RB configuration and SCG RLC bearer configuration and it cannot include them in the reference or candidate MCG LTM configurations. As a consequence, at MCG LTM cell switch, the UE will release all SN-terminated bearers (including SRB3) and SCG RLC bearers while the SN does not expect this. Also, SCG LTM cell switch is supposed not to involve the MN and the SN does not have MN RB and MCG RLC bearer configurations anyway, so it cannot include them in the candidate SCG LTM reference or candidate configurations, and the UE will released SRB1/SRB2 and all MN RBs and RLC bearers at SCG LTM cell switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At MCG LTM cell switch execution, the UE only releases MCG RLC bearers that are neither in the reference nor candidate configuration, and MN-terminated RBs that are neither in the reference nor candidate configuration, but SN-terminated RBs and SCG RLC bearers are kept unchanged (unless the SCG is released). At SCG LTM cell switch execution, the UE only releases SCG RLC bearers that are neither in the reference nor candidate configuration, and SN-terminated RBs that are neither in the reference nor candidate configuration, but MN-terminated RBs and MCG RLC bearers are kept unchanged. Impact Analysis Impacted 5G architecture options: NR-DC Impacted functionality: LTM Inter-operability: If the network is implemented according to the CR and UE is not, SCG LTM cell switch execution will trigger the UE to move to RRC_IDLE or RRC re-establishment. MCG LTM cell switch execution in NR-DC without SCG release will cause SCG failure. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCG LTM cell switch execution will trigger the UE to move to RRC_IDLE or RRC re-establishment. MCG LTM cell switch execution in NR-DC without SCG release will cause SCG failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.18.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	for each srb-Identity in the current UE configuration:

2>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.