3GPP TSG-RAN WG2 Meeting #130	R2-2504158

St Julian’s, Malta, May 19 - 23, 2025

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues for "AI/ML Positioning Case 1"

Document for:		Discussion and Decision

# 1.	Introduction

In this contribution we discuss some remaining issues for the specification support of "AI/ML Positioning Case 1".

The following LPP Open Issues according to [1] are discussed in this contribution:

-	LPP-10a: Applicability of "batch reporting" for AI/ML positioning

-	LPP-15: Applicability of Positioning Integrity to AI/ML positioning

-	LPP-16: Signalling of Monitoring Outcome

-	LPP-17: Signalling of "ground-truth label" information

In addition, the status of the current RAN1 UE features list for positioning in [2] is discussed.

# 2.	LPP Open Issues

## 2.1	LPP-10a: Applicability of "batch reporting" for AI/ML positioning

"Batch-reporting", i.e., reporting of up to 32 location results in a single report, has been introduced in Rel-17 for the NR positioning methods. The location request can include the multiMeasInSameReport flag to indicate to the target UE to provide multiple measurement instances in a single measurement report (multiple location measurements (in case of UE-assisted) or multiple location estimates (in case of UE-based)). The corresponding capabilities (multiMeasInSameMeasReport, multiLocationEstimateInSameMeasReport) can be included in the LPP Provide Capability messages for each NR positioning method.

Since the "NR AI/ML Positioning Case 1" also falls into the category of NR positioning methods (e.g., measurements are based on NR DL-PRS), it seems consequent that batch reporting is also applicable to NR AI/ML positioning. When multiple NR positioning methods are requested (e.g., "hybrid positioning" or "fallback"), it may be inconsistent if for example DL-TDOA batch-reporting is requested, but for "NR AI/ML Positioning Case 1" not. I.e., a LMF may be interested in multiple location estimates regardless of which NR positioning method is being used.

Proposal 1 (LPP-10a):	"Batch reporting", i.e., reporting of up to 32 location results in a single report, is also applicable to "NR AI/ML Positioning Case 1".

## 2.2	LPP-15: Applicability of Positioning Integrity to AI/ML positioning

Positioning integrity refers to the trustworthiness of the position information [3]. Per RAN1 agreement from RAN1#119 all "assistance information from legacy UE-based DL-TDOA, other than info #7", can be provided from LMF to UE for AI/ML positioning Case 1. For info#7 (TRP coordinates), RAN1 is still investigating various options. However, the current list of assistance data includes info#15, which are the integrity assistance data:

| 15 | Data facilitating the integrity results determination of the calculated location |
| --- | --- |

The "data facilitating the integrity results determination" are currently defined for the following error causes/assistance data elements [3] (incl. DL-TDOA and DL-AoD assistance data):

| Error | NR Assistance Data | Integrity Fields |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Integrity Alerts | Integrity Bounds (Mean) | Integrity Bounds (StdDev) | Residual Risks | Integrity Correlation Times |
| TRP location | NR-TRP-LocationInfo | TRP DNU | Mean TRP/ARP location error | Standard deviation TRP/ARP location error | Probability of Onset of TRP fault Mean TRP fault duration | TRP/ARP location error correlation time |
| Boresight Direction of DL-PRS Resource | NR-DL-PRS-BeamInfo | DL-PRS Boresight Direction DNU | Mean Azimuth/ Elevation Error of DL-PRS Resource boresight direction | Standard deviation Azimuth/Elevation Error of DL-PRS Resource boresight direction |  | Beam Boresight Direction Angle Error Correlation Time |
| Beam information of DL-PRS | NR-TRP-BeamAntennaInfo | DL-PRS Beam Information DNU | Mean Beam Power Error per direction | Standard deviation Beam Power Error per direction |  | Beam Power Error Correlation Time |
| Inter-TRP synchronization | NR-RTD-Info | RTD DNU | Mean RTD error | Standard deviation RTD error |  | RTD error correlation time |

So far, it seems RAN1 is not defining new AI/ML positioning specific Assistance Data. Therefore, the error sources for the existing positioning assistance data summarized in the Table above would in principle also be applicable to AI/ML positioning.

For AI/ML positioning, uncertainty estimation is typically part of the Life Cycle Management, e.g., during training and model performance monitoring. For example, a Protection Level (i.e., the bound within which the true position lies with high probability) may be estimated using worst-case error bounds from training/validation data together with the system error bounds from the assistance data. Runtime monitors typically check for consistency between predicted and observed data.

The uncertainty estimation directly leads to Protection Level Calculation (e.g., from validation/monitoring data the distribution of errors can be directly computed and fitted to a confidence interval appropriate to define the desired Protection Level). However, changes in the system degrade model performance over time. Therefore, like model inference, also the error bounds of the assistance data must be consistent between training and inference. In addition, the target UE needs to be notified when anomalies are detected in "real-time", which is defined as "Integrity Alerts" (Do Not Use (DNU)) in the integrity assistance data per Table above, especially in safety-critical applications like autonomous vehicles, drones, and robotics.

Therefore, integrity calculation for ML-based positioning is an immanent capability and is inherent in the overall Life Cycle Management operation. Similar to inference, the key point here is also that the "system conditions" (i.e., the error bounds in the various assistance data) are consistent between training and inference. Therefore, the integrity assistance data summarized in the Table above must also be provided for AI/ML Positioning Case 1 (if supported).

Proposal 2 (LPP-15):	Positioning Integrity is also applicable to NR AI/ML Positioning Case 1. The existing NR integrity related assistance data can also be provided for NR AI/ML Positioning Case 1.

## 2.3	LPP-16: Signalling of Monitoring Outcome

For model performance monitoring of AI/ML positioning Case 1, RAN1 made the following agreements:

Agreement (RAN1#119) For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

Agreement (RAN1#120bis) For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

Per above agreements, the content of monitoring outcome includes an indication that the target UE cannot perform the Case 1 positioning method. If the target UE cannot perform the Case 1 positioning method – even though previously indicated in the capabilities – a change of "applicable functionality" would have occurred during an ongoing LPP session. For this, RAN2 already agreed:

RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered.

In the current version of the running CR [4], the IE NR-DL-AIML-TargetDeviceErrorCauses is used by the target device to provide NR DL AI/ML positioning error reasons to the location server (i.e., as common with all LPP positioning methods). However, current/typical error cause values are positioning measurement specific (i.e., assuming the positioning method is possible, but may have failed due to various reasons (e.g., not enough neighbour TRPs received, etc.)). Therefore, a new error cause seems needed to also indicate a change of "applicable functionality" during an ongoing LPP session. This could be a generic error cause, e.g., 'AI/ML positioning not possible'.

Proposal 3 (LPP-16):	In the case the target UE cannot perform the requested Case 1 positioning method (e.g., due to change of "applicable functionality", due to model performance monitoring issues, etc.), the target UE returns the IE NR-DL-AIML-Positioning-Error, e.g., with error cause ''AI/ML positioning not possible''.

## 2.4	LPP-17: Signalling of "ground-truth label" information

RAN1 studied various options for model performance monitoring metric calculation and agreed to support at least Option A, where the target UE performs the monitoring metric calculation. Option A includes 4 sub-options:

Agreement (RAN1#116bis) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side. - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Conclusion (RAN1#118) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Sub-option A-3 (and possibly A-4) make use of the existing PRU measurement information, as already specified in IE NR-PRU-DL-Info.

For sub-option A1, a LMF provides the "ground-truth label" to the target UE.

For sub-option A2, a LMF provides location calculation assistance data to the target UE.

The "ground-truth label" information for Case 1 includes the location of the target UE:

Agreement (RAN1#120) For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

Both, sub-option A1 and sub-option A-2 are existing MO-LR features, as summarized in TS 38.305 [3], and in more detail in TS 23.273 [5]. The following MO-LR types (see TS 23.273 [5]) can support the above sub-options:

(a)	Basic Self Location (UE can receive its own location).

(b)	Autonomous Self Location (UE can receive location assistance data).

MO-LR type (a) is typically used in the case a target UE does not support any UE-based location method. In this case, the UE sends a MO-LR request, specifying the details of the request, which is primarily the QoS (desired location accuracy). The LMF then instigates the necessary procedures and provides the UE location in the MO-LR Response to the UE. Any location method supported by the UE and LMF can be used by an LMF to obtain the UE location.

For MO-LR type (b), the MO-LR request would include an embedded LPP Request Assistance Data PDU for the desired/supported positioning method. The LMF then instigates an LPP session and sends the requested assistance data to the UE in a LPP Provide Assistance Data message. The UE may then use the provided assistance data to calculate a location using UE-based mode.

Note, an MO-LR for Autonomous Self Location may not be needed if the required position calculation assistance data are received from Positioning System Information (posSIB).

Therefore, Option A can be supported with the existing specifications. However, "ground-truth label" related issues in general depend on the data collection framework for positioning, which would be a separate discussion.

Proposal 4 (LPP-17):	A target UE can obtain the "ground-truth label" information via existing MO-LR procedures. No additional RAN2 specification impacts are foreseen.

# 3.	Discussion on UE Feature List

The lasted RAN1 UE Feature List [2] includes some features for "AI/ML Positioning Case 1":

| 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] |
| --- | --- | --- |
| 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 |

Given that RAN2 agreed to introduce "AI/ML Positioning Case 1" as a separate/new LPP positioning method, the above two capabilities are already supported per existing LPP principles.

A LPP 'method'-ProvideCapabilities IE is used by the target device to indicate its capability to support the indicated 'method' and to provide its 'method' specific positioning capabilities to the location server. In the current version of the LPP running CR [4], this is the IE NR-DL-AIML-ProvideCapabilities for "AI/ML Positioning Case 1".

The Assistance Data supported by a UE are indicated within a 'method'-ProvideCapabilities IE. I.e., the 'method'-RequestAssistanceData and 'method'-ProvideAssistanceData usually match the 'method'-ProvideCapabilities IE.

Observation 1:	Given that RAN2 agreed to introduce "AI/ML Positioning Case 1" as a separate/new LPP positioning method, the RAN1 Features 58-2-1/58-2-2 are already supported per existing LPP principles.

In addition to the above two features, the feature list for Rel-19 NR provided in [2] also includes initial DL-PRS Resource Capabilities (58-2-3/3a/3b). This was already discussed as part of the "LPP Open Issues List" discussion [1] and the current running CR has Editor's Notes accordingly [4]. A decision on whether the existing LPP DL-PRS Resource Capabilities can be reused for "AI/ML Positioning Case 1" or whether a new IE specific to "AI/ML Positioning Case 1" is needed depends on further progress of the RAN1 UE features.

Observation 2:	A decision on whether the existing LPP DL-PRS Resource Capabilities can be reused for "AI/ML Positioning Case 1" or whether a new IE specific to "AI/ML Positioning Case 1" is needed depends on further progress of the RAN1 UE features.

# 4.	Summary

In this contribution we discussed some remaining issues for the specification support of "AI/ML Positioning Case 1". The following Observations and Proposals were made.

Observation 1:	Given that RAN2 agreed to introduce "AI/ML Positioning Case 1" as a separate/new LPP positioning method, the RAN1 Features 58-2-1/58-2-2 are already supported per existing LPP principles.

Observation 2:	A decision on whether the existing LPP DL-PRS Resource Capabilities can be reused for "AI/ML Positioning Case 1" or whether a new IE specific to "AI/ML Positioning Case 1" is needed depends on further progress of the RAN1 UE features.

Proposal 1 (LPP-10a):	"Batch reporting", i.e., reporting of up to 32 location results in a single report, is also applicable to "NR AI/ML Positioning Case 1".

Proposal 2 (LPP-15):	Positioning Integrity is also applicable to NR AI/ML Positioning Case 1. The existing NR integrity related assistance data can also be provided for NR AI/ML Positioning Case 1.

Proposal 3 (LPP-16):	In the case the target UE cannot perform the requested Case 1 positioning method (e.g., due to change of "applicable functionality", due to model performance monitoring issues, etc.), the target UE returns the IE NR-DL-AIML-Positioning-Error, e.g., with error cause ''AI/ML positioning not possible''.

Proposal 4 (LPP-17):	A target UE can obtain the "ground-truth label" information via existing MO-LR procedures. No additional RAN2 specification impacts are foreseen.

# References

[1]	R2-2504130, "LPP open issues for feature "AI/ML for NR air interface"".

[2]	R2-2503308, "LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis (R1-2502979; contact: NTT DOCOMO, AT&T)".

[3]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[4]	R2-2504129: "Running CR for AI/ML Positioning Accuracy Enhancements".

[5]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".