3GPP TSG-RAN WG2 #130	R2-2504159

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.2.3

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

A new work item for A-IoT has been recently agreed in RP#106 [1]. The following objectives are defined for RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

We focus on the open issues on RACH summarized in the email discussion [POST129bis][017][AIoT] 38.391 Running CR (Huawei) in this paper.

# 2	Discussion

RAN2 has made below progress on random access related aspects in RAN2#129 bis

Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

## 2.1	R2D trigger message and Msg1 related

Issue 2-1: Msg1 resource selection - Whether/how to specify the device detailed behavior of randomly selecting the Msg1 resource based on the R2D trigger message

At RAN2#129bis, RAN2 has made the below agreement regarding the Access Occasion Trigger message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Based on the above agreements, the paging message allocates Msg1 resources (in frequency and time). The Msg1 resources allocated by the paging message are valid during the whole paging round. The Access Occasion Trigger message indicates the start of each access occasion in time where a set of Msg1 resources are available on the access occasion.

In addition, RAN1 has made the below agreement regarding Msg1 resources at RAN1#120bis

Agreement

For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19.

  - All devices support the above

  - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized

Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

Agreement

At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions.

On each access occasion determined by one Access Occasion Trigger message, either 1 or 2 time domain resource(s) can be available. In addition, multiple frequency domain resource(s) with the same data rate can be available.

On each access occasion determined by one Access Occasion Trigger message, X time domain resource(s) are available where X is indicated by the reader. In addition, Y frequency domain resource(s) with the same data rate can be available.

The total number of Msg1 resources can be determined based on the below equation:

The total number of Msg1 resources = number of time domain resources per access occasion (X) x number of frequency domain resource per access occasion (Y) x number of access occasions (Q)

We think the Msg1 resources will be only allocated by the paging message, and the Access Occasion Trigger message will indicate the time boundary of access occasions without allocating Msg1 resources.

The Access Occasion Trigger message doesn’t include Msg1 resource allocation.

Upon reception of a paging message, the device randomly selects Msg1 resources among all Msg1 resources allocated by the paging message, which is considered to be sufficient in this release.

Upon reception of a paging message, the device randomly selects Msg1 resources that are allocated by the paging message.

Issue 2-2: Paging & first R2D trigger message - Whether the R2D trigger message is needed in CFRA, and whether the first R2D trigger message will be merged into paging message in CBRA.

For CBRA, the first Access Occasion Trigger message is better to be merged into the paging message. Since the paging message is anyway to trigger start of an access round. The first access occasion starts immediately during the period of the access round. In this case, the paging message can serve dual purposes to indicate both the start of the access round and the start of the first access occasion.

The paging message includes the first Access Occasion Trigger message in CBRA,

Issue 2-3: R2D trigger message byte alignment

It was agreed in RAN2-129b that MAC PDU should be byte-aligned and FFS for the R2D trigger message (access occasion trigger), i.e., open issue 2-3 in [4].

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

We assume the Access Occasion Trigger message needs to include additional control information rather than only the message type identifier (which is around 3 bits). For example, as discussed in [4], there may need information to enable devices to distinguish the received access occasion trigger message is for the ongoing access round rather than from another round/reader. In multi-reader scenario, a device may use unintended time/frequency resources if it is not possible for the device to know from which reader the access occasion trigger message is received. This can be addressed by introducing an identifier in the message Such information may require several bits. In addition, an “end” indication may be needed for the reader to terminate an access round. Moreover, some reserved bits may be needed when considering the future extensions. Therefore, message size for the access occasion trigger message can easily go up to almost 8 bits. It is therefore proposed to have byte alignment for this R2D message.

The Access Occasion Trigger message is 1 byte.

## 2.2	CBRA procedure related

Issue 2-4: CBRA failure detection - How to determine CBRA failure/contention resolution failure:

The device performs re-access (CBRA failure detection) in subsequent access round based on numerous following events:

- if it fails to transmit CBRA Msg1 or initial contention message in its selected access occasion in the current access round,

- if it fails to receive CBRA Msg2 (response from reader) for its CBRA Msg1 transmission,

- if it receives CBRA Msg2, but the device’s random number is not indicated,

- if it has transmitted CBRA Msg3, but subsequently receives NACK for triggering re-access (agreed in RAN2#129bis).

To conclude, device performs re-access if it detects CBRA failure

- implicitly, e.g., by not receiving Msg2, or

- explicitly via

  - no device indication presence in Msg2, or

  - NACK feedback.

Device determines CBRA failure in the current access round if one of the below conditions is met:

    - No successful reception of Msg2 after transmission of Msg1.

    - Reception of Msg2(s) not including its random ID after transmission of Msg1.

    - Reception of a NACK message addressed to the device.

## 2.3 FDMA and Msg2

In case if multiple devices accessing different resources in the same occasion (FDMA resources or time resources) but coincidentally picked same random number, i.e., RN16, then devices can be differentiated in Msg2 based on their selected FDMA indices. In addition, listing resource (FDMA mapping) information Msg2 can offer reduced blind decode for subPDU identification. The device will not try to decode sub-payloads correspond to headers which does not include its selected FDMA index, and if it finds the header indicating its FDMA index and only then it proceeds with sub-payload decoding. Below are discussed various options how random number or RN16s from multiple devices can be indicated in Msg2.

For each echoed RN16, we believe there should have some mapping between the echoed RN16 and FDMA resource. This allows devices not to parse every RN16 and look for own RN16 only subject to mapping behaviour between the FDMA resource and indicated RN16. The mapping can be either static, or dynamic,

In case of static mapping, the Msg2 contains the same number of placeholders as number of available FDMA access occasions. The FDMA occasion, which is accessed successfully, the corresponding placeholder in Msg2 indicates RN16 which is transmitted over the access FDMA occasion. Other placeholders may indicate dummy RN16 where no device is identified. The drawback is the Msg2 length is long even if the number of accessed devices is small.

In the other option, the Msg2 content can be dynamic and include only successful RN16s. Compared to static scenario, the message length is small as its size is limited by the number of devices responded to in a particular Msg2, not by total number of FDMA occasions configured. For a dynamic mapping, there can two options:

- One option is bitmap based: For each FDMA occasion, there is a corresponding bit in the bit string which represents an association with FDMA occasion. If the bit is set, then corresponding payload contains successful RN16 transmitted over FDMA occasion which maps to the set bit in the string. One issue with this approach is that the devices need to know the order of bits mapped to the FDMA occasions a-priory, and if Msg1 resource is updated, then bitmap needs to be updated as well.

- In the other option, the FDMA occasion index value is included in the header and the corresponding payload contains the accessed device’s RN16 transmitted over the FDMA occasion denoted by the same FDMA index value.

We believe a dynamic option based on FDMA index as an additional header field associated with a corresponding RN16 in the mag payload is most suitable. The option with bitmap can also work but the mapping order needs to be specified (which bit in a bit string maps to which FDMA resource).

If the resources in the same access occasion contain both frequency domain resources and time domain resources, it can be further study how to index these resources. An example of the mapping logic is defined as below.

- First, in increasing order of frequency domain resource indexes for frequency multiplexed resources

- Second, in increasing order of time domain resource indexes for time multiplexed resources

Based on above discussion, we propose following observation and proposals.

Issue 2-5: random ID differentiation in Msg2

Different devices which have chosen same random number in the same access occasion can be differentiated based on their resource indices in Msg2.

In case there are multiple resources configured, reader signals to the devices a mapping between resources indices and received random IDs in Msg2. FFS on how to index resources in the same access occasion determined by one Access Occasion Trigger message

Issue 2-6: number indication of echoed random IDs in Msg2

This is the issue on whether/how to indicate the number of echoed random IDs included in Msg2.

Assuming all the other parts in Msg2 (i.e. message type, scheduling info, FDM occasion index, random ID) have fixed length and are put in the beginning of the MAC PDU, the device can consider the remaining part is random ID/random ID list, and then decode random ID one by one. In this case, Msg2 need not contain an indication of the number of random IDs.

Msg2 need not contain an indication of the number of random IDs.

Issue 2-7: present/absent indication of assigned AS ID in Msg2

Another issue on Msg2 content is how to indicate AS ID to device in Msg2 if reader assigns it.

Since the presence of AS ID in Msg2 is optional, it is reasonable to include 1 bit to indicate presence or absence of AS ID for each entry of the random ID list in Msg2.  If AS ID is absent, the device assumes that the reader would use the random ID to address the device.

Include one bit indicating AS ID presence/absence for each entry of the random ID included in Msg2.

If the AS ID is absent for a random ID in Msg2, the corresponding device assumes that the reader will use the random ID to address the device.

## 2.4	CFRA

Issue 2-8: no re-access for CFRA

RAN2 has made the below agreement regarding CFRA at RAN2#129bis

For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve.

For a failed D2R transmission in CFRA, the reader can trigger the device to perform retransmissions by retransmitting the R2D message or send a new paging indicating CFRA.

For CFRA, the reader may trigger the device to perform retransmissions by retransmitting the R2D message or sending a new paging indicating CFRA.

Issue 2-9: AS ID assignment in multi-device CFRA

To provide CFRA, the reader needs to allocate dedicated (collision-free) resources to the devices.

In one option, the device IDs are part of NAS container or kept hidden from reader in paging message. In order to allocate CFRA resource, the

- the reader needs to know the group size or number of devices from CN and this information can be retrieved by reader from CN via NGAP signaling, and based on this information, the reader provides a group/list of resources corresponding to the number of the devices without any explicit mapping;

- An implicit mapping rule is defined by the reader, for instance, the device can locate its resource in the resource list according to the position of its device ID in the device ID list.

Further, we believe CFRA should be permitted for multiple devices, similar to CBRA based allocation (can be considered as baseline). The allocation behaviour can be inspired from CBRA where instead the occasions are allocated in a dedicated manner. We do not see reason on limiting CFRA to single device.

For CFRA, it is assumed that the reader knows at least the number of devices (e.g., provided by CN) to allocate contention free resources corresponding to the number of devices.

The device can find its resource in the resource list included the paging message according to an implicit mapping rule, e.g., the device can locate its resource in the resource list according to the position of its device ID in the device ID list.

Support CFRA for multiple devices triggered by a single paging message.

For CFRA, reader indicates a mapping rule between the set of allocated resources and the corresponding devices explicitly or implicitly to device.

Further, on issue of allocating AS ID assignment for multiple devices, it is non-trivial issue as for each CFRA occasion, a specific or single device transmits even if paging message addresses multiple devices for CFRA. For instance, if paging message indicating CFRA mode, which is addressing to two devices, and thus the paging message allocates two separate CFRA occasion, whereas in each CFRA occasion, a single device transmits Msg1 containing device ID, then in Msg2, the reader can allocate AS ID if needed.

In addition, RAN2 has agreed that

For CFRA, command message is used for AS ID assignment

For CFRA with multiple devices, each device may be associated with different command messages. Multiplexing multiple AS IDs in the same R2D message may increase the size of the R2D message dramatically. Therefore, we would like to give the below proposal.

For CFRA, the AS ID assignment for multiple devices occur in separate device-specific R2D messages.

## 2.5	NACK feedback

Issue 2-10: NACK before paging or R2D trigger message

For the above issue, it is FFS whether the subsequent R2D message is the R2D trigger message or paging message.  If the device cannot receive NACK message before the subsequent R2D message, the device interprets its previous transmission has been successful, i.e., interpreted as ACK. Since RAN2 has already agreed to rely on the R2D trigger message to indicate the end of the current access occasion, it is simple for the device to conclude whether the current transmission has been successful or not when the access occasion ends. Otherwise, the device would need to store information on previous transmissions across multiple access occasion, which increases the device operational complexity.

For CBRA, device determines to re-access if device receives NACK message when the device receives the subsequent Access Occasion Trigger message.

Concerning the NACK transmission in the last occasion, there is no further Access Occasion trigger message (as there are no more occasions to indicate). Hence, if a device transmits a Msg3 on the last access occasion in a CBRA procedure, the device will not receive a subsequent Access Occasion trigger message indicating end of the current occasion, then the device has no deterministic understanding of monitoring window for NACK, i.e., until what time it should monitor. RAN2 agreed in the last meeting not to have timer for NACK monitoring window, and if there is no further R2D trigger message (until NACK can be monitored) after last access occasion, then we need to specific rules for monitoring and receiving NACK. Few options can be defined, for instance,

- Always assume ACK (no explicit signalling is sent, i.e., whenever device transmits Msg3, it always assumes success),

- Always assume NACK (no explicit signalling is sent, i.e., whenever device transmits Msg3, it always assumes failure); this will render last access occasion useless, and device perhaps will never select if the status always be assumed NACK implicitly,

- Leave it to implementation, if device can monitor up to certain time depending on its hardware capabilities,

- Send an additional R2D trigger message after last access occasion which act as an end window for possible reception of NACK.

Among the above options, the last option is simplest. Up reception of the additional Access Occasion Trigger message, the devices which are on the last access occasion would understand the last occasion has ended.

Although, the above issues focus on NACK monitoring, which a type of R2D message, however, this problem exists for every corresponding R2D message monitoring whenever device transmits its D2R message (Msg1, Msg3, Msg5, so on).

For CBRA, it is up to reader implementation to send an additional Access Occasion Trigger message to devices.

Issue 2-11: explicit message for NACK

For this issue, it is beneficial to have an explicit NACK message, since the NACK message has different format compared to Msg2. Msg2 is supposed to include resources to devices, while NACK message does not include resources. In addition, the NACK message is solely used to trigger re-accesses rather than retransmissions of the data. Therefore, it is clearer to have a separate message type for the NACK message.

For CBRA, support a new R2D message type for the NACK message.

Issue 2-12: multiplexing for NACK indication

Whether to support multiplexing of information for multiple devices in NACK feedback.

- Relevant agreements:

- Support multiplexing of information for multiple devices in R2D message for msg2.  FFS others for multicast messages.

Status in running CR: not captured yet.

For the above issue, it is sufficient to leave to the reader decision/implementation to decide whether multiplexing of NACK indicators for multiple devices can be applied or not. The reader may have better knowledge on status of connection and coverage for devices in DL, based on which, the reader can decide whether information of NACK indicators for multiple devices can be supported or not. From message format design perspective, the message format shall allow multiplexing of NACK indicators to be supported.

Multiplexing of information for multiple devices in NACK message is supported.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	On each access occasion determined by one Access Occasion Trigger message, X time domain resource(s) are available where X is indicated by the reader. In addition, Y frequency domain resource(s) with the same data rate can be available.

Observation 2	Different devices which have chosen same random number in the same access occasion can be differentiated based on their resource indices in Msg2.

Observation 3	For CFRA, it is assumed that the reader knows at least the number of devices (e.g., provided by CN) to allocate contention free resources corresponding to the number of devices.

Observation 4	The device can find its resource in the resource list included the paging message according to an implicit mapping rule, e.g., the device can locate its resource in the resource list according to the position of its device ID in the device ID list.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The Access Occasion Trigger message doesn’t include Msg1 resource allocation.

Proposal 2	Upon reception of a paging message, the device randomly selects Msg1 resources that are allocated by the paging message.

Proposal 3	The paging message includes the first Access Occasion Trigger message in CBRA,

Proposal 4	The Access Occasion Trigger message is 1 byte.

Proposal 5	Device determines CBRA failure in the current access round if one of the below conditions is met:

i.	No successful reception of Msg2 after transmission of Msg1.

ii.	Reception of Msg2(s) not including its random ID after transmission of Msg1.

iii.	Reception of a NACK message addressed to the device.

Proposal 6	In case there are multiple resources configured, reader signals to the devices a mapping between resources indices and received random IDs in Msg2. FFS on how to index resources in the same access occasion determined by one Access Occasion Trigger message

Proposal 7	Msg2 need not contain an indication of the number of random IDs.

Proposal 8	Include one bit indicating AS ID presence/absence for each entry of the random ID included in Msg2.

Proposal 9	If the AS ID is absent for a random ID in Msg2, the corresponding device assumes that the reader will use the random ID to address the device.

Proposal 10	For CFRA, the reader may trigger the device to perform retransmissions by retransmitting the R2D message or sending a new paging indicating CFRA.

Proposal 11	Support CFRA for multiple devices triggered by a single paging message.

Proposal 12	For CFRA, the AS ID assignment for multiple devices occur in separate device-specific R2D messages.

Proposal 13	For CBRA, device determines to re-access if device receives NACK message when the device receives the subsequent Access Occasion Trigger message.

Proposal 14	For CBRA, it is up to reader implementation to send an additional Access Occasion Trigger message to devices.

Proposal 15	For CBRA, support a new R2D message type for the NACK message.

Proposal 16	Multiplexing of information for multiple devices in NACK message is supported.

# 4	References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.