3GPP TSG-RAN WG2 #130	R2-2504160

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.13.3

Source:	Ericsson

Title:	Discussion on control plane procedures

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on control plane procedures have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

We discuss aspects on control plane procedures in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

## 2.1 SRAP configuration

RAN2 has made the below agreements regarding SRAP at RAN2#129bis

- SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17.

- For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs.

- For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.  We will review next meeting if this explicit mapping functionality is needed.

- FFS whether to introduce reflective bearer mapping as an optional feature.

And for how to configure the Remote UE ID and BEARER ID, in R17 U2N Relay, network configures the SRAP configuration for one or more remote UEs at the U2N Relay as follows:

-- ASN1START -- TAG-SL-L2RELAYUE-CONFIG-START SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N ..., SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE { sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M ... } -- TAG-SL-L2RELAYUE-CONFIG-STOP -- ASN1STOP

As shown in the above specification, the network configures the SRAP configuration for each remote UE at the U2N Relay UE based on the L2 ID of the remote UE.

RAN2 has agreed the baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.  We will review next meeting if this explicit mapping functionality is needed.

Based on the above baseline, each relay UE (i.e., last relay UE and intermediate relay UE) is configured by the network with the SRAP configuration for each directly connected child UEs wherein the SARP configuration entry is indexed by the L2 ID of each connected child UE. Each SRAP configuration entry of each directly connected child will also include a list of SRAP configuration entries for indirectly connected child UEs. The ASN.1 (as captured in the RRC running CR) is illustrated in the below

-- ASN1START -- TAG-SL-L2RELAYUE-CONFIG-START SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N ..., } SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE { sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M ..., [[ sl-SRAP-ConfigRelayList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-r17 OPTIONAL -- Need R ]] } -- TAG-SL-L2RELAYUE-CONFIG-STOP -- ASN1STOP

For the above baseline approach, two drawbacks are observed

- Upon reception of a SRAP data PDU, the relay UE needs to first locate the matching SRAP configuration entry according to the local ID contained in the SRAP header, then to find the L2 ID of the directly connected child. The searching procedure is two-step wise, which is not efficient.

- It is not feasible for the gNB to provide updated delta configuration for one indirectly connected child based on its L2 ID. In the current ASN.1. the gNB has to include SRAP configurations for all indirectly connected child UEs and SRAP configuration for the directly child UE, in order to reconfigure SRAP configuration for one indirectly connected child. This would increase the RRC signalling overhead.

For the baseline approach, upon reception of a SRAP data PDU, the relay UE needs to first locate the matching SRAP configuration entry according to the local ID contained in the SRAP header, then to find the L2 ID of the directly connected child. The searching procedure is two-step wise, which is not efficient.

For the baseline approach, it is not feasible for the gNB to provide delta configuration for one indirectly connected child based on its L2 ID.

Instead, it is better to introduce a flatten structure. So, for each relay UE, the relay UE can be configured with SRAP configuration for each child UE separately/independently, regardless of it is directly connected or indirectly connected. In addition, such alternative may be easier for relay UE to search the matching entry when receiving a SRAP data PDU, the relay UE doesn’t need to perform nested search. In such alternative, we can introduce a new field nextHopL2ID-r19 in SL-RemoteUE-ToAddMod-r17, which is only present for indirectly connected child UE, the egress link for indirectly connected child UE is determined based on nextHopL2ID.

By adopting a flatten structure for the SRAP configuration of each relay UE, it is easier to find matching SRAP entity upon reception of an SRAP data PDU since the searching procedure is 1 step wise, in addition, it is feasible for the gNB to provide delta configuration for each indirectly connected child UE.

By adopting a flatten structure for the SRAP configuration of each relay UE, it is easier to find matching SRAP entity upon reception of an SRAP data PDU since the searching procedure is 1 step wise.

By adopting a flatten structure for the SRAP configuration of each relay UE, it is feasible for the gNB to provide delta configuration for each indirectly connected child UE.

For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for both directly connected child UEs and indirectly connected child UEs, based on their L2 IDs, i.e., the flatten structure for SRAP configuration is adopted.

In the flatten structure for SRAP configuration, at each relay UE, the SRAP configuration is configured for each indirectly connected child UE, in this case, an additional field is needed to indicate the next hop L2 ID in DL mapping, since the child UE’s own L2 ID is not feasible to be used for determining the egress link.

In the flatten SRAP structure at the relay UE (both last relay and intermediate relay UE), include a field sl-DLNextHop-L2Identity-r19 for configuring the next hop L2 ID for downlink mapping.

In the flatten SRAP structure at the relay UE (both last relay and intermediate relay UE), for each received SRAP data PDU, the relay UE determines the egress link based on sl-DLNextHop-L2Identity-r19 if it is present, otherwise, based on sl-L2IdentityRemote.

Regarding the below FFS

FFS whether to introduce reflective bearer mapping as an optional feature.

We think it is feasible for relay UE however it should be an optional feature for each relay UE.

Reflective mapping is optional to be supported at relay UE for DL mapping.

Regardless of if the baseline approach or the flatten structure for SRAP configuration, reflective mapping is feasible to be supported for each relay UE.

Reflective mapping comprises reflective mapping for egress DL link and reflective mapping for egress DL relay RLC channel.

For the former, for each received SRAP data PDU, the relay UE can determine the egress link based on the PC5 link from which the local ID (of the SRAP data PDU) is received. Reflective mapping for determining the egress link can be always applied (if the relay UE supports it), since each child UE will always start with SRB transmission.

Reflective mapping for determining the egress link can be always applied (if the relay UE supports it), since each child UE will always start with SRB transmission.

For the latter, the relay UE can determine the egress RLC channel based on the RLC channel from which the RB ID (of the SRAP data PDU) is received. Reflective mapping for determining the egress RLC channel may be not always feasible (even if the relay UE supports it), since each child UE may not always start with UL transmission for an RB (e.g., SRB2 and/or DRBs). When the gNB provides SRAP configurations to each relay UE, it is not clear on whether a remote UE will start its UL transmission for an SRB2 and/or DRBs. Therefore, the gNB cannot determine whether reflective mapping can be applied for the RB. Based on this analyses, we think it is reasonable to not support Reflective mapping for egress RLC channel determination

Reflective mapping for determining the egress RLC channel may be not always feasible (even if the relay UE supports it), since each child UE may not always start with UL transmission for an RB (e.g., SRB2 and/or DRBs).

Reflective mapping is only supported for egress link determination.

If Reflective mapping for determining the egress link is applied (if the relay UE supports it), the SRAP configuration for a child UE can ignore the field sl-DLNextHop-L2Identity-r19 for the flatten structure. While the field sl-LocalIdentity-r17 is still needed to be included in the SRAP configuration for each child for indexing the corresponding SRAP configuration.

When Reflective mapping is applied, the field sl-DLNextHop-L2Identity-r19 for the flatten structure is absent, while the field sl-LocalIdentity-r17 is still needed to be included in the SRAP configuration for each child for indexing the corresponding SRAP configuration.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For the baseline approach, upon reception of a SRAP data PDU, the relay UE needs to first locate the matching SRAP configuration entry according to the local ID contained in the SRAP header, then to find the L2 ID of the directly connected child. The searching procedure is two-step wise, which is not efficient.

Observation 2	For the baseline approach, it is not feasible for the gNB to provide delta configuration for one indirectly connected child based on its L2 ID.

Observation 3	By adopting a flatten structure for the SRAP configuration of each relay UE, it is easier to find matching SRAP entity upon reception of an SRAP data PDU since the searching procedure is 1 step wise.

Observation 4	By adopting a flatten structure for the SRAP configuration of each relay UE, it is feasible for the gNB to provide delta configuration for each indirectly connected child UE.

Observation 5	Reflective mapping for determining the egress link can be always applied (if the relay UE supports it), since each child UE will always start with SRB transmission.

Observation 6	Reflective mapping for determining the egress RLC channel may be not always feasible (even if the relay UE supports it), since each child UE may not always start with UL transmission for an RB (e.g., SRB2 and/or DRBs).

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for both directly connected child UEs and indirectly connected child UEs, based on their L2 IDs, i.e., the flatten structure for SRAP configuration is adopted.

Proposal 2	In the flatten SRAP structure at the relay UE (both last relay and intermediate relay UE), include a field sl-DLNextHop-L2Identity-r19 for configuring the next hop L2 ID for downlink mapping.

Proposal 3	In the flatten SRAP structure at the relay UE (both last relay and intermediate relay UE), for each received SRAP data PDU, the relay UE determines the egress link based on sl-DLNextHop-L2Identity-r19 if it is present, otherwise, based on sl-L2IdentityRemote.

Proposal 4	Reflective mapping is optional to be supported at relay UE for DL mapping.

Proposal 5	Reflective mapping is only supported for egress link determination.

Proposal 6	When Reflective mapping is applied, the field sl-DLNextHop-L2Identity-r19 for the flatten structure is absent, while the field sl-LocalIdentity-r17 is still needed to be included in the SRAP configuration for each child for indexing the corresponding SRAP configuration.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.