3GPP TSG-RAN WG2 #130	R2-2504161

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.13.2

Source:	Ericsson

Title:	Discussion on relay discovery and relay (re)selection

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on relay discovery and relay (re)selection have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

.

We discuss the remaining open issues summarized in Post129bis][405][Relay] Rel-19 relay main CR to 38.331 (Huawei) and related to discovery and/or relay (re)selection in this paper.

# 2	Discussion

Open issue RRC-5: NotificationMessageSidelink message handling

For the above open issue, there are two options suggested by the rapporteur.

Option 1 – Forward a copy of the original notification (with the same cause) to its child UEs.
Option 2 – Generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment.

The issue with Option 1 is that it will lead to duplicate notifications being sent by the intermediate relay UE: first, a forwarded copy of the parent’s original notification, and then a second notification reflecting the intermediate relay’s own actions. This redundancy is clearly undesirable as it does not provide the child UE with any additional information.

To avoid this, the Rapporteur suggests adopting Option 2, where the intermediate relay UE generates a new notification message. This message would reflect the relay's own status or reaction (e.g., relay reselection or re-establishment) and is always sent, allowing the Remote UE to take appropriate action based on the most relevant and current context of its parent UE.

We agree with the rapporteur that the Option 2 should be adopted to limit the spec changes.

When an intermediate relay UE receives a NotificationMessageSidelink message from the parent, the intermediate relay UE generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment

In addition, RAN2 has agreed as the below:

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

- FFS detailed information in the indication and child UE handling.

- FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

- FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

For the above FFS, the discussion was initially triggered for Model A discovery scenario. For Model A discovery scenario, when the parent UE in idle/inactive performs relay reselection, our understanding is that the parent UE can trigger discovery message to update the path. However, for cases where the reselected path/parent remains the same serving cell without changing the hop count etc, it is reasonable to not trigger discovery message. However we think such procedure is not in RAN2 domain, rather handled by SA2.

For Model A discovery, when the parent UE in idle/inactive performs relay reselection, the parent UE may trigger discovery message towards child UE(s). No RAN2 impact is assumed. FFS whether to inform SA2 on this.

We think the issue is also relevant to Mode B discovery, in that case, the patent UE may send a NotificationMessageSidelink to child UE(s) to trigger relay reselection.

For Model B discovery, when the parent UE in idle/inactive performs relay reselection, the parent UE sends a NotificationMessageSidelink message to child UE(s) to trigger relay reselection. FFS on whether new cause is needed.

In addition, in Solution #1(TR 23.700-03), SA2 has discussed how to use Hop-Count value in relay (re)selection procedure. In an example, Remote UE selects the Relay UE announcing the least Hop-Count value to build the path with shortest hop numbers. We think it is not necessary to disclose Hop-Count value into AS, due to below reasons

1) Hop-count is included in Discovery or DCR/DCA messages, which are not seen by AS by nature.

2) It is sufficient to categorize such criteria as Upper layer criteria, and leave them to be handled by Upper layers. Disclose them to AS may introduce double control/usage of these criteria at both Upper layer and AS layer, which increases additional complexity.

Therefore, we propose that

Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

# 3	Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	When an intermediate relay UE receives a NotificationMessageSidelink message from the parent, the intermediate relay UE generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment

Proposal 2	For Model A discovery, when the parent UE in idle/inactive performs relay reselection, the parent UE may trigger discovery message towards child UE(s). No RAN2 impact is assumed. FFS whether to inform SA2 on this.

Proposal 3	For Model B discovery, when the parent UE in idle/inactive performs relay reselection, the parent UE sends a NotificationMessageSidelink message to child UE(s) to trigger relay reselection. FFS on whether new cause is needed.

Proposal 4	Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.