3GPP RAN WG2 Meeting #130 	R2-2504180

# St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.6.4

Source:	InterDigital, Inc.

Title:	LCP for LTM measurement report MAC CE.

Document for:	Discussion

# Introduction

The Rel-19 WI on mobility enhancements contains the following measurement related objectives.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In RAN2#127bis, the following agreement was made:

- 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In RAN2#128, the following agreements were made:

- Support the truncated measurement report MAC CE.

In RAN2#129, the following agreements were made:

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

In RAN2#129bis, the following agreements were made:

- When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

# Discussion

It has been agreed to use separate LCID for truncated vs. non-truncated report. The truncated report is used when there is not enough available grant, after the LCP procedure has been applied, to transmit the non-truncated report. If there is not enough grant to transmit the truncated report then an SR needs to be sent. It has also been agreed that a dedicated SR can be used for this purpose.

We have not discussed the relative priority of each of the new LCIDs, even though there is a text proposal in the running CR.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation, or MAC CE for Event Triggered L1 Measurement Report; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; - MAC CE for Timing Advance Report; - MAC CE for Delay Status Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for SL-PRS Resource Request; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding. - NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation. The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

According to the current running CR, the truncated and non-truncated MAC CE for Event Triggered L1 Measurement Report are not distinguished. Usually, we specify a priority for each of the new LCIDs individually.

If the priority of truncated and non-truncated MAC CE is the same, and higher priority than data, as per the current running CR, then it would be a very unusual case for the truncated MAC CE to ever be used. There would need to be just enough grant to transmit the truncated report so that an SR is not needed, but not enough grant for the non-truncated report.

The truncated report contains the essential information for triggering LTM, while the non-truncated report contains “nice to have” additional information. Therefore, the truncated report should be a higher priority than the non-truncated report.

Proposal 1: Truncated MAC CE for Event Triggered L1 Measurement Report has a higher LCP priority than non-truncated report.

Since the truncated report contains the essential information for triggering LTM, we think it can be higher than any other LCID except MAC CE for C-RNTI, or data from UL-CCCH

Currently the running CR puts the MAC CE report at the same priority as MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation, which is higher priority than all other LCIDs except for MAC CE for C-RNTI, or data from UL-CCCH (although this may need clarification, because the current TP does not mention the truncated report).

From the running CR there is no specified priority for the MAC CE for Event Triggered L1 Measurement Report relative to MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation. This suggests that, for the case the UE has e.g. a configured grant confirmation and Event Triggered L1 Measurement Report to transmit, then the priority is up to UE implementation.

Proposal 2: Truncated MAC CE for Event Triggered L1 Measurement Report has higher LCP priority than any other LCID except MAC CE for C-RNTI, or data from UL-CCCH (as per the current running CR).

Since the non-truncated report has non-essential information, then the non-truncated report should not have higher priority than some other MAC CEs.

For example, a TA report MAC CE could be included before the additional information in the non-truncated report to allow for TA adjustment as a priority over the additional measurement information. It may even be desirable for the non-truncated report to have lower priority than data, so that data is prioritised over the non-essential measurement information. However, this may be too extreme since it will typically mean that the additional measurement information is rarely transmitted, assuming that most of the time there will be data for uplink transmission. It would be beneficial for the non-truncated MAC CE report to have lower priority than BSR – by omitting BSR in favour of L1 measurement report, an SR would be triggered for requesting resources for BSR transmission, further delaying the transmission of additional measurement information compared to including the BSR instead, allowing for a grant in the next TTI.

Proposal 3: Non-truncated MAC CE for Event Triggered L1 Measurement Report has lower LCP priority than BSR and higher priority than data from any Logical Channel, except data from UL-CCCH.

# Conclusion

In this paper we make some suggestions on LCP as follows:

Proposal 1: Truncated MAC CE for Event Triggered L1 Measurement Report has a higher LCP priority than non-truncated report.

Proposal 2: Truncated MAC CE for Event Triggered L1 Measurement Report has higher LCP priority than any other LCID except MAC CE for C-RNTI, or data from UL-CCCH (as per the current running CR).

Proposal 3: Non-truncated MAC CE for Event Triggered L1 Measurement Report has lower LCP priority than BSR and higher priority than data from any Logical Channel, except data from UL-CCCH.

# References

- RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

# Text Proposal

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - Truncated MAC CE for Event Triggered L1 Measurement Report - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; - MAC CE for Timing Advance Report; - MAC CE for Delay Status Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for SL-PRS Resource Request; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - MAC CE for Event Triggered L1 Measurement Report - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding. - NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation. The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.