3GPP TSG-RAN WG2 Meeting #130	Tdoc R2-2504186

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	8.12.3

Source:	Ericsson

Title:	Impacts from UE-initiated/event-driven beam management

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the impacts from NR MIMO phase 5 for the following objective:

- Specify enhancement to facilitate UE-initiated/event-driven beam management

# 2	Discussion

It is FFS whether UEI BM parameters should be moved to CSI-MeasConfig. This should be discussed later once most of the UEI BM parameters are resolved and thus can be postponed for now.

No need to address the FFS on “whether UEI BM parameters should be moved to CSI-MeasConfig” in RAN2-130.

Moreover, it was also discussed during [Post129bis][214][ MIMO_Ph5] how to update UEI BM parameters based on the dependencies between the parameters. We understand that the suggestions given during this discussion can be taken as baseline and the update can be done once more parameters are defined by RAN1 for this objective. Namely, it could be confirmed that a new reportConfigType is used. For other fields, we understand that naturally if certain fields can be bundled together in ASN1 structure this can be done during CR update.

As a baseline, the events defined for UEI BM should be captured as a new reportConfigType which can also include parameters applicable only for one event. This can be captured once more parameters for this objective are defined by RAN1.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	No need to address the FFS on “whether UEI BM parameters should be moved to CSI-MeasConfig” in RAN2-130.

Based on the discussion in the previous sections we propose the following:

Proposal 1	As a baseline, the events defined for UEI BM should be captured as a new reportConfigType which can also include parameters applicable only for one event. This can be captured once more parameters for this objective are defined by RAN1.