3GPP TSG-RAN WG2 Meeting #130	Tdoc R2-2504189

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	8.20.1

Source:	Ericsson

Title:	Additional aspects on Type 4 for non-collocated deployment

Document for:	Discussion, Decision

# 1	Introduction

RAN4 has sent an LS (R4-2505161) asking RAN2 to define UE capability and RRC signalling for Type 4a/4b requirements for both NR-CA and EN-DC. This contribution discusses how to address the RAN4 request related to the EN-DC case.

# 2	Discussion

For EN-DC, RAN4 had the following info in the LS:

UE capability Type

- For intra-band non-contiguous NR-CA, Type 4b is considered, Type 4a is not considered

- For inter-band non-contiguous EN-DC with overlapping or partially overlapping bands, both Type 4a and Type 4b are considered

UE requirements:

- Reuse the power imbalance of Type 2

- Reuse the MRTD and MTTD requirements of Type 2

UE capability:

- Define one capability for NR CA and one capability for EN-DC

- For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier

- If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS)

BS signalling:

- When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed

- When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed

- When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2

Even though it may be possible to rely on existing capabilities (i.e. MIMO-CapabilityDL-r10), that would imply that a UE already supporting MIMO-CapabilityDL-r10 set to 4 layers and intending to support only Type 4a capability would have to either refrain from supporting the Type 4a capability or refrain from indicating MIMO-CapabilityDL-r10 set to 4 layers (and set it to 2 layers instead). Therefore, it could be safer to have separate capabilities for Type 4a and Type 4b in EN-DC.

Type 4a/b capabilities for EN-DC are defined separately.

Similarly, the configuration for EN-DC case should have separate fields for Type 4a and Type 4b. It should be noted that we should not rely on existing capabilities to define the UE behaviour (as indicated in the guideline below from RAN2, R2-2002378) but rather define a proper RRC configuration for such behaviour.

5 	Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

RAN2 has the guideline to avoid defining functionality that has no RRC configuration but is dependent on capability bits.

For EN-DC, Type 4a and Type 4b are configured separately.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has the guideline to avoid defining functionality that has no RRC configuration but is dependent on capability bits.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Type 4a/b capabilities for EN-DC are defined separately.

Proposal 2	For EN-DC, Type 4a and Type 4b are configured separately.