3GPP TSG-RAN WG2 Meeting #130	R2-2504207

St. Julian’s, Malta,  May 19th – 23rd , 2025

Agenda item:	8.8.2

Source:	Samsung

Title:	Open issues on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NR NTN, according to the WID [1], link-level enhancements to be specified are PDCCH repetition for CSS (except for Type-3) and PDSCH repetition for SIB1 and Msg4, system-level enhancement to be specified is SSB periodicity extension to 160ms as default value in initial access.

In this contribution, we share our views on RAN2 impacts of these aspects.

# Discussion

## SMTC selection in RRC idle/inactive

In the last meeting, RAN2 has agreed to support configuring more than 4 SMTCs (e.g., 6) for idle/inactive UEs and UE can select by implementation a smaller number of SMTCs up to its capability, for which NW can provide location assistance information.

Regarding the location information, it should assist UE to know neighbor cells in certain SMTC(s) are close to UE’s location so that UE only needs to apply that SMTC(s). To this end, the association between geographic areas and SMTCs should be provided. The geographic areas can be indicated explicitly or implicitly. Here we discuss the pros and cons of several approaches. Examples for option 1 and 2 are shown in the Figure below on the left and option 3 on the right.

- Reference location

How it works: one or multiple reference locations is provided associated to a SMTC. A reference location indicates a point (e.g., center point) in the area of one or multiple adjacent neighbor cells. If the serving cell is an earth moving cell, the reference location(s) are the given for the epoch time and UE can derive the real-time reference location(s) based on serving cell ephemeris which is already supported for earth-moving serving cell. UE can apply the information when it has a valid UE location, determine the close reference location(s), and apply the associated SMTC(s) to measure the neighbor cells for the selected SMTC(s).

Pros:

- Explicit information for geographic area and easy to be used for UE;

- NTN well-used approach to signal geographic location information.

Cons:

- Signaling overhead as other location-based features. A reference location (i.e., ellipsoid point as currently used in NTN) requires 48 bits as defined in TS 37.355.

- SSB bitmap

How it works: An SSB bitmap is provided associated to a SMTC. The signaled SSB index indicates the area covered by the corresponding SSB beam. UE selects an SSB (e.g., based on the measurements of SSB beams) of the serving cell and applies the associated SMTC(s) to measure the neighbor cells for the selected SMTC(s).

Pros:

- Overhead can be small for FR1, i.e., short bitmap of 4 bits for FR1-NTN.

- No need to know UE’s location.

Cons:

- Overhead can be large for FR2-NTN, i.e., long bitmap of 64 bits for FR2-NTN;

- cannot work when there is only one SSB beam transmitted per cell;

- SSB beam selection may not be reliably reflecting the UE location, e.g., RSRP measurement of SSB beam in NTN may not work well; geographic area covered by each SSB beam may change, e.g., as SSB beam sweeping or satellite rotating.

- Area bitmap with respect to a common reference location(s)

How it works: An area bitmap w.r.t a common reference location is provided associated to a SMTC. Each bit in the bitmap indicates an area with respective to the reference location. With the origin at the reference location, a bitmap with N bits equally divide 360 degrees into N parts. One or multiple bits of 1 in a 4-bit bitmap can indicate one or multiple areas of north, west, south, east. One or multiple bits of 1 in an 8-bit bitmap can indicate one or multiple areas of NW, WN, WS, SW, SE, ES, EN, NE. One or multiple common reference locations can be provided and each SMTC is linked to one of the reference locations. Alternatively, one common reference location should be enough for all SMTCs, e.g., the serving cell reference location in SIB19 for cell reselection measurement initiation can be reused. If the serving cell is an earth moving cell, the reference location(s) are given for the epoch time and UE can derive the real-time reference location(s) based on serving cell ephemeris. UE can apply the information when it has a valid UE location, determine the area(s) it locates in w.r.t the reference location(s), and apply the associated SMTC(s) to measure the neighbor cells for the selected SMTC(s).

Pros:

- Explicit information for geographic area and easy to be used for UE;

- Small overhead. Multiple SMTCs can use a common reference location. Additionally, only a bitmap of 4 bits or 8 bits is needed for each SMTC.

Cons:

- More calculation for UE than option 1.

Figure 1: SMTC associated with location information.

Observation 1: For SMTC selection in idle/inactive mode, an area bitmap w.r.t a common reference location can achieve a good trade-off between overhead and reliability/efficacy.

Proposal 1: For SMTC selection in idle/inactive mode, discuss and select one approach for the assistance information per SMTC:

- Option 1: a reference location,

- Option 2: an SSB bitmap,

- Option 3: an area bitmap w.r.t a common serving cell reference location.

## SMTC in RRC connected

The principle of SMTC is that the window covers SSB burst position in time so that UE can detect and measure SSBs at each SMTC window. Basically, NW should configure SMTC properly to ensure the cells associated to the SMTC are illuminated during the corresponding SMTC window.

For RRC_CONNECTED, RRM measurement objects and the associated SMTCs are UE dedicated configuration controlled by NW. NW should configure SMTC(s) subject to the UE capability of supported SMTC number (i.e., currently at most 4). We have agreed that at any time UE does not consider more SMTCs than previous release. Hence, at most 4 SMTCs per frequency layer are configured for RRC_CONNECTED.

Proposal 2: For RRC_CONNECTED, the number of SMTCs configured per frequency layer in measurement object is at most 4 as legacy.

To configure SMTC(s) for neighbor cells that are close to UE, NW may need to know an approximate UE location.  From NW perspective, UE location can be obtained or estimated by means of UE assistance information, measurement report, TA report, NW-verified UE location, etc. The following means are supported currently.

- NW configures coarseLocationRequest in UEInformationRequest message, and UE reports location if available in UEInformationResponse message.

- NW configures coarseLocationRequest and/or includeCommonLocationInfo in the measurement report configuration, and UE reports location in periodical or event-triggered measurement report.

- NW configures eventD1/D2 for event-triggered measurement report to track UE distance to a first reference location associated to serving cell and to a second reference location associated to neighbor cell(s).

- NW estimates the UE located area in the serving cell based on the TA report.

- NW derives the UE location via NW-verified UE location.

Based on the reported/estimated UE location, NW can configure SMTC(s) for close neighbor cells only. As in the figure above, UE 1 can be configured with SMTCs for neighbor cell 1, 2, 3, UE 2 can be configured with SMTCs for neighbor cells 4, 5, 6.

Observation 2: For RRC_CONNECTED, NW can configure UE dedicated SMTC(s) for close neighbor cells based on UE location obtained/estimated from measurement report (e.g., event D1/D2, coarseLocationRequest), UE assistance information (e.g., coarseLocationRequest), TA report, NW-verified UE location, etc. No need of additional report of UE location information.

Proposal 3: For RRC_CONNECTED, no need of UE autonomous SMTC selection and no need of additional report of UE location information.

## Backward compatibility for DL CE

RAN1 has agreed that only 160ms is supported in additional to the existing default SSB periodicity of 20ms for initial access. For cell reselection, one open issue is whether to provide separate frequency priorities for legacy UEs and DL-CE capable UEs to prevent legacy UEs to select DL CE cells. In our view, there is no need for such enhancement. The reason are as follows.

1) Legacy UEs are not barred to access DL CE cells. If legacy UE can read MIB/SIB1 of DL CE cell during cell reselection operation, the UE can camp on DL CE cell. If legacy UE cannot read MIB/SIB1 of DL CE cell, the UE considers the cell as barred. Therefore, there is no need to prevent/deprioritize DL CE cells in cell reselection for legacy UE.

2) DL CE cells and legacy cells may be deployed in same frequencies (we already consider to support different SSB/SMTC periodicities on the same frequency), (de)-prioritization in the granularity of frequency cannot work to (de)-prioritize cells in the same frequency.

3) SMTC periodicity configuration {sf5, sf10, sf20, sf40, sf80, sf160} can imply the SSB periodicity of neighbor cells per frequency, and UE can properly choose cells/SMTCs to measure by implementation, e.g., with or without (de)prioritization of cells operating with larger SSB periodicity.

4) If there is a need, NW can provide dedicated frequency priorities (e.g., CellReselectionPriorities in RRCRelease) differently for legacy UE and DL-CE UEs.

Proposal 4: No need to introduce new frequency priority to prevent/deprioritize DL CE cells in cell reselection for legacy UE.

In the last meeting, RAN2 has agreed no need of new barring bit for SSB extension of 160ms, and wait for RAN1 progress to discuss the need of new barring for link level enhancement.

According to RAN1 progress (see Appendix), PDCCH for type0 CSS is repeated in two consecutive slots associated to the same SSB index, PDSCH for SIB1 is repeated in the same slots, and introduce enabling/disabling indication(s) for PDCCH repetition for Type0 CSS and for PDSCH repetition for SIB1 and Msg4. RAN1 has not identify any backward compatibility issue. Legacy UE has no capability of link level enhancements (i.e., PDCCH/PDSCH repetition) anyway and cannot interpret the enabling/disabling indication(s), thus just operates with legacy behavior in initial access. If legacy UE cannot read MIB/SIB1 of a DL CE cell, the cell is considered as barred according to the current specification. Therefore, no backward compatibility issue is seen.

Observation 3: For legacy UE, there is no backward compatible issue in terms of link level enhancement (PDCCH/PDSCH repetition).

Proposal 5: For DL CE PDCCH/PDSCH repetition, no need to introduce new barring bit(s).

# Conclusion

In this contribution we have the following proposals.

Observation 1: For SMTC selection in idle/inactive mode, an area bitmap w.r.t a common reference location can achieve a good trade-off between overhead and reliability/efficacy.

Proposal 1: For SMTC selection in idle/inactive mode, discuss and select one approach for the assistance information per SMTC:

- Option 1: a reference location,

- Option 2: an SSB bitmap,

- Option 3: an area bitmap w.r.t a common serving cell reference location.

Proposal 2: For RRC_CONNECTED, the number of SMTCs configured per frequency layer in measurement object is at most 4 as legacy.

Observation 2: For RRC_CONNECTED, NW can configure UE dedicated SMTC(s) for close neighbor cells based on UE location obtained/estimated from measurement report (e.g., event D1/D2, coarseLocationRequest), UE assistance information (e.g., coarseLocationRequest), TA report, NW-verified UE location, etc. No need of additional report of UE location information.

Proposal 3: For RRC_CONNECTED, no need of UE autonomous SMTC selection and no need of additional report of UE location information.

Proposal 4: No need to introduce new frequency priority to prevent/deprioritize DL CE cells in cell reselection for legacy UE.

Observation 3: For legacy UE, there is no backward compatible issue in terms of link level enhancement (PDCCH/PDSCH repetition).

Proposal 5: For DL CE PDCCH/PDSCH repetition, no need to introduce new barring bit(s).

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

# Appendix

RAN1#120bis Agreements

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2.

- Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index

    - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot.

Note: further discuss the potential solution for M=1 and M=1/2.

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1:

- Enabling/disabling using reserved bit(s) in PBCH payload

  - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

Agreement

When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition.

- UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted.

- Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI

- The two associated PDSCHs have the same RV

FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination.

Agreement

For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options:

- Option 1: Using reserved bit(s) in PBCH payload.

- Option 2: Using scheduling PDCCH.

- Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Agreement

For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options:

- Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0

  - FFS: indication details.

  - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB)

- Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition.

- Option 3: The enabling/disabling is indicated by SIB1 configuration

  - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB)

FFS: Whether UE reports its capability

Agreement

For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots:

- The same resource allocation is assumed for all repetitions

- The supported repetition factors are: 2 and 4

  - The network configures a single value between 2 and 4 at a given time

- The RV cycling is used for each repetition

  - If N=4:

| Starting RV | RV applicable to the nth repetition/transmission |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |