3GPP TSG-RAN2 Meeting #130	R2-2504223

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

In RAN2 #127, the following agreements were made regarding random access for SBFD operation.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis, the following agreements were made regarding random access for SBFD operation.

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2 #128 made agreements related to random access for SBFD operation.

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

RAN2 #129 made agreements related to random access for SBFD operation.

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

RAN #130 has the following agreements related to random access of SBFD operation.

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we would like to discuss the configurations of random-access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

RAN2 #129bis meeting made the following agreements that ‘Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.’. For RACH on SBFD resource for LTM, the following proposal was made by the rapporteur in the post meeting email discussion.

[Proposal for RRC-2] RAN2 to decide whether ‘Early UL synchronization with an LTM candidate cell’ and/or ‘RACH-based LTM’ can be supported in SBFD symbols and the RRC signallings if to support. (based on companies’ meeting contributions, 11/16 companies prefer not to support.)

In our view, there are two issues on whether supporting RACH on SBFD resource for LTM. One is whether RRC signaling is enhanced to support early UL synchronization with an LTM candidate cell on SBFD resource, while the other one is MAC CE enhancement to support CFRA triggered by the MAC CE for RACH-based LTM cell switch on SBFD resource.

For the latter one, it is similar to the PDCCH order triggered CFRA with an RO type indication to indicate UE performing on additional-RO. The source cell can indicate the type of RO in the LTM cell switch command MAC CE. Thus, we propose that an RO type indication is added in LTM cell switch command MAC CE to indicate whether RACH triggered by MAC CE can be performed on additional-RO.

For early UL synchronization with an LTM candidate cell, candidate cell may send EarlyUL-SyncConfig to source cell. Whether the candidate cell indicates type of RO in EarlyUL-SyncConfig to source cell or not is related to whether the EarlyUL-SyncConfig can be extendable to include the SBFD time and frequency information to source cell/UE and how to indicate the type of RO from the candidate cell may involve other WGs further study. Even if the source cell triggers UE to perform early UL synchronization on the additional-RO, the current enhancement on the PDCCH order to indicate the RO type to UE is only for the source cell but not for the candidate cell which may also need RAN1 further check.

Proposal 1: An RO type indication is added in LTM cell switch command MAC CE to indicate whether RACH triggered by MAC CE can be performed on additional-RO.

RAN1 #120 discussed two options whether the parameter ssb-SharedRO-MaskIndex should be shared, or separate parameter configured for the additional-ROs for RACH configuration option 1.

Agreement For RACH configuration Option 1, down-select from the following two alternatives: - Alt-1: Shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. - Alt-2: Separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs.

RAN1 #120bis further made the following conclusion regarding the PRACH mask index and left this issue for RAN2 to decide.

Conclusion For RACH configuration Option 1, there is no consensus in RAN1 to support separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs under the same FeatureCombinationPreambles configuration and it’s up to RAN2 to decide whether to support separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs.

The parameter ssb-SharedRO-MaskIndex indicates subset of ROs where preambles are allocated for this feature combination, FeatureCombinationPreambles. If the field ‘ssb-SharedRO-MaskIndex’ is absent, all ROs configured in RACH-ConfigCommon are shared can be used for that feature combination. The feature combination can be configured either using preamble partitioning within the ROs of the RACH-ConfigCommon or using separate ROs by additional PRACH configuration ‘AdditionalRACH-Config-r17’.

In our view, Alt-2 gives more flexibility than Alt-1 such that additional-ROs are masked in a different way than the legacy-ROs. However, the feature combination is still indicated using the same set of preambles in both RO types. So, the flexibility is more limited to RO selection rather than preambles partitioning. And this flexibility is only when SSB is mapped to multiple ROs. Thus, Alt 1 is sufficient.

Proposal 2: For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs.

Regarding whether the additional featureCombinationPreamblesList configurations for additional-ROs and legacy-ROs is needed or not, it is beneficial to use single FeatureCombinationPreambles of one particular feature combination with the same set of preambles for both RO types.

Proposal 3: Separate featureCombinationPreamblesList configuration for Additional-ROs and Legacy-ROs is not supported.

When a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there may be a UE power gap due to the different power ramping steps configured in different RACH configurations.

RAN1 #120bis has the following conclusions.

Conclusion For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

From RAN2 perspective, UE would pick up the appropriate power control parameters based on the type of RO of the PRACH transmission. For example, if the RO type is initially selected as the additional-RO and switch to legacy-ROs after certain times of PRACH transmission, the UE would to use the preambleReceivedTargetPoweradditional-RO and PreamblePowerRampingStepadditinal-RO . Then, when UE switches to legacy-RO after number times of PRACH attempts, UE would to switch to the power control parameters of the legacy-ROs and apply a power offset to compensate for the difference of the power step size and number of PRACH attempts.

Poffset=N×powerRampingStepadditional-RO-powerRampingStepLegacy-RO

The UE behavior is similar to the existing power offset used when 2-step fallback to 4-step RACH

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure: 3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

### 5.1.3 Random Access Preamble transmission The MAC entity shall, for each Random Access Preamble: <omit> 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

Proposal 4: When UE switches to the other type of RO from the initial selected type of RO, a power offset given by the difference between the two quantities of preamble power step sizes is added.

Regarding PRACH transmission repetition, RAN1 #118 made the following agreements regarding the PRACH transmission with preamble repetitions.

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-Ros and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases.

RAN1 #118bis had a conclusion.

Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported.

RAN1 #119 made the following agreements regarding the PRACH transmission repetition.

Agreement Confirm the following working assumption Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Based on RAN1 agreements, it is supported that PRACH transmission with preamble repetitions can be configured within the additional-ROs. Besides the Msg1 repetition, network may configure the sets of RACH resource associated with different feature combinations in the RACH partitioning framework.

For RACH configuration option 2, the RSRP thresholds for Msg1 repetition and number of Msg1 repetition numbers for PRACH repetitions are separately configured for additional-ROs and legacy-ROs. The value of those parameters may be different in two types of ROs. For RACH configuration option 1, the RSRP thresholds for Msg1 repetition are also separately configured. So, the conditions for Msg1 repetition and the corresponding Msg1 repetition number are different for additional-ROs and legacy-ROs.

Since the number of the Msg1 repetition are evaluated in the RACH resources selection step, it is simpler to select RO type (additional-ROs and legacy-ROs) before the RACH resource set selection than after. If RO type selection happens after the set of RACH resource selection for the feature combinations, UE has to evaluate and determine the RACH resource of feature combinations on both types of ROs. This is not aligned with the current RO type selection criteria agreed in RAN2 (e.g., network indication, RSRP threshold, configured priority, and etc..).

Further, it may happen that different feature combinations are selected on the additional-ROs and legacy-ROs and it may cause unnecessary complexity of RACH resource selection procedure. Considering the simplicity of RACH procedure and minimal specification impact, RAN2 can confirm the working assumption made in RAN2 #129bis, i.e., the RO type selection should be performed before the selection of the set of RACH resource associated with the feature combinations.

Proposal 5: RAN2 confirms that the RO type selection between the additional-ROs and legacy-ROs should be performed before the selection on the set of RACH resources associated with the feature combinations.

For the RACH configuration option 2, the RACH resource partitioning can be separately configured in additional-ROs and legacy-ROs. Network may configure RACH resources for certain feature combinations on legacy-ROs and may not configure RACH resources for those feature combinations in additional-ROs (or vice versa).

If RAN2 can confirm the Proposal 5 that RO type selection is performed before the RACH resource set selection, the availability checking and RACH resources selection should be limited to the selected RO type. Then, UE may lose some opportunities to select some feature combinations configured on the non-selected RO type.

However, how to configure the RACH resource partitioning in a given RACH resource type (SBFD or non-SBFD) is up to network implementation. Network can decide to configure the RACH resource associated with the feature combinations on one type of RO, on the other type of RO or on both types.  Cross-checking the RACH resource partitioning resource in both types of ROs may have too much complexity for the RACH resource partitioning selection procedure and it is not preferred.

Proposal 6: Upon RACH initiation, if UE has met the conditions of selection on one type of RO, UE does not need to evaluate the set of RACH resources of the feature combinations configured in the other type of ROs.

In case of PRACH attempt failures, UE may perform PRACH re-attempt. In RAN2 #127bis, it has been agreed that ‘after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.’

RAN2 #129bis further agreed that ‘RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.’

So, the two ways of RO type switch are both supported and the RO type switch within one RACH procedure is allowed only once.

Observation 1: After the number of PRACH attempts on one type of RO exceeds a threshold, the RO type switch from legacy-RO to additional-RO or from additional-RO to legacy-RO are both supported, and the RO type switch within one RACH procedure is only allowed once.

Given that RACH resources associated with the feature combinations may be configured differently, the selected set of RACH resources associated with the feature combinations in one type of RACH resource may not be applied to the RACH resources of the feature combinations in the other type of RO. Thus, when the RO type switches from one type of RO to the other type of RO, UE needs to evaluate the set of RACH resources of feature combinations configured in the other type of ROs.

Proposal 7: When RO type switches from one type of RO to the other type of RO, UE should evaluate the set of RACH resources of the feature combinations configurated in the other type of RO.

In legacy RACH Msg1 repetition fallback, the fallback is only allowed to the same feature combinations with the increased Msg1 repetition number in order to reduce the complexity of RACH fallback under RACH partitioning framework. In the context of RACH fallback between SBFD RACH and legacy RACH resource, the similar design principle can be applied. The RACH fallback can be allowed when the additional-ROs and legacy-ROs have the same feature combinations. UE is allowed to switch to the other type of ROs with the same feature combinations. If there is no same feature combination on the other type of ROs when performing fallback, it can be for further study.

Proposal 8: For RACH fallback from one type of RO to the other type of RO, at least UE is allowed to switch the type of RO configured with the same feature combinations. FFS the case of no same feature combination configured on the other type of RO when performing RACH fallback.

When PRACH transmission with preamble repetition is configured on both legacy-ROs and additional-ROs, assuming that UE has selected one type of RO and determined the number of preamble repetition for the first PRACH transmission, If PRACH attempts fail, UE should decide when/whether UE switches to higher prepetition number in the same type of RO or switch to the other type of RO.

RAN1 #120bis had the following conclusion and it is up to RAN2 to further study.

Agreement It is up to RAN2 to determine the following issues for PRACH transmission with preamble repetition: - RO group selection for initial PRACH transmission attempt in one RACH procedure - RO group switch for PRACH transmission re-attempt in one RACH procedure

For RACH initiation, RAN2 has already made a working assumption that RO type selection between the additional-ROs and legacy-ROs should be performed before the selection on the set of RACH resources associated with the feature combinations (e.g., preamble repetition).

After UE selects one type of RO and determines the number of preamble repetitions for the first PRACH transmission, UE should be allowed to follow the legacy preamble repetition rule to fall back to higher repetition number in the same type of RO and should not exceed the threshold of the number of PRACH transmission attempts for RACH type switch.

Proposal 9: After UE selects one type of RO and determines the number of preamble repetition for the first PRACH transmission, UE is allowed to fall back to a higher repetition number in the same type of RO but not exceed the threshold of the PRACH transmission attempts for RACH type switch.

After RO type switches when reaching the threshold of the number of PRACH transmission attempts within one RACH procedure, UE should evaluate and apply the number of PRACH transmission attempts in the other type of RO. (as the Proposal 6)

It is important for gNB to know the UE SBFD-aware capability in early stage of the random-access procedure to leverage the SBFD operation. The SBFD-aware indication is useful to inform the UE SBFD capability at the earliest time during RACH, e.g., network may schedule the resource for Msg3 in the SBFD resource in Msg2 if network can be aware the SBFD capable UE performing the RACH.

If network has configured the RACH occasions on SBFD symbols and UE also supports it, the SBFD-aware UE sends the Msg1 in the SBFD RACH occasions as the SBFD-aware indication to network. So, by using the SBFD RACH occasions can be already an implicit way as the SBFD-aware indication.

If the SBFD-aware UE selects the legacy RACH occasion, the additional SBFD-aware indication is needed. If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 10: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

In RACH configuration Option 2, there is possibility that legacy-RO configured in legacy PRACH configuration and additional-RO configured in the SBFD PRACH configuration may be time overlapping and have same start symbol index in the uplink slot. If this time overlapping happens, at least the s_id and t_id (in RA-RNTI formulation) of the overlapping legacy RO and SBFD RO are the same.

In such case, for the f_id (in RA-RNTI formulation), if legacy UE selects the legacy-RO and SBFD-aware UE selects the additional-RO, the RA-RNTI value computation from the existing RA-RNTI formulation may be the same. This is because the f_id is a relative frequency index of the RO, and the legacy-RO and the additional-RO may be picked-up with the same value by two different UEs. This leads to the same RA-RNTI value and confusion for these UE when receiving the same RA-RNTI. The gNB can’t identify the two UEs uniquely via the RA-RNTI.

It also be noted that similar issues have been discussed in other WI before, e.g. network slicing. The conclusion is to rely on gNB implementation to resolve this issue, e.g., by non-time-overlapping RO configuration in time or different SSB to RO mapping for SBFD ROs and legacy ROs. It seems that up to network implementation to resolve issue is the simplest solution.

RAN2 to study either update the RA-RNTI formulation or rely on gNB implementation to avoid overlapping of legacy-RO and additional-ROs in the configuration

Proposal 11: RAN2 to study the RA-RNTI collision issue for RACH configuration Option 2.

In last RAN2 meeting, there were discussions on whether CSI-RS based CFRA can use SBFD resource or not. In our understanding, we don’t see there is an issue to support this feature from RAN2 perspective. If there might be some concerns, RAN2 can further check with RAN1 or other WGs.

Proposal 12: From RAN2 perspective, the CSI-RS based CFRA on SBFD resource is supported.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: After the number of PRACH attempts on one type of RACH resource exceeds a threshold, the RO type switch from legacy-RO to additional-RO or from additional-RO to legacy-RO are both supported, and the RO type switch within one RACH procedure is only allowed once.

Proposal 1: An RO type indication is added in LTM cell switch command MAC CE to indicate whether RACH triggered by MAC CE can be performed on additional-RO.

Proposal 2: For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs.

Proposal 3: Separate featureCombinationPreamblesList configuration for Additional-ROs and Legacy-ROs is not supported.

Proposal 4: When UE switches to the other type of RO from the initial selected type of RO, a power offset given by the difference between the two quantities of preamble power step sizes is added.

Proposal 5: RAN2 confirms that the RO type selection between the additional-ROs and legacy-ROs should be performed before the selection on the set of RACH resources associated with the feature combinations.

Proposal 6: Upon the RACH initiation, if UE has met the conditions of selection on one type of RO, UE does not need to evaluate the set of RACH resources of the feature combinations configured in the other type of ROs.

Proposal 7: When RO type switches from one type of RO to the other type of RO, UE should evaluate the set of RACH resources of the feature combinations configurated in the other type of RO.

Proposal 8: For RACH fallback from one type of RO to the other type of RO, at least UE is allowed to switch the type of RO configured with the same feature combinations. FFS the case of no same feature combination configured on the other type of RO when performing RACH fallback.

Proposal 9: After UE selects one type of RO and determines the number of preamble repetition for the first PRACH transmission, UE is allowed to fall back to a higher repetition number in the same type of RO but not exceed the threshold of the PRACH transmission attempts for RACH type switch.

Proposal 10: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 11: RAN2 to study the RA-RNTI collision issue for RACH configuration Option 2.

Proposal 12: From RAN2 perspective, the CSI-RS based CFRA on SBFD resource is supported.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei