3GPP TSG RAN WG2 Meeting #130                          	   R2-2504244

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on Adaptation of Common Signal/Channel Transmissions

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives for adaptation of common signal and channel transmissions are outlined with the follows [1].

The objectives of the work item are the following: - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g., adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss PEI with paging adaptation in time and BFR using additional ROs with PRACH adaptation in time.

# Discussion

## Adaptation of paging in time domain

At RAN2 #129, the following agreements were made for paging adaptation in time [2].

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

At RAN2 #129bis, the following agreements were made for paging adaptation in time [4].

Agreements on common channel adaptation 1. For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17. 2. For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17. 3. Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

Based on RAN2’s agreements that a separate PEI configuration may be used for Release 19 paging adaptation, the following PEI parameters may be configured seperately for Release 19 paging adaptation.

- po-NumPerPEI: is the number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e., N x Ns. The maximum number of PF associated with one PEI monitoring occasion is 2. With Ns increased to 8, the po-NumPerPEI may also be increased if the max number of 2 PFs is supported.

- firstPDCCH-MonitoringOccasionOfPEI-O: is the offset in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O. When Release 19 paging adaptation is supported, a separate PEI-O may be needed for Release 19 paging adaptation. However, this can be RAN1’s final decision

Proposal 1. A new PEI configuration supporting Release 19 paging adaptation contains at least the following parameters

- po-NumPerPEI-r19

- firstPDCCH-MonitoringOccasionOfPEI-O-r19

Additionally, a new UE capability may be introduced for Release 19 PEI using Rel 19 paging adaptation in time.

Proposal 2. RAN2 introduces a new UE capability for Rel 19 PEI based on paging adaptation in time.

## Adaptation of PRACH in time domain

At RAN1 #120bis, the following agreements were made for PRACH adaptation in time [3].

Agreement For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs Agreement For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. Agreement For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of - PRACH association period - This PRACH mask applies to every [configured] K SSB RO association pattern period(s) Agreement For adaptation of PRACH in time-domain, for a connected mode UE, - One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

After RAN 2 #129bis, offline email discussion ([POST129b][122][NES] (OPPO)) was made on the following proposal [4].

The following proposals were made from the email discussion [5]:

Proposal 1 R2 not pursue spec impact specifically for the usage of R19 additional RACH resources for CFRA initiated by - PDCCH order for additional RACH; - L3 HO command. - LTM CSC MAC-CE; - PDCCH order for LTM early UL sync; Proposal 2 R2 not pursue spec impact specifically for the usage for R19 additional RACH resources for CFRA initiated by BFR. And taking that as a premise, R2 discuss whether UE does not base on DCI 1_0 of P-RNTI to know the availability of RACH resource indicated in rach-configGeneric for BFR. Proposal 3 R2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

The BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection, which includes rach-ConfigBFR with RACH-ConfigGeneric, ssb-perRACH-Occasion and ra-ssb-OccasionMaskIndex.

Based on RAN1’s agreements, PRACH adaptation in time can be dynamically activated with DCI 1_0 with P-RNTI (i.e., short message DCI) for a validity duration configured. For a UE intended to use the additional ROs with PRACH adaptation in time for BFR, the UE needs to monitor the DCI 1_0 with P-RNTI in order to know if the additional ROs are valid or not.

Observation 1. For BFR, a UE needs to monitor DCI 1_0 with P-RNTI in order to use the additional ROs with PRACH adaptation in time.

Additionally, PDCCH order (DCI 1_0 with C-RNTI) with a new DCI field can be used to indicate RRC Connected UEs the availability of additional ROs associated with PRACH adaptation in time. In this case, the UE needs PDCCH order for using the additional ROs with PRACH adaptation in time.

Observation 2. For BFR, a UE needs to receive PDCCH order to use the additional ROs with PRACH adaptation in time.

# Conclusion

In this contribution, we discussed PEI with paging adaptation in time and BFR using additional ROs with PRACH adaptation in time and concluded with the following observations and proposals.

Adaptation of paging in time domain

Proposal 1. A new PEI configuration supporting Release 19 paging adaptation contains at least the following parameters

- po-NumPerPEI-r19

- firstPDCCH-MonitoringOccasionOfPEI-O-r19

Proposal 2. RAN2 introduces a new UE capability for Rel 19 PEI based on paging adaptation in time.

Adaptation of PRACH in time domain

Observation 1. For BFR, a UE needs to monitor DCI 1_0 with P-RNTI in order to use the additional ROs with PRACH adaptation in time.

Observation 2. For BFR, a UE needs to receive PDCCH order to use the additional ROs with PRACH adaptation in time.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- “R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129, Athens, Greece, February 17th – 21st, 2025

- “Chair notes RAN1#120bis eom0”, RAN1 #120bis, Wuhan, China, Apr. 7th – 11th, 2025

- “R2_129bis_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025

- “R2-250xxxx - Summary of [POST129b][122] [NES] (OPPO)_V20_Rapp”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025