3GPP TSG-RAN WG2 Meeting #130	R2-2504247

St.Julians, Malta, 19th – 23rd May 2025

Agenda item:	8.10.3

Source:	Samsung

Title:	Discussion on NTN, slicing and other open issues

Document for:	Discussion and Decision

# Introduction

One of the objectives of Rel-19 SON/MDT WI [1] is to support SON/MDT enhancements for Intra-NTN mobility. We discuss the remaining issues for this objective.

Another objective of Rel-19 SON/MDT is SON/MDT for network slicing. In this regard, RAN2 has received an LS [2] from RAN3 in which they have identified one scenario and asked RAN2 whether/how it can be addressed and any standard changes are needed for the same, as follows. We discuss this scenario in this contribution.

We also address some open issues from LTE RRC running CR in this contribution.

# Discussion

## MRO for NTN

There could be some issues specific to NTN which may lead to radio link failures. A handover failure may occur if the Timing Advance value that the UE estimated to the target cell is incorrect. Thus including the Timing Advance field of the Timing Advance Report MAC CE, if it was triggered during the random access procedure will be helpful for the networks to identify the exact cause of failure.

Similarly, in order to get the satellite position for the UE to self-precompensate the TA, the network signals the satellite position in SIB19, also called the ephemeris. This ephemeris value has a validity duration, which forces the UE to re-acquire the SIB19 every now and then. Some of the typical cause of failures in NTN are related to the uplink validity. It will be useful to report information related to the uplink validity and ephemeris.

Proposal 1: UE reports following additional information in RLF report for NTN related failures

- Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

- A flag indicating whether T430 had expired when RLF or HOF occurred.

- Information related to satellite position.

## Supporting multiple TACs for NTN

UE logs the TAC of the relevant cells in various SON/MDT reports. CGI-Info-Logging including tracking area code of the cell. Since a NTN cell can have multiple TAC(s), UE may log the tackingAreaList when it is logging CGI-Info-Logging in various SON/MDT reports.

Proposal 2: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

## Slice-based cell reselection and logged MDT

A UE in RRC_IDLE or in RRC_INACTIVE state may perform slice-based cell reselection. Slice based reselection depends on the network configuration and the UE capability.

According to TS 38.304, UE performs slice based cell reselection taking into account of the following:

- NAS provided NSAG information

- Dedicated or Broadcasted slice information, per frequency including NSAG priorities per NSAG.

- Legacy Cell reselection priorities

Slice based cell reselection is basically a prioritisation operation, as all other parameters with respect to the cell reselection are same as that of legacy cell reselection

From the RAN3 LS, they are considering only one scenario as relevant for SON/MDT for network slicing.

RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario:

- A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.

RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

Our understanding is that the existing logged measurement framework is sufficient for the network to handle this scenario.

Let us analyse this based on the usage of RSRP measurements for the cell reselection. RSRQ case will be also similar to this.

A UE camped in a seving cell C1 may perform cell reselection to an equal priority cell C2 (i.e. cell on a frequency which has equal priority) when the RSRP/RSRQ difference between the serving and neighbour cells is greater than or equal to a value configured in the cell reselection parameters from system information or dedicated signalling. For e.g. UE camped in serving cell C1 may move to an equal priority neighbour cell C2 when the difference in RSRP between C1 and C2 is p dBm. For e.g. let us consider the RSRP measurements at the point for cell reselection of C1 is X1 dBm and that of C2 is Y1 dBm, and the RSRP difference is (greater than or equal to) p dBm.

Cell reselection to a higher priority neighbour cell depends mainly on the neighbour cell’s measurements. For e.g. the UE will move from serving cell C1 to a higher priority neighbour cell C2 when the RSRP measurements for the neighbour is greater than some value, Y2 dBm. Cell reselection to a lower priority neighbour cell depends mainly on both the serving neighbour cell’s measurements. For e.g. the UE will move from serving cell C1 to a lower priority neighbour cell C2 when the RSRP measurements for the serving cell is less than that of X3 and the RSRP measurements of the  neighbour is greater than Y3 dBm.

Since the network has an understanding of its own topology and the various parameters, it can know the values of X1, X3 or Y1, Y2, Y3 where the cell reselection to equal priority/low priority/high priority cell reselection occurs.

Since the logged MDT provides the information about the cells where the UE has moved and the measurements in those cells, existing MDT can identify the priorities used for the cell reselection from the logged measurements. If the measurements indicate that UE moved from C1 to C2 when C1’s measurements is X3 and C2’s measurements is Y3, MDT system can understand that the cell reselection is a higher priority cell reselection. Thus if the legacy cell reselection priority was configured such that the priority of C1 is lesser than that of C2, the MDT system can understand that the UE has used NSAG based cell reselection and the NSAG(s) used for the priority calculation.

UE also informs the network the S-NSSAI list that triggered the RRC connection establishment. Network can identify whether the S-NSSAI list that triggered the RRC connection establishment is  (efficiently) supported in the serving cell . This also will help the network to identify the cases where the prioritisation is not efficient.

Observation 1: It is possible to determine the following using the existing logged MDT framework

- Whether legacy cell reselection is used or NSAG based cell reselection is used

- The NSAG(s) used for the NSAG based cell reselection

- Whether the UE has reselected to an optimal cell supporting the NSAG(s) triggering the NSAG based cell reselection.

Based on the above observation, we can conclude that the enhancements for logged measurements are not required for slice-based cell reselection. RAN2 can inform the RAN3 that the scenario can be supported without any enhancements.

Proposal 3:RAN2 to inform RAN3 that it is possible to address the scenario of UE supporting slice-based cell reselection not finding any suitable cell in the frequencies corresponding to the highest ranked NSAG, without any enhancement of logged MDT.

## Open Issues in TS 36.331

In LTE RRC specification, rapporteur has captured the below Open issue.

=>Proposal 1: It is proposed RAN2 to discuss the open issue LTERRC-1, i.e. whether to keep the following condition or not (it has been captured in section 5.6.13a.3 in running 36.331 CR):

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

As we have commented in the CR discussions, the condition will always be true before SCG Failure in (NG)EN-DC, since ReconfWithSync is present during PSCellChange, PSCellAddition and SCG resume based on the definition in TS 38.331.  We understand that there is no difference of opinion regarding this, during the discussion. Therefore, there is no need for this condition.

We understand that this condition is harmless and is present in NR RRC specification. While it can be argued that it is a bit too late for correction in TS 38.331, there is no need to port the error to TS 36.331, when we are still in review phase.

Proposal 4: For LTERRC-1, the following condition is not required:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

There is another open issue captured in TS 36.331

=>Proposal 3: It is proposed RAN2 to discuss the open issue i.e. whether to have a single container to carry all the fields (PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800) or not.

Since each of perRAInfoListNR, perRAInfoList-v1660-NR, perRAInfoList-v1840-NR are optional, if we use a single container, the network cannot understand which IEs are present. Similarly, it is also not possible to ensure that the order of decoding the IEs in the network is the same as the order in which IEs are encoded in the UE with a single container.

Observation 2: With a single container, optional presence and order of each of perRAInfoListNR, perRAInfoList-v1660-NR, perRAInfoList-v1840-NR, can’t be communicated to the network.

Proposal 5: For LTERRC-3: PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 are carried in separate containers.

# Conclusion

Based on the discussion, we have the following observation and proposals.

## MRO for NTN

Proposal 1: UE reports following additional information in RLF report for NTN related failures

- Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

- A flag indicating whether T430 had expired when RLF or HOF occurred.

- Information related to satellite position.

Proposal 2: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

## Slice-based cell reselection and logged MDT

Observation 1: It is possible to determine the following using the existing logged MDT framework

- Whether legacy cell reselection is used or NSAG based cell reselection is used

- The NSAG(s) used for the NSAG based cell reselection

- Whether the UE has reselected to an optimal cell supporting the NSAG(s) triggering the NSAG based cell reselection.

Proposal 3:RAN2 to inform RAN3 that it is possible to address the scenario of UE supporting slice-based cell reselection not finding any suitable cell in the frequencies corresponding to the highest ranked NSAG, without any enhancement of logged MDT.

## Open Issues in TS 36.331

Proposal 4: For Open issue LTERRC-1: the following condition is not required.

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

Observation 2: With a single container, optional presence and order of each of perRAInfoListNR, perRAInfoList-v1660-NR, perRAInfoList-v1840-NR, can’t be communicated to the network.

Proposal 5: For Open issue LTERRC-3: PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 are carried in separate containers.

# References

[1] RP-234038: New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4.

[2] R3-247908: LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.