3GPP TSG-RAN WG2 Meeting #130		R2-2504248

Malta, MT, 19th – 23rd May. 2025

Source:	China Telecom

Title:	Discussion on asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In last meeting, RAN2 has reached the following agreements.

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE. RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA. We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording. From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE. For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario. For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

In this contribution, we will discuss potential RAN2 specific impacts of asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

2.1 MAC CE for TCI State activation/deactivation

RAN 1 has made the following conclusion and one remaining MAC issue is that whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP.

Conclusion RAN1#117 For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

As specified in clause 6.1.3.70 of TS 38.321, the Rel-18 legacy activation/deactivation MAC CE for joint TCI States can support up to two joint TCI state for each codepoint, which can be well adapted to asymmetric scenarios. As specified in clause 6.1.3.71 of TS38.321, the Rel-18 legacy activation/deactivation MAC CE for separate TCI States can support {up to two DL TCI states + up to two UL TCI states}, which can cover the combination of {one DL TCI state + up to two UL TCI state}. And NW can control the number of activated DL/UL TCI states by setting the value of Fi,j/Si,j field, e.g, setting {Fi,0 =1, Fi,1=0,  Si,0 =1, Si,1=1} to achieve {one DL TCI state + two UL TCI state} for i-th codepoint. Based on the above discussion, we think the Rel-18 legacy activation/deactivation MAC CE can work for asymmetric DL sTRP/UL mTRP with no enhancement.

Proposal 1: The Rel-18 legacy activation/deactivation MAC CE can work for asymmetric DL sTRP/UL mTRP with no enhancement.

2.2 Asymmetric UL mTRP in LTM cell switch

In Rel-17, inter-cell mTRP cooperative transmission scheme is supported. In addition to the SSB of serving cell, NW can dynamically activate a set of SSB information from neighboring cell. Additionally, RAN1 has also confirmed that the UL-only TRP may transmit SSB. If the neighboring cell happens to be an LTM candidate cell, then the NW can obtain the corresponding pathloss offset.

Take the following case into account, if NW does not obtain the pathloss offset when configures the candidate LTM cell, but obtains the pathloss offset for the LTM target cell when it is capable to switch to, then it is beneficial to carry the corresponding pathloss offset in LTM cell switch command.

Proposal 2: It is beneficial to introduce PL offset in the LTM cell switch MAC CE if PL offset does not include in LTM Config but NW can provide it before the LTM cell switch procedure is applied.

- Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1: The Rel-18 legacy activation/deactivation MAC CE can work for asymmetric DL sTRP/UL mTRP with no enhancement.

Proposal 2: It is beneficial to introduce PL offset in the LTM cell switch MAC CE if PL offset does not include in LTM Config but NW can provide it before the LTM cell switch procedure is applied.