3GPP TSG-RAN WG2 #130	R2-2504256

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.11.3

Source:	Ericsson

Title:	CSI-RS measurements in SBFD

Document for:	Discussion

# 1	Introduction

In this paper, we discuss CSI-RS measurements for RLM/BFD/CBD and for L3 mobility in SBFD.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

## 2.1	CSI-RS measurements for RLM/BFD/CBD

RAN1 has sent a LS reply in “R1-2501560 - Reply LS on CSI-RS measurement with SBFD operation”. The relevant content is:

Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

Based on the LS reply, the below observations can be made:

Configuration 1 and Configuration 2 on UL transmissions and DL reception across SBFD symbols and non-SBFD symbols in different slots are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement.

UE can provide separate CSI reports to the gNB according to the configured valid symbol type in the corresponding CSI-ReportConfig.

RAN1 doesn’t indicate whether the valid symbol type can be included in RadioLinkMonitoringConfig and BeamFailureRecoveryConfig.

Since RAN1 has not indicated whether the valid symbol type can be included in RadioLinkMonitoringConfig and BeamFailureRecoveryConfig, RAN2 can assume that there is no enhancement needed for RLM/BFD/CBD measurements unless RAN1/RAN4 indicates to RAN2 for further actions. Even without any enhancement for RLM/BFD/CBD measurements, the gNB can provide proper settings (e.g., beamFailureDetectionTimer and/or beamFailureInstanceMaxCount) to the UE to accommodate differences between CSI-RS measurement results in legacy symbols and in SBFD symbols.

No enhancement is needed for CSI-RS based RLM/BFD/CBD measurements.

In the MAC spec, the MAC entity monitors BFIs. When the number of consecutively occurred BFIs reaches the maximum number, the UE triggers BFR. There is no need for the MAC entity to trigger BFR for non SBFD symbols and SBFD symbols separately.

Keep the legacy RLM, BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

## 2.2	CSI-RS measurements for L3 mobility

In RRC, L3 mobility related RRC IEs can be observed as the below

It can be observed that

- Measurement results for each cell comprises both cell results and beam results

- Cell results may base on the best beam or averaged among best beams which are above the threshold

- Measurement results may base on either SSB or CSI-RS.

If SSB is configured as the measurement RS, there is no impact on handover due to SBFD operation.

If SSB is configured as the measurement RS, there is no impact on handover due to SBFD operation.

Regarding CSI-RS based L3 mobility, two possible options for the way forward:

Option 1 – the UE is provided with the SBFD configuration of neighbor cells. The UE may provide cell measurement results and beam measurement results for SBFD symbols and non SBFD symbols separately.

The potential spec impact for Option 1 would require cross-working group impacts (impacts to RAN1, RAN2 and RAN4). Serving cell/gNB needs to signal SBFD T/F configuration of each neighbor cell to UEs via dedicated RRC signaling. The UE measures each neighbor cell considering its SBFD T/F configuration.

Option 2 – the UE is not aware of the SBFD T/F configuration of neighbor cells.

The UE measures each neighbor cell without knowing SBFD T/F configuration of neighbor cells, i.e., as if the cell is not configured with SBFD.

In this case, the SBFD aware UE would behave as if the UE were a legacy UE.

Comparing the two options, we think option 2 is sufficient, given there is no TU left for other working groups to handle this issue if RAN2 would go for Option 1.

Enhancement for L3 mobility CSI-RS measurement is not pursued.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Configuration 1 and Configuration 2 on UL transmissions and DL reception across SBFD symbols and non-SBFD symbols in different slots are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement.

Observation 2	UE can provide separate CSI reports to the gNB according to the configured valid symbol type in the corresponding CSI-ReportConfig.

Observation 3	RAN1 doesn’t indicate whether the valid symbol type can be included in RadioLinkMonitoringConfig and BeamFailureRecoveryConfig.

Observation 4	If SSB is configured as the measurement RS, there is no impact on handover due to SBFD operation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No enhancement is needed for CSI-RS based RLM/BFD/CBD measurements.

Proposal 2	Keep the legacy RLM, BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

Proposal 3	Enhancement for L3 mobility CSI-RS measurement is not pursued.

# 4	References

- [1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Shanghai, China, June 17-20, 2024, RAN#104.

[2] R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, MediaTek, Athens, Greece, February 17th – 21st, 2025, RAN1#120.