3GPP RAN WG2 Meeting #130	R2-2504269

St.Julians, Malta May 19th – 23rd, 2025

Agenda Item:	8.6.4

Source:	Ofinno

Title:	Discussion on open issues of CLTM

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss open issues on conditional LTM.

# Identified open issues:

## Target cell selection (RRC-3)

A network may configure a L1 execution condition for an CLTM candidate cell A and a L3 execution condition for an LTM candidate cell B. A UE evaluates both execution conditions. the UE may determine both execution conditions being fulfilled. RRC performs filtering mechanism on the L3 measurements which makes the L3 measurements more reliable and less sporadic. Thus, the L3 execution condition is more reliable than the L1 execution condition. It is beneficial for UE to execute  CLTM  to candidate cell B.

Proposal 1: (RRC-3) When more than one CLTM triggered candidate cells exist, a UE selects a candidate cell associated with L3 execution condition.

## TA timer of candidate cell (MAC-14)

A UE may be configured with up to 8 candidate cells for CLTM. UE may perform early UL sync to the candidate cells for CLTM. Based on performing the early UL sync, the UE may receive TA value for the candidate cells. The UE starts a TA timer corresponding to each candidate cell for which TA value is received. If The UE satisfies an execution condition associated with the candidate cel, the UE may execute CLTM to one of these candidate cells, for which TA timer is running. The candidate cell becomes the target PCell. While executing CLTM to the target PCell, the UE stops the TA timer of this candidate cell target PCell and starts TA timer for PCell with the remaining value of the TA timer of this candidate cell. The UE continues the TA timer for the other candidate cells.

RAN2 agreed that network can send an LTM cell switch command MAC CE indicating a candidate cell for CLTM. The candidate cell may already have a TA timer running based on performing early UL sync procedure. The TA value of the candidate cell is also known to the network. Further, the network may have its own TA management mechanisms e.g., the network may maintain the TA of the candidate cell and knows whether the TA value of the candidate cell is valid or not. The LTM cell switch command MAC CE can indicate a TA value for the candidate cell. If the network determines the TA time value of the candidate cell to be valid, then the network indicates TA value in the LTM cell switch command MAC CE.

Observation 1: (MAC-14) If TA of a candidate cell is valid, the LTM cell switch command MAC CE includes the TA value (i.e., Timing Advance Command field is not set as FFF).

It is possible that the network determines that the TA value of the candidate cell is no longer valid. When a cell switch to the candidate cell is initiated, the network sends LTM cell switch command MAC CE indicating that TA value is not valid (i.e., Timing Advance Command field is set as FFF).

Observation 2: (MAC-14) If TA of a candidate cell is not valid, the LTM cell switch command MAC CE does not include the TA value (i.e., Timing Advance Command field is set as FFF).

Some companies proposed to use the TA value of the candidate cell, maintained on the UE based on receiving the TA value during/ after early UL sync procedure, if LTM cell switch command MAC CE is received for the candidate cell while TA timer for the candidate cell is running. The Timing Advance Command field is always included in the LTM cell switch command MAC CE i.e., it is not an optional field. The network always includes the Timing Advance Command field in an LTM cell switch command MAC CE. This means that the network always provides either a valid TA value (i.e., Timing Advance Command field is not set as FFF) or an indication that TA is not available ((i.e., Timing Advance Command field is set as FFF). If a network does not include TA value for the candidate cell, it means that that network sends the LTM cell switch command MAC CE with Timing Advance Command field set to FFF. If the network already has valid TA value for the candidate cell, there is no reason why the network would not include this in the LTM cell switch command MAC CE (i.e., there is no signalling gain or complexity reduction by not including the TA value). This value of the Timing Advance Command field already has a defined meaning/ purpose – to indicate to UE that a valid TA value is not included. The network may use this if the TA value is no longer valid. If the UE executes LTM cells switch to the candidate cell using the TA value received prior to receiving the LTM cell switch command, the LTM execution can fail.

Proposal 2: (MAC-14) If UE receives LTM cell switch command MAC CE for LTM to a candidate cell while TA timer for the candidate cell is running, the UE uses the TA value indicated in the LTM cell switch command MAC CE and not the TA value received prior to receiving the LTM cell switch command MAC CE.

RAN2 agreed that when an LTM or CLTM is executed to a candidate cell, the UE continues (i.e., does not stop) the TA timer for other candidate cells, if running. The candidate cells are maintained on the UE until they are released by the network, or until a condition to release the candidate cells is met e.g., RRC release procedure, MR-DC release etc. We think it is beneficial to extend the same principle to any mobility mechanism i.e., to any use case where the UE performs a reconfiguration with sync procedure. This can include, for the MCG – a PCell handover, a CHO etc., – and for SCG – a PSCell change, SCPAC etc.

Proposal 3: (MAC-14) TA timer for candidate cell is continued (not stopped) when MAC reset is performed due to a reconfiguration with sync procedure.

A TA value of a candidate cell can be used only when the candidate cell is still available for CLTM. If a candidate cell configuration of the candidate cell for CLTM is released, the UE can no longer use the TA value associated with this candidate cell. After the candidate cell configuration is released, it is possible that the UE is configured with the same candidate cell configuration ID. If this happened while the TA timer for the candidate cell for which the candidate cell configuration is released is still running, this may cause problems with accessing the candidate cell during LTM cell switch. We think it is beneficial to stop the TA timer of the candidate cell when the corresponding candidate cell configuration is released.

Proposal 4: (MAC-14) Stop TA timer or a candidate cell when the candidate cell configuration corresponding to the CLTM candidate cell is released.

# Other open issues

## CLTM execution

In Rel-18, RAN2 agreed that a UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message. For CLTM, a UE needs to include the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

A candidate cell may be configured for a normal LTM and a conditional LTM. The same LTM configuration identifier of the candidate cell is configured for the normal LTM and the conditional LTM. When a network receives an RRCReconfigurationComplete message including the applied LTM candidate configuration identifier, the network may not be able to know which LTM between the normal LTM and the CLTM the UE executed. For example, the network may transmit an LTM cell switch command for the normal LTM to a UE, but the UE executes the CLTM (e.g., without receiving the LTM cell switch command). Based on executing the CLTM, the UE transmits an RRC reconfiguration complete message comprising the applied LTM candidate configuration identifier. The network receiving the applied LTM candidate configuration identifier may consider that the UE executed the normal LTM while the UE executed the CLTM. The network may update/release LTM configuration parameters. The network may update/release wrong LTM configuration parameters.

Proposal 5: RAN2 to discuss whether a UE executing CLTM includes a CLTM indication within the RRCReconfigurationComplete message.

## Fast MCG failure information

If a UE is configured with timer T316 and the UE detects a failure, the UE initiates MCG failure information procedure. When MCG failure information procedure is triggered, the UE stops CLTM evaluation and starts the timer T316. Since CLTM supports subsequent cell switching, the UE needs to restart or resume CLTM evaluation at some point. There is no procedure based on which the UE can resume or restart CLTM evaluation. We think the UE needs to restart/ resume the CLTM evaluation on successfully recovering from the MCG failure. The successful recovery can be determined based on receiving an RRC reconfiguration message for the MCG, while timer T316 is running.

There are multiple ways to handle this. In a first option, the UE stops CLTM evaluation on initiating MCG failure information procedure and resumes CLTM evaluation if RRC reconfiguration for MCG is received while T316 is running. This requires clear handling of starting and stopping of the CLTM evaluation. The purpose of stopping CLTM evaluation during MCG failure recovery procedure is to avoid a case where the UE executes a CLTM while the MCG failure recovery is in progress, as this can lead to misalignment between the UE and the network. In a second option, the UE executes CLTM only if MCG failure is not in progress. This is simpler in stage 3 as clear handling of starting and stopping of CLTM evaluation is not required in the specification.

Proposal 6: UE resumes CLTM evaluation after successfully recovery from MCG failure based on one of the following options:

- Option 1: resume CLTM evaluation if RRC reconfiguration for MCG is received while T316 is running;

- Option 2: executing CLTM only when timer T316 is not running.

## CG resource for CLTM

A network may allocate a configured grant (CG) resource when transmitting to a UE an LTM configuration. The network may need to keep allocating the CG resource of a candidate cell until the UE execute an LTM cell switch procedure for the candidate cell. The UE may not execute the LTM cell switch procedure for the candidate cell. The CG resource is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

Observation 3: a CG resource of a candidate cell for CLTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

The network may need to release or update the CG resource (e.g., to avoid the waste of the CG resource). For releasing or updating the CG resource, the network needs to transmit an RRC reconfiguration message comprising an LTM configuration. It causes signal overheads and is not desired for subsequent CLTM without the RRC reconfiguration.

Proposal 7: RAN2 discuss dynamic management of CG resource of candidate cell for CLTM.

## MAC CE for early TA acquisition

In RAN2 #128, RAN2 agreed that a network may transmit new MAC CE to inform the candidate cell’s TA information to UE via LTM Candidate Timing Advance Command MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. The network can transmit the LTM Candidate Timing Advance Command MAC CE in response to receiving a RA preamble from a UE for early UL sync procedure. The UE may have performed early UL sync for multiple candidate cells. As a result, the network may have more than one TA values to be sent to the UE i.e., 1 TA value per candidate cell. The network may transmit the LTM Candidate Timing Advance Command MAC CE to manage the TA of a candidate cell (e.g., for restarting TA timer or updating TA value). To improve the efficiency of signaling and simplify downlink scheduling, it is beneficial for the network to transmit the LTM Candidate Timing Advance Command MAC CE to with more than one TA values where each TA value corresponds to a different candidate cell.

Proposal 8: LTM Candidate Timing Advance Command MAC CE to include more than one TA values.

In RAN2 #129, RAN2 agreed that the UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. It is also understood that the UE completes an RA procedure after transmitting an RA preamble for early uplink sync, similar to Rel-18 LTM. If at least one execution condition is fulfilled for a candidate cell and if the UE has completed RA procedure for the candidate cell but has not yet received the MAC CE informing the candidate cell’s TA, then the UE has two options:

Option 1: UE does not have a valid TA for the candidate cell, since it has not received the MAC CE informing the candidate cell’s TA. So, the UE initiates a RACH-based CLTM cell switch.

Option 2: UE waits for a duration of time to receive the MAC CE informing the candidate cell’s TA. If the duration of time expires and the UE still does not receive the MAC CE, UE initiates RACH-based cell switch. If the UE receives the MAC CE within the duration of time, UE initiates a RACH-less cell switch.

Option 1 has the benefit of slightly simpler implementation, but it also adds overhead in terms of performing two RA procedures (one during early uplink sync and one for RACH-based cell switch) to obtain TA for the candidate cell. It also increases cell switch interruption, which is a drawback, since one of the main goals of LTM is to reduce this interruption. Option 2 has the benefit of using RACH-less cell switch by waiting for the MAC CE. During this waiting time, the UE is still connected to the source cell and is therefore not wasting time. Also, the duration of time can be controlled by the network through RRC signaling, so that network can ensure UE does not wait for an unnecessarily long time. For instance, this duration can also be set to 0, in which case, an identical behavior to Option 1 can also be achieved.

Proposal 9: After transmitting RA preamble for early uplink sync, UE waits for a duration of time to receive the MAC CE. The duration of time can be RRC configured.

# Conclusions

The contribution discussed aspects related to open issues for CLTM. The following observations and proposals are made:

For identified open issues:

Proposal 1: (RRC-3) When more than one CLTM triggered candidate cells exist, a UE selects a candidate cell associated with L3 execution condition.

Observation 1: (MAC-14) If TA of a candidate cell is valid, the LTM cell switch command MAC CE includes the TA value (i.e., Timing Advance Command field is not set as FFF).

Observation 2: (MAC-14) If TA of a candidate cell is not valid, the LTM cell switch command MAC CE does not include the TA value (i.e., Timing Advance Command field is set as FFF).

Proposal 2: (MAC-14) If UE receives LTM cell switch command MAC CE for LTM to a candidate cell while TA timer for the candidate cell is running, the UE uses the TA value indicated in the LTM cell switch command MAC CE and not the TA value received prior to receiving the LTM cell switch command MAC CE.

Proposal 3: (MAC-14) TA timer for candidate cell is continued (not stopped) when MAC reset is performed due to a reconfiguration with sync procedure.

Proposal 4: (MAC-14) Stop TA timer or a candidate cell when the candidate cell configuration corresponding to the CLTM candidate cell is released.

For other open issues:

Proposal 5: RAN2 to discuss whether a UE executing CLTM includes a CLTM indication within the RRCReconfigurationComplete message.

Proposal 6: UE resumes CLTM evaluation after successfully recovery from MCG failure based on one of the following options:

- Option 1: resume CLTM evaluation if RRC reconfiguration for MCG is received while T316 is running;

- Option 2: executing CLTM only when timer T316 is not running.

Observation 3: a CG resource of a candidate cell for CLTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

Proposal 7: RAN2 discuss dynamic management of CG resource of candidate cell for CLTM.

Proposal 8: LTM Candidate Timing Advance Command MAC CE to include more than one TA values.

Proposal 9: After transmitting RA preamble for early uplink sync, UE waits for a duration of time to receive the MAC CE. The duration of time can be RRC configured.