3GPP TSG-RAN WG2 Meeting #129-bis	R2-2504275

St Julian’s, Malta May 19th – 23rd, 2025

Agenda Item:	8.13.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity for Multi-hop Relay

Document for:	Discussion and Decision

# Introduction

In the WID for NR sidelink multi-hop relay in [1], we have the following description for service continuity scenarios for multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In RAN2 128 meeting, the two scenarios Scenario A and B in the first priority were discussed and it was agreed that the Figure 1 and Figure 2 of R2-2410589 [3] can be used as a baseline for intra-gNB multi-hop indirect to direct path switching or intra-gNB multi-hop indirect to single hop indirect path switching procedure to be captured in TS 38.300.

In this paper, we discuss the two scenarios in the second priority, Scenarios C and D, and provide the signalling flows for these two scenarios. Additionally, we also provide the text proposal for scenario C and D in the annex which can be used as a baseline and captured in  TS 38.300.

# Discussion

## 2.1 Intra-gNB direct to multi-hop indirect path switching

Figure 1 shows the procedure for the intra-gNB direct to multi-hop indirect path switching case which is extended from intra-gNB direct to single-hop indirect path switching procedure as defined in clause 16.12.6.2 in TS 38.300.

Figure 1.  Intra-gNB direct to multi-hop indirect path switching procedure

Step 1.	The UE (i.e., L2 U2N Remote UE in next steps)  reports one or multiple candidate L2 U2N Relay UE(s), after it measures/discovers the candidate L2 U2N Relay UE(s)/path. The triggering Event Y1 and Event Y2 can be used in the measurement configuration.

-	Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

-	Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

Step 2.	The gNB decides to switch the L2 U2N Remote UE onto multi-hop indirect path.

Step 3.     The gNB sends the RRCReconfiguration message to the targe L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE) to reconfigure the connection for Remote UE.

Step 4.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the first relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

Step 5.	The L2 U2N Remote UE establishes PC5-RRC connection with the first relay UE.

Step 6.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UEs (last relay, intermediate relay and the first relay UE).

Step 7.	The data path is switched from direct to multi-hop indirect path between the L2 U2N Remote UE and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the L2 U2N Remote UE for lossless delivery during path switch if gNB configures it.

2.1.1 Proposal for Open issue RRC-1: Measure Events for scenario C and D

To summarize, we have the following proposal addressing Open issue 1

Proposal 1: The triggering Event Y1/Y2 can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 2: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

2.1.2 Proposal for TS 38.300

We have the following proposal TS 38.300

Proposal 3: RAN 2 agrees the above procedure in Figure 1 along with the text proposal in the Annex as the baseline for intra-gNB direct to multi-hop indirect path switching procedure and  be captured in TS 38.300.

## 2.2 Intra-gNB single-hop indirect to multi-hop indirect path switching

Figure 2 shows the intra-gNB single-hop indirect to multi-hop indirect path switching procedure which is extended from single hop intra-gNB  indirect to indirect path switching procedure as defined in clause 16.12.6.3 in TS 38.300.

Figure 2.  Intra-gNB single-hop indirect to multi-hop indirect path switching procedure

Step 1.	 The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s), when the sidelink measurement quantity shows deteriorated link quality. The triggering Event Z1 and Event Z2 can be used in the measurement configuration.

- Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

- Event Z2 (if agreed): Candidate L2 U2N Relay UE becomes an offset better than the serving L2 U2N Relay UE

Step 2.	The gNB decides to switch the L2 U2N Remote UE onto multi-hop indirect path.

Step 3.    The gNB sends the RRCReconfiguration message to the target L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE) to reconfigure the connection for Remote UE.

Step 4.   The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the single-hop indirect path via the source L2 U2N relay UE after the reception of the RRCReconfiguration message from the gNB.

Step 5.     The L2 U2N Remote UE establishes PC5-RRC connection with the target First Relay UE.

Step 6.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the targe L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE), using the configuration provided in the RRCReconfiguration message. From this step, the L2 U2N Remote UE uses the RRC connection via the targe L2 U2N multi-hop relay to the gNB.

Step 7.  The gNB sends the RRCReconfiguration message to the source L2 U2N relay to reconfigure the connection between the source L2 U2N relay UEs and the gNB. The RRCReconfiguration message to the source L2 U2N relay UE can be sent any time after step 4 based on gNB implementation (e.g., to release Uu or PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

Step 8.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7.

Step 9.	The data path is switched from the source L2 U2N Relay UE to the target L2 U2N multi-hop Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 7.

2.2.1 Proposal for Open issue RRC-1: Measure Events for scenario C and D

To summarize, we have the following proposal addressing Open issue 1

Proposal 4: The triggering Event Z1 and Event Z2 (if agreed) can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 5: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

2.2.2 Proposal for TS 38.300

We have the following proposal TS 38.300

Proposal 6: RAN 2 agree the above procedure in Figure 2 along with the text proposal in the Annex as the baseline

for intra-gNB single-hop indirect to multi-hop indirect path switching and  be captured in TS 38.300.

## 2.3 Open issue RRC-3: Source relay becomes target for scenario D

In the previous meeting, the following agreement on path switch scenarios were agreed on scenario B.

RAN2#128

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

For scenario D, we need to further discuss whether the target first/intermediate/last relay UE are all new relay UEs which are not the source single-hop U2N relay UE. In fact, the following cases do exist:

When the remote UE switches from intra-gNB single-hop indirect to multi-hop indirect path, the source single-hop relay UE can be the target last relay UE

Remote UE

target first

relay UE

target intermediate relay UE

target last relay UE

(source relay UE)

gNB

Remote UE

Figure 3.  The target last relay UE is the source single-hop U2N relay UE in scenario D

Therefore, the target last relay UE can be the source single-hop U2N relay UE in scenario D. However, the target first/intermediate relay UE cannot be the source single-hop U2N relay UE, based on the tree-like topology architecture.  In this architecture, the first/intermediate relay UE cannot be the source single-hop U2N relay UE because it connects to only one parent relay UE.

Proposal 7: The target last relay UE can be the source single-hop U2N relay UE, but the target first/intermediate relay UE cannot be the source single-hop U2N relay UE.

## 2.4 Multi-hop target relaying path selection

Unlike the conventional single hop direct-to-indirect path switching, the gNB needs to select a multi-hop relaying path based on the measurement reporting, not just a target relay UE,  for the remote UE to do path switching. The following information can help gNB to choose a better target path.

- Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent relay UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

Proposal 8: The gNB can determine the target multi-hop relaying path based on the following information:

- First Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent relay UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

The information in the first bullet could already be contained in the measurement reporting by reusing the Rel-17 mechanism.

The information in the second bullet  is important, since the sidelink measurement quantity on each hop can impact the data forwarding of the remote UE. Based on legacy mechanism, remote UE can only measure and report the sidelink measurement quantity of the first relay UE. Therefore, enhancements are needed to take the sidelink measurement quantity between adjacent relay UEs along the candidate relaying path into consideration when selecting the target relaying path.

One possible solution is each  relay UEs can indicates the sidelink measurement quantity  of its adjacent relay UE (in upstream direction) to the its adjacent relay UE in the downstream direction.  With this method, the first relay UE can get all the sidelink measurement quantity of each hop of the Relay UEs along its relaying path, then it can forward the sidelink measurement quantity to the remote UE in the discovery message. Remote UE can use the individual sidelink measurement quantity to determine candidate relaying path, or just report this information to gNB for target path determination.

Proposal 9: The remote UE can obtain the sidelink measurement quantity information between each  adjacent relay UEs  along the candidate multi-hop relaying path to help determining a better target path. This information can be provided by the first relay UE in the discovery message.

Additionally, if the first relay UE provides its RRC state information in the discovery message and is in the RRC_CONNECTED state, the remote UE can infer that all upstream relays in the multi-hop chain are also in the RRC_CONNECTED state. This enables the remote UE to select an appropriate path where all relay UEs are in the RRC_CONNECTED state, ensuring service continuity for scenarios C and D

Proposal 10: The remote UE can obtain the RRC state information of the first relay UE from the discovery message, enabling it to select an appropriate path where all relay UEs are in the RRC_CONNECTED state.

# Conclusion

In this paper, we discuss the detailed procedures for scenarios C and D, and the enhancements for the path information reporting and path selection.  We have the following observation and proposals.

Additionally, the text proposal for all the 4 scenarios is also provided in the annex.

Service Continuity Procedures

.

Intra-gNB direct to multi-hop indirect path switching

Proposal for Open issue RRC-1: Measure Events for scenario C and D

Proposal 1: The triggering Event Y1/Y2 can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 2: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

Proposal for TS 38.300

Proposal 3: RAN 2 agrees the above procedure in Figure 1 along with the text proposal in the Annex as the baseline for intra-gNB direct to multi-hop indirect path switching procedure and be captured in TS 38.300.

Intra-gNB single-hop indirect to multi-hop indirect path switching

Proposal for Open issue RRC-1: Measure Events for scenario C and D

Proposal 4: The triggering Event Z1 and Event Z2 (if agreed) can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 5: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

Proposal for TS 38.300

Proposal 6: RAN 2 agree the above procedure in Figure 2 along with the text proposal in the Annex as the baseline for intra-gNB single-hop indirect to multi-hop indirect path switching to be captured in TS 38.300.

Open issue RRC-3: Source relay becomes target for scenario D

Proposal 7: The target last relay UE can be the same as the source single-hop U2N relay UE, but the target first/intermediate relay UE cannot be the source single-hop U2N relay UE.

Multi-hop target relaying path selection

Proposal 8: the gNB can determine the target multi-hop relaying path based on the following information:

- First Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

Proposal 9: The remote UE can get the sidelink measurement quantity information between each adjacent relay UEs  and the RRC State information along the candidate multi-hop relaying path to help determining a better target path.

Proposal 10: The remote UE can obtain the RRC state information of the first relay UE from the discovery message, enabling it to select an appropriate path where all relay UEs are in the RRC_CONNECTED state.

# References

[1] RP-242349 Revised WID SL_relay_multihop

[2] 3GPP TS 38.300, V18.3.0, NR; NR; NR and NG-RAN Overall Description

[3] R2-2410589 Discussion on service continuity for Multi-hop Relay

# Annex: Text Proposal

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start Changes>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 16.12.X.3		Intra-gNB direct to multi-hop indirect path switching

For service continuity of multi-hop U2N relay, the following procedure is used, in case of U2N Remote UE switching from direct to multi-hop indirect path under the same gNB:

Figure 16.12.X.3-1.  Intra-gNB direct to multi-hop indirect path switching procedure

- The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s), after it measures/discovers the candidate L2 U2N Relay UE(s)/path.

-	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

-	The reporting includes at least a L2 ID of the first relay UE , the First Relay UE’s serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

2.	The gNB decides to switch the L2 U2N Remote UE onto multi-hop indirect path.

3.	The gNB sends the RRCReconfiguration message to the target L2 U2N multi-hop relay (the first relay UE, intermediate relay and last relay) to reconfigure the connection for Remote UE.

4.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID of the first relay, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the single-hop indirect path via the source L2 U2N relay UE after the reception of the RRCReconfiguration message from the gNB.

5.	The L2 U2N Remote UE establishes PC5-RRC connection with the target First Relay UE.

6.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the target L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE.

7.	The data path is switched from direct to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

#### 16.12.X.4		Intra-gNB single-hop indirect to multi-hop indirect path switching

For service continuity of multi-hop U2N relay, the following procedure is used, in case of multi-hop U2N Remote UE switching from single-hop indirect to multi-hop indirect path under the same gNB:

Figure 16.12.X.4-1.  Intra-gNB single-hop indirect to multi-hop indirect path switching procedure

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s), when the sidelink measurement quantity shows deteriorated link quality.

-	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

-	The reporting includes at least a L2 ID of the first relay UE , the First Relay UE’s serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

2.	The gNB decides to switch the L2 U2N Remote UE onto multi-hop indirect path under the same gNB.

3.   The gNB sends the RRCReconfiguration message to the target L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE) to reconfigure the connection for Remote UE .

4.   The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the single-hop indirect path via the source L2 U2N relay UE after the reception of the RRCReconfiguration message from the gNB.

5.  The L2 U2N Remote UE establishes PC5-RRC connection with the target First Relay UE.

6.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the targe L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE.

7.  The gNB sends the RRCReconfiguration message to the source L2 U2N relay to reconfigure the connection between the source L2 U2N relay UEs and the gNB. The RRCReconfiguration message to the source L2 U2N relay UE can be sent any time after step 4 based on gNB implementation (e.g., to release Uu or PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

8.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7.

9.	The data path is switched from the source L2 U2N Relay UE to the target L2 U2N Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 7.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End Changes>>>>>>>>>>>>>>>>>>>>>>>>>