3GPP TSG-RAN WG2 Meeting #130	R2-2504280
Malta, May 19 – 23, 2025

Agenda Item:	8.5.2

Source:	InterDigital

Title:	On demand SSB transmission for SCell

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]. In RAN2#127 to 129bis, the objective was discussed and the following was agreed with regards to MAC CE signalling of OD-SSB:

Agreements on OD-SSB indication

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this contribution, on-demand SSB activation/deactivation is discussed along with the MAC CE design to indicate it.

# Discussion

For CA deployments, SCells with SSB-less Scell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less Scell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-base CA case, the NW signals to the UE which cell is the reference cell (e.g. a Pcell or another Scell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the Scells operate in FR2 while other cells in FR1. For energy saving purposes in CA, it is important for the Scells to operate in a NES mode as long as possible without or with limited SSB transmission.

## Signalling for OD-SSB reception

Scells which are used as capacity cells and operating in NES mode are not expected to transmit SSBs or only transmit sparse SSBs. gNB initiated triggering of OD-SSB in the context of Scell activation procedure has been agreed by R1. Per R2 agreement, reception of on-demand SSB on an SCell is indicated by a new MAC CE signalling separate from the legacy MAC CE for SCell (de)activation. Further, since the activation of the SCell itself is based on the network’s decision, i.e. based on the reception of UE measurement and L1 CSI, the activation decision may lag the indication for the reception of an OD-SSB. For scenario 2A where the OD-SSB and SCell are indicated and activated together, R2 agreed to not introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication.

Given R1 agreed to indicate a value associated with the OD-SSB periodicity, R2 can assume that there will be an additional field to indicate the periodicity index from the values configured in RRC. RAN1 agreed that no new periodicity configuration values will be introduced in Rel-19 for the OD-SSB, other than legacy values (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). In addition to periodicity, RAN1 agreed that the following parameters can be indicated, and multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell:

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

- SSB periodicity

Per RAN1 agreements, multiple candidate values can be configured by RRC for these parameters of the on-demand SSB, and the applicable value can be indicated by MAC CE.

Observation 1:	RAN1 agreed to indicate, via a configuration an index, the following OD-SSB parameters: SSB positions within an on-demand SSB burst, the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated, and SSB periodicity.

These parameters can be abstracted by a configuration index, which is then indicated in the MAC CE to indicate the applicable value. Assuming an octet-aligned design, this provides the ability to indicate up to 2^8 (256) combinations of such parameters. 1 octet is therefore sufficient to indicate which is applied for the OD-SSB.

Proposal 1:	1 octet is used to indicate in the OD-SSB MAC CE which configuration index associated with the activated OD-SSB.

From a MAC CE design perspective, R2 agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Therefore, a bit map can be used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) octet is added to indicate the applicable configuration index. The OD-SSB MAC CE therefore has a variable size. A sample MAC CE is illustrated in Figure 1 for such assumption, at least for the case of up to 7 SCells.

Figure 1: illustration of a variable-size OD-SSB MAC CE short format

Provided that the MAC CE does not provide (de)-activation of SCells, it is beneficial to keep the MAC CE simple and fixed in size.

Similar to the SCell Activation MAC CE, there can be two formats: one short variable size format, which indicates up to 7 SCells. Another long variable size format, which indicates up to 31 SCells. The short variable size format is illustrated in Figure 1, while the long variable size format is illustrated in Figure 2, where 3 bits are used to indicate the OD-SSB periodicity.

Figure 2: illustration of a variable-size OD-SSB MAC CE long format

Proposal 2:	The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional (optional) octet is added to indicate the applicable OD-SSB configuration index.

A text proposal to TS 38.321 is added for a possible MAC CE description. It is proposed to adopt this TP as a starting point.

Proposal 3:	Adopt the text proposal to TS 38.321 in Annex A for the OD-SSB MAC CE.

## Implicit deactivation of OD-SSB  transmission

RAN1 agreed to implicitly deactivate an OD-SSB transmission after the transmission of N occasions, where N can be indicated in the MAC CE. The modelling of such should be similar to SCell deactivation timer, which restarts after the reception of the SCell Activation/Deactivation MAC CE:

## 5.9 Activation/Deactivation of SCells If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers. The configured SCell(s) is activated and deactivated by: - receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10; - receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55; - configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry; […] The MAC entity shall for each configured SCell: 1> if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell: […] 2> start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; […] 1> else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or 1> if the sCellDeactivationTimer associated with the activated SCell expires; or 1> if the SCG associated with the activated SCell is deactivated: 2> deactivate the SCell according to the timing defined in TS 38.213 [6]; 2> stop the sCellDeactivationTimer associated with the SCell; 2> stop the bwp-InactivityTimer associated with the SCell; 2> deactivate any active BWP associated with the SCell; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> flush all HARQ buffers associated with the SCell; 2> cancel, if any, triggered consistent LBT failure for the SCell.

Observation 2:	SCell deactivation timer restarts after the reception of the SCell Activation/Deactivation MAC CE.

Proposal 4:	For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the SCell Activation/Deactivation MAC CE:

## measurements on OD-SSBs

In RAN2#129bis, the following working assumption is made: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell. This understanding allows the UE to uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements, including when they’re on different frequencies.

Proposal 5:	Confirm working assumption: when A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Observation 1:	RAN1 agreed to indicate, via a configuration an index, the following OD-SSB parameters: SSB positions within an on-demand SSB burst, the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated, and SSB periodicity.

Proposal 1:	1 octet is used to indicate in the OD-SSB MAC CE which configuration index associated with the activated OD-SSB.

Proposal 2:	The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional (optional) octet is added to indicate the applicable OD-SSB configuration index.

Proposal 3:	Adopt the text proposal to TS 38.321 in Annex A for the OD-SSB MAC CE.

Observation 2:	SCell deactivation timer restarts after the reception of the SCell Activation/Deactivation MAC CE.

Proposal 4:	For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the SCell Activation/Deactivation MAC CE:

Proposal 5:	Confirm working assumption: when A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

# Appendix A: TP for introduction of OD-SSB MAC CE

# 6	Protocol Data Units, formats and parameters

### 6.1.3	MAC Control Elements (CEs)

#### 6.1.3.x	On-demand SSB Activation/Deactivation MAC CE

The On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2. It has a variable size, and includes an Activation/Deactivation bitmap and, in ascending order based on the ServCellIndex, an On-Demand SSB configuration index field for SCells activated in the bitmap. A single octet bitmap is used for Activation/Deactivation of the On-Demand SSB transmission and indicating the presence of an On-demand SSB configuration index per Serving Cell, containing seven C-fields and one R-field. The On-Demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet is defined as follows (Figure 6.1.3.x-1).

The On-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2. It has a variable size, and includes an Activation/Deactivation bitmap and, in ascending order based on the ServCellIndex, an On-Demand SSB configuration index field for SCells activated in the bitmap. A four-octet bitmap is used for Activation/Deactivation of the On-Demand SSB transmission and indicating the presence of an On-demand SSB configuration index per Serving Cell, containing 31 C-fields and one R-field. The On-Demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets is defined as follows (Figure 6.1.3.x-2).

-	Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the on-demand SSB activation/deactivation status for the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that On-Demand SSB transmission on the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that On-Demand SSB transmission on the SCell with SCellIndex i shall be deactivated;

-	ConfigIndexi: The on-demand SSB configuration index to apply for SCellIndex i as. This field is present for SCell with SCellIndex I if Ci is 1, and not present if Ci is 0.

-	R: Reserved bit, set to 0.

Figure 6.1.3.x-1: On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet

Figure 6.1.3.x-2: On-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets

Editor’s note: the size of ConfigIndexi is FFS