3GPP TSG-RAN WG2 Meeting #130	R2-2504308

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:	8.7.4.1

Source: 	Huawei, HiSilicon

Title: 	Remaining open issues of LCP enhancements

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we discuss one of the remaining issues related to LCP enhancement raised in the open issue list below:

- [MAC-02] Impact of congestion (i.e. PSI-based SDU discard) on priority adjustment

# 2.	Discussion

## 2.1	MAC Open Issue 02

In the MAC running CR of post RAN2#129 meeting [1], there is issue about additional priority-based LCH enhancement, as described in the below Editor’s Note

Editor’s Note: The above conditions on priority adjustment may need further clarifications, due to a corner case which has not been discussed yet. In this case, the data with the smallest remaining discard timer is queued behind non-delay-critical data and the UL grant is not large enough to include it in the transmission. It is FFS whether this logical channel should apply its additionalPriority or its Priority in this case.

Our understanding is that the above Editor’s Note is now listed as Open Issue 02 in the MAC running CR after RAN2#129bis meeting [2]:

- [MAC-02] Impact of congestion (i.e. PSI-based SDU discard) on priority adjustment

Our understanding is that additional priority should be applied when the smallest remaining time of the LCH is less than the priorityAdjustmentThreshold for the LCH, regardless of whether there are non-delay critical data in front of the delay critical data. Note that we have also agreed the following during RAN2#126:

LCP prioritization within a logical channel will not be considered in RAN2 discussions

Observation 1:	RAN2 agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions”.

We further analyse the occurrence of the case. In the case where DSR is triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH < remainingTimeThreshold of the LCG), the DSR will include both the delay critical data and the non-delay critical data ahead in the buffer status calculation to the NW and the NW can schedule enough UL grant based on the reported R19 DSR MAC CE, so it is corner case that the UL grant is not enough to carry all the data, e.g., LCH-adjusted data and the other data ahead of LCH-adjusted data. It only occurs when the DSR is not triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH > remainingTimeThreshold of the LCG), in this case the UL grant may not be large enough. However, we do not see any issue here, as the UE will trigger the DSR to request for more UL grant later, if the previous grant is not large enough.

Observation 2:	The case occurs only when DSR is not triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH > remainingTimeThreshold of the LCG) and the UL grant is not enough to cover both the delay-critical data and the non-delay critical data ahead. The delay critical data (if not completely transmitted in the previous UL grant) will trigger the DSR to request for more UL grant later.

Therefore, there is no need to consider the case where the LCH priority-adjusted data with the smallest remaining time of discardTimer is queued behind other data (e.g., non-delay-critical data) and the UL grant is not large enough to include the LCH priority-adjusted data for transmission.

Proposal 1:	(MAC-02) Additional priority should be applied on the LCH if the smallest remaining time is less than the priorityAdjustmentThreshold for the LCH as agreed. This also includes the case where the LCH priority-adjusted data with the smallest remaining time of discardTimer is queued behind other data (e.g., non-delay-critical data) and the UL grant is not large enough to include the LCH priority-adjusted data for transmission, no enhancement for this case is needed.

# 3. Conclusion

Proposal 1:	(MAC-02) Additional priority should be applied on the LCH if the smallest remaining time is less than the priorityAdjustmentThreshold for the LCH as agreed. This also includes the case where the LCH priority-adjusted data with the smallest remaining time of discardTimer is queued behind other data (e.g., non-delay-critical data) and the UL grant is not large enough to include the LCH priority-adjusted data for transmission, no enhancement for this case is needed.

# 4. References

[1]. [POST129][509][XR] MAC running CR (Qualcomm) https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23129%5D/%5BPOST129%5D%5B509%5D%5BXR%5D%20MAC%20running%20CR%20(Qualcomm)

[2]. List of open issues in MAC_summary_r2, Qualcomm Incorporated, St Julian, Malta, 19-23 May 2025 https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23129bis%5D/%5BPOST129bis%5D%5B502%5D%5BXR%5D%20MAC%20running%20CR%20and%20open%20issues%20(Qualcomm)