3GPP RAN WG2 Meeting #130	R2-2504309

Malta, Malta, May 19th – 23rd 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	Remaining open issues: LCM for UE-sided model for BM use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining open issues related to the LCM procedure has been provided as part of a post meeting TS 38.331 running CR review [2]. This contribution discusses these remaining issues, specifically those related to UE sided model for BM use case.

# Remaining Open Issues

## Classified as “to be treated in RAN2#130”

The following issues have been captured by the RRC CR Rapporteur and classified as “suggested to be treated in RAN2#130”. Companies are invited to refer to [2] for a more comprehensive description of each open issue.

[RRC-1] Cause of inapplicability

In RAN2#129bis it was agreed that together with inapplicability reporting, UE further indicates a simple cause value of inapplicability. Which cause values are supported and how to define this in the specification is currently FFS, and is captured as an editor’s note within the TS 38.331 running CR.

The inapplicability cause value should help the network determine whether to maintain or release the configuration based on the likelihood a configuration will become applicable in the future. For example, if a model is not available at the UE (e.g., not downloaded, trained, supported etc.) this could indicate that the configuration may not become applicable for a while and UE should release it. Alternatively, a configuration inapplicable due to UE/NW side conditions may be more likely to change applicablity (e.g., UE changes speed) and the UE should maintain the configuration. Therefore, simple cause values like “model not available” and “UE/NW side conditions not suitable” should be supported.

Proposal 1:	[RRC-1] Support at least the following inapplicability cause values: 1) Model not available; 2) UE-side conditions not suitable; and 3) NW-side conditions not suitable.

If the cause for inapplicability does not follow one of the above cases, the UE may not include a cause value, or a fourth option “Other” may additionally be supported.

[RRC-2]: Content of otherConfig for enabling applicability reports in UAI

From the baseline LCM procedure agreed in RAN2#127, a configuration in OtherConfig enables reporting a change in applicability via UAI. However, the content of this configuration is currently FFS in the running CR, with the main consideration being whether additional configurations are needed.

Based on subsequent RAN2 agreements, UE behaviour for applicability reporting can be summarized as follows:

- Upon receiving a full inference configuration, UE explicitly reports the (in)applicability of all provided configurations via an initial applicability report in RRCReconfigurationComplete.

- UE can report a change of applicability (applicability to inapplicable and vice-versa) via UAI if configured. UE explicitly reports the configuration that change applicability and includes a simple cause value of inapplicability. There is no additional prohibit timer to restrict this subsequent reporting.

Apart from the configuration to enable UAI reporting, UE behaviour is clear and no other configurations are currently needed for applicability reporting. However, a possible issue is how this flag is interpreted by Option A (full inference configuration) and Option B (set of inference related parameters). For example, does this flag only apply to Option A? Are separate flags necessary? etc.

Assuming that Option B follows the same principle as Option A and applicability report can be included in both RRCReconfigurationComplete and UAI (further discussed in open issue [RRC-7]), it seems that a single flag can indicate the support for subsequent reporting via UE for both Option A and B.This can be confirmed once the signaling design for Option B is finalized. The content of ApplicabilityReportConfig therefore includes only a simple flag indicating whether subsequent applicability reporting via UAI is enabled or disabled.

Proposal 2:	[RRC-2] ApplicabilityReportConfig includes a flag indicating whether applicability reporting via UAI is enabled or disabled. RAN2 assumes a single flag applies for both Option A and B.

[RRC-3] UE data collection request

Details of the UE request for UE-side data collection require further clarification, including how a UE refers to a preferred data collection configuration among a list of candidate configurations. As the number of possible configurations within this list may vary depending on NW implementation/configuration, a bitmap of a fixed size is not suitable. Instead, each configuration within the candidate list may have an associated index/ID value which can be referred to in the preference indication. The details of this index/ID (e.g., whether an existing ID can be reused, whether a new ID is needed etc.) can be FFS.

Proposal 3:	[RRC-3] UE indicates preferred data collection configuration via an index/ID associated with a configuration in the candidate list.

Based on current RAN2 agreements, network can control data collection via the following mechanisms:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

From the above, the only agreed configuration needed at the UE is to enable/disable data collection request, which can be accomplished via a simple flag. It is noted that the network may always disable this preference indication by simple providing only one (or none) candidate configurations to the UE, however this can be done without the need for explicit configuration.

Proposal 4:	[RRC-3] dataCollectionPreferenceConfig includes at least a flag indicating whether UE data collection request is enabled or disabled.

[RRC-4] Activation of a periodic CSI report configuration upon change from inapplicable to applicable

Previous agreements support autonomous activation of a periodic CSI report configuration upon reporting applicability in Step 4. However, if the configuration is not applicable, the UE maintains the configuration until it becomes applicable or is released by the network. [2] Notes that for an inapplicable periodic CSI report configuration it is unclear whether/how it can be activated upon a change of applicability (i.e., from inapplicable to applicable) since there is no legacy procedure to activate such a configuration other than the initial sending of the RRC configuration.

Introducing a separate (de)activation mechanism for periodic CSI reporting requires additional specification impact to acheive essentially the same functionality as semi-persistent CSI reporting (which could always just be configured in the first place). Furthermore, the reason for (in)applicability reporting (and including a cause value for inapplicability) is to support the network decision to (de)configure an inference configuration. After receiving the initial applicability report, the network can simply de-configure any inapplicable periodic CSI report configuration. Therefore, as proposed by the Rapporteur, this issue can be left to NW implementation (i.e., no specification impact).

Proposal 5:	[RRC-4] NW implementation can deconfigure a periodic CSI report configuration after UE initially reports it as inapplicable (i.e., no specification impact).

[RRC-5] Reporting behaviour of inapplicable periodic beam prediction configuration

In a related issue to RRC-4, [2] notes that if periodic beam prediction configuration becomes inapplicable, it is unclear what/if the UE continues reporting. Options listed include ceasing reporting, or send the input (measurements) to the NW once it becomes inapplicable, or continue sending (inaccurate) predictions.

In legacy, UE configured with periodic reporting is expected to report results until deconfigured. If the UE is configured to enable reporting a change in applicability via UAI, UE can indicate if/when the periodic configuration becomes inapplicable and the network can subsequently deconfigure it (e.g., as per Proposal 5). If subsequent reporting via UAI is not configured, there could be other ways to determine that periodic beam predictions are erroneous like performance monitoring. Furthermore, if the network notices that applicability is in general unstable (e.g., due to a narrowly trained model, fluctuating side conditions etc.) it can always use a semi-persistent configuration which can be more easily adjusted.

Either way, the network should be able to determine a configuration is inapplicable and and perform corrective action relatively quickly. Therefore, the UE should function according to legacy and continue to report the periodic predictions for an inapplicable configuration (even if temporarily invalid) until the network takes corrective action.

Proposal 6:	[RRC-5] UE continues to report periodic beam predictions until the associated periodic CSI report configuration is deconfigured by the network.

[RRC-6] Handling of inference, applicability reporting and UE data collection preference configurations when UE goes to RRC_IDLE/INACTIVE state and during RRCReestablishment

Previous agreements dictate that a UE maintains full inference configuration(s) until explicitly released by the network, regardless of whether it is applicable or inapplicable. It is currently open how the UE handles such configurations upon release to RRC_IDLE/INACTIVE.

The intention is that an inapplicable configuration may again become applicable, and additional signaling overhead can be avoided if the configuration is maintained and stored by the UE instead of releasing and having to resend. However, there is no guarantee that the NW side conditions will be consistent across cells, and if the UE reselects to a different cell the set of inference configurations may no longer be applicable (the probability of this happening obviously increases the longer the UE remains in IDLE/INACTIVE and reselects to more and more cells).

Once the UE re-enters RRC CONNECTED, maintaining configurations will also increase the size of the initial applicability report, and will require additional (re)configuration by the network to adjust the configurations accordingly. Otherwise, the UE could continue to accumulate more and more configurations (e.g., with the hope that one of them will again be useful) which can complicate UE implementation.

A simple solution would therefore be that UE releases full inference configuration (i.e., Option A) upon release to RRC IDLE/INACTIVE. This agreement can also be extended to include inference related parameters (i.e., Option B).

Proposal 7:	[RRC-6] UE releases inference-related configuration(s) (full inference configuration(s) and set(s) of inference related parameters) upon release to RRC IDLE/INACTIVE.

[RRC-7]: Applicability reporting for option B in RRCReconfigurationComplete

It is currently FFS whether the applicability report for option B (sets of inference related parameters) can be included in RRCReconfigurationComplete (or if it can only be included in UAI).

In RAN2#127bis, the following was agreed regarding the signalling for the applicability report:

UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled.

Although the above agreement initially targeted Option A, unless RAN1 conclusions require a signalling design which makes the above overly complicated, there seems no reason why the same principle cannot also apply to Option B. Therefore, RAN2 can assume Option B applicability report can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A) until the final signalling design is confirmed (e.g., based on RAN1 conclusions).

Proposal 8:	[RRC-7] RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

[RRC-8] Coexistence between option A and option B

As RAN2 agreed both Option A and B are to be specified, consideration may be needed for how the 2 options co-exist. An example provided in [2] is whether or not both Option A and Option B can be configured or not, which can affect signaling design of ApplicabilityReportList.

Option A and B were both agreed because they offer different benefits. For example, Option A allows fast activation of inference configuration but at the risk of providing a full configuration for an inapplicable configuration. Option B allows the UE to first evaluate the applicability before receiving the full configuration, saving on signalling overhead. Supporting both options simultanously can enable a tradeoff between fast activation and signalling overhead reduction, which can be balanced based on NW implementation. RAN2 can therefore assume that both Option A and B can be simultaneously configured, however as with other agreements related to Option B this can be confirmed based on RAN1 agreements/final signaling design.

Proposal 9:	[RRC-8] RAN2 assumes Option A and Option B can be simultaneously configured. This can be revisited based on RAN1 conclusions/final signaling design.

# Conclusion

In this contribution the following proposals are made regarding remaining open issues for LCM for beam management use case:

Proposal 1:	[RRC-1] Support at least the following inapplicability cause values: 1) Model not available; 2) UE-side conditions not suitable; and 3) NW-side conditions not suitable.

Proposal 2:	[RRC-2] ApplicabilityReportConfig includes a flag indicating whether applicability reporting via UAI is enabled or disabled. RAN2 assumes a single flag applies for both Option A and B.

Proposal 3:	[RRC-3] UE indicates preferred data collection configuration via an index/ID associated with a configuration in the candidate list.

Proposal 4:	[RRC-3] dataCollectionPreferenceConfig includes at least a flag indicating whether UE data collection request is enabled or disabled.

Proposal 5:	[RRC-4] NW implementation can deconfigure a periodic CSI report configuration after UE initially reports it as inapplicable (i.e., no specification impact).

Proposal 6:	[RRC-5] UE continues to report periodic beam predictions until the associated periodic CSI report configuration is deconfigured by the network.

Proposal 7:	[RRC-6] UE releases inference-related configuration(s) (full inference configuration(s) and set(s) of inference related parameters) upon release to RRC IDLE/INACTIVE.

Proposal 8:	[RRC-7] RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

Proposal 9:	[RRC-8] RAN2 assumes Option A and Option B can be simultaneously configured. This can be revisited based on RAN1 conclusions/final signaling design.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

Report from [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)