3GPP RAN WG2 Meeting #130	R2-2504310

St Julian’s, Malta, May 19th – 23rd 2025

Agenda Item:	8.1.2.3

Source:	InterDigital

Title:	Remaining open issues: LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining open issues related to the LCM procedure has been provided as part of a post meeting TS 37.355 running CR review [2]. This contribution discusses these remaining issues, specifically those related to the Positioning use case. In addition, this contribution discusses specification impacts of agreements related to ground truth delivery from the LMF made in RAN1.

# Remaining open issues

## [LPP-16]: Applicability reporting and performance monitoring for Case 1

In RAN1, details related to performance monitoring are being discussed. During performance monitoring, the UE checks whether the AIML model(s) at the UE can be used for AIML based positioning.

In RAN1#119, the following agreement was made:

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

In RAN1#120b, the following agreement was made related to the FFS regarding the content of monitoring outcome:

Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

UE reports a change of applicable functionality to the LMF via an unsolicited LPP provide capabilities message. Unlike UAI (used in beam management) which has built in control mechanisms to restrict excessive reporting, there are no LMF controls on UE transmission of an unsolicited LPP provide capabilities message. In RAN2#129b, it was agreed not to introduce LMF control over unsolicited LPP capability reporting.

As the agreement in RAN1#120b indicates, the UE can report, as the outcome of performance monitoring, an indication that the target UE cannot perform the Case 1 positioning method, i.e., the UE declares non-applicability as the result of performance monitoring. Thus the following proposal is made.

Proposal 1:	[LPP-16] The UE can indicate that the target UE cannot perform the Case 1 positioning method in an unsolicited capability report

Another topic RAN2 should discuss is condition(s) that performance monitoring can be triggered. For example, the UE can determine to initiate performance monitoring on its own. Alternatively, the UE can be configured with conditions (e.g., periodic performance monitoring, change in the serving cell) to initiate performance monitoring. Specification of triggers for performance monitoring will ensure consistent performance of AIML model(s) at the UE

Proposal 2:	[LPP-16] Specify triggers to initiate performance monitoring. FFS details.

## [LPP-17]: Delivery of ground truth from the LMF to the UE

In RAN1#120, the following agreement was made.

Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

In the above agreement, it requires the ground truth (UE location information) can be delivered to the UE from the LMF. RAN2 should clarify whether there is a need for a change in LPP to reflect the RAN1 agreement as the TS37.355 does not have a procedure to allow the LMF to transfer location information to the UE.

For the LMF to generate the ground truth for the UE, the LMF requires measurements reported to the LMF from the UE and/or gNB. In TS 23.273, delivery of location information from the AMF to the UE is specified under MO-LR (Mobile Originated Location Request). As a reference, Figure 6.2-1 form TS 23.273 is reproduced below.

Figure 1 , Figure 6.2-1 form TS 23.273, “5GC-MO-LR Procedure”

As explained in Section 6.2 in TS 23.273, the UE can send an MO-LR request message which can indicate requirements for the location information desired by the UE. Such procedure can be used to obtain the ground truth at a desired accuracy. In addition, this procedure allows the network to determine appropriate positioning method(s) to generate the ground truth for the UE, ensuring quality for the ground truth. Therefore, the following proposal is made

Proposal 3:	[LPP-17] From the RAN2 perspective, for Case 1, the MO-LR procedure specified in TS 23.273 can be used to collect the ground truth from the LMF by the UE

## [LPP-15]: Applicability of Positioning Integrity to AI/ML positioning

For this open issue, we need to wait for RAN1 to decide whether TRP locations can be signalled explicitly. It should be noted that integrity information includes integrity information related to TRP location, nr-IntegrityParametersTRP-LocationInfo. If RAN1 decides not to include TRP locations in the assistance data, integrity information related to TRP locations are not useful. On the other hand, if TRP locations are included, corresponding integrity information can be included. Therefore, we propose to wait for conclusion of RAN1 discussion for [LPP-15].

Proposal 4:	[LPP-15] Wait for RAN1 discussion to determine whether integrity information should be included or not as part of integrity information contain uncertainty related to TRP locations.

## [LPP-6]: Applicability of NR-PRU-DL-Info

It has been discussed in the email discussion “[POST129bis][015][AI PHY] 37.355 Running CR” whether the content of NR-PRU-DL-Info should be updated for Case 1 AIML positioning. The open issue is whether AoD measurements should be transferred to the UE or not as the current NR-PRU-DL-Info contains nr-PRU-DL-AoD-MeasInfo-r18.  It should be noted that NR-PRU-DL-Info is provided to the UE which is configured with the UE-based DL-TDOA. Therefore, the purpose of AoD measurements in NR-PRU-DL-Info is for calibration purpose only and being configured with a DL-AoD positioning method is not a prerequisite to receive AoD measurements via NR-PRU-DL-Info.

Observation:	Being configured with a DL-AoD positioning method is not a prerequisite to receive AoD measurements via NR-PRU-DL-Info

It has been raised during the discussion that RAN1 has not agreed to include assistance data related to the AoD positioning method in the assistance data for the Case 1 positioning method. According to the above observation, AoD related measurements are provided regardless of the AoD related configuration for the UE. Thus, the following proposal is made;

Proposal 5:	[LPP-6] The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed

# Conclusion

In this contribution the following observations and proposals are made regarding open issues for LCM for Positioning use case:

Observation:	Being configured with a DL-AoD positioning method is not a prerequisite to receive AoD measurements via NR-PRU-DL-Info

Proposal 1:	[LPP-16] The UE can indicate that the target UE cannot perform the Case 1 positioning method in an unsolicited capability report

Proposal 2:	[LPP-16] Specify triggers to initiate performance monitoring. FFS details.

Proposal 3:	[LPP-17] From the RAN2 perspective, for Case 1, the MO-LR procedure specified in TS 23.273 can be used to collect the ground truth from the LMF by the UE

Proposal 4:	[LPP-15] Wait for RAN1 discussion to determine whether integrity information should be included or not as part of integrity information contain uncertainty related to TRP locations.

Proposal 5:	[LPP-6] The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

[POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)