3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504319

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.9.4

Source:	Qualcomm Incorporated

Title:	Discussion on PWS in NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

Following agreements were made in RAN2#129 meeting.

Agreements:

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

- Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

- Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

- We don’t introduce a new SIB for carrying geographical information for emergency areas

Following agreement was made in RAN2#129bis meeting.

Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

In this document, we provide details on remaining aspects of PWS in NB-IoT.

- Discussion

Currently NB-IoT UEs do not support “acceptable cell”, “Camped on Any cell state” and “emergency call” functionalities. NB-IoT UEs cannot also read broadcast notification message in RRC_CONNECTED. When PWS is supported in a cell (i.e., in NB-IoT RAT) as new feature, there will be mix of cells supporting and not supporting the PWS feature.

## 4.4 NB-IoT functionality in Idle Mode This specification is applicable to NB-IoT, except for the following functionality which is not applicable to NB-IoT: - Acceptable cell - Accessibility measurements - Access Control based on ACDC categories - Camped on Any cell state - CSG, including support for manual CSG selection and CSG or Hybrid cell related functionality in PLMN selection, or HNB name (SIB9), Cell selection and Cell reselection. - Emergency call - E-UTRAN Inter-frequency Redistribution procedure - Inter-RAT Cell Selection and Reselection including measurements in other RATs - Logged measurements - Mobility History Information - Mobility states of a UE - Priority based reselection - Public warning system including CMAS, ETWS, PWS.

In NB-IoT, cell reselection is based on ranking regardless of frequency priorities. If a UE is PWS capable and camps in a cell that does not support PWS while there are neighbor cells supporting PWS, then UE implementing such important feature will not be able to take benefit and is going to miss the warning message. Therefore, we think UE needs to be aware of the PWS capability of the cell to prioritize cell search and PLMN search. In addition, while searching for the suitable cell, the NB-IoT UE may be able to monitor the paging for emergency message in an acceptable cell. To avoid reading SIB1 and speed up the search, such indication can be added in MIB as there are enough spare bits available.

Introduce a new indication in MIB using a spare bit that a NB-IoT cell supports PWS feature.

While searching for the suitable cell, NB-IoT UEs may camp on an acceptable cell that supports the PWS feature.

In TS 36.306, currently it is specified in section 6.2 that PWS reception is optional for UE without UE capability signaling. Similarly, the support of geofencing is also optional without UE capability signaling.

However, the support of PWS will be a new feature for NB-IoT and there will be mix of UEs supporting and not supporting the feature. From RAN1 and RAN plenary discussion, it is already now clear that the NB-IoT UE in RRC_CONNECTED will not support PWS notification. Therefore, it is agreed that a new UE capability signaling is introduced so that network can release PWS capable UEs to RRC_IDLE when PWS message is broadcast.

RAN2 has already agreed to introduce geofencing for ETWS and CMAS messages. We think the support of reception of PWS message does not necessarily mean UE can support the geofencing. Therefore, support of geofencing for PWS should still be optional feature without capability signaling.

Support of geofencing for PWS is optional feature without capability signaling.

Skipping SIB1 acquisition

When a UE receives paging notification for PWS message, it would need to acquire SIB1 to know the scheduling information of SIB10 or SIB11 or SIB12. The SystemInformationBlockType1-NB (SIB1-NB) uses a fixed schedule with a periodicity of 2560 ms. Due to coverage enhancement operation, the SIB1 acquisition itself can add delay to acquire the SIB10 or SIB11 or SIB12. An optimization that can be considered is that the scheduling information of SIB10, SIB11 and SIB12 are pre-configured in SIB1, however, they are not transmitted until indicated as valid/active or usable. The scheduling information is stored along with the valid SIB1.

When there is warning message, then only the network sends PWS notification in DCI/Paging message which will then activate/validate the stored scheduling information of SIB10 or SIB11 or SIB12.

Consider pre-configuring the scheduling information of SIB10/11/12 in SIB1 and validating it by the PWS notification.

PWS assembly

When cell switch happens during IDLE mode, the UE needs to discard the old segments of PWS and acquire the PWS from new cell. The reason is the PWS size can be different for different cell. Depending on resource availability and segmentation policy of different cells or eNBs, the PWS segment size can be different and cannot be assembled across different cell. We think this cannot be achieved without close coordination between eNBs on the segmentation policy and its awareness at UE. Therefore, the UE should discard old segments upon new cell reselection.

Old PWS segments are discarded upon cell change.

- Conclusion

Following proposals are made:

Proposal 1	Introduce a new indication in MIB using a spare bit that a NB-IoT cell supports PWS feature.

Proposal 2	While searching for the suitable cell, NB-IoT UEs may camp on an acceptable cell that supports the PWS feature.

Proposal 3	Support of geofencing for PWS is optional feature without capability signaling.

Proposal 4	Consider pre-configuring the scheduling information of SIB10/11/12 in SIB1 and validating it by the PWS notification.

Proposal 5	Old PWS segments are discarded upon cell change.