3GPP TSG RAN WG2 #130	R2-2504338

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.9.3

Source:	Nordic Semiconductor ASA

Title:					IoT-NTN uplink capacity enhancement

Document for:		Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#129bis, RAN2 has reached the following agreements on CB-Msg3 EDT.

Agreements:

1.	Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

2.	We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

3.	For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

6.	If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

18.	For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Agreements (part 2):

1.	For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

2.	The Msg4 monitoring window configuration (e.g. length) is CE level specific

3.	RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

4.	RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

5.	RAN2 also intends to support CB-msg3-EDT for MT cases

6.	The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

7.	Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

8.	The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

10.	Backoff information could be included in CB-Msg4.

11.	L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

12.	HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In this contribution, we continue discussion on details of the CB-Msg3 configuration and other remaining issues.

# Configuration for CB-Msg3 transmission

A preamble-less EDT is beneficial for reducing uplink and downlink signaling in NTN. Preamble transmission and RAR reception unnecessarily increases delay and, even more importantly, consumes significant uplink and downlink resources. Work on contention based Msg3 (CB-Msg3) in RAN2 has progressed and it has already been agreed on configuring a pool of PUSCH resources for a plurality of CE levels via SIB signaling. UEs are choosing randomly a resource for their Msg3 transmission using a CE level which is selected based on RSRP measurement. There will be different RSRP threshold values for each specified/configured CE level. In addition, RAN2 has agreed to introduce DSA (Diversity Shotted Aloha) mechanism for CB-Msg3-EDT transmission where more than one replica of the Msg3 packet can be transmitted.

Given the PUSCH resource with a fixed length in time for each CE level, it is up to RAN1 to discuss on details of mapping CB-Msg3 on the given physical resource. For example, whether the number of resource units and TBS size are fixed per CE level or UE can select TBS and the number of resource units from a configured set of values similarly what was done in Rel-15 EDT. Also, RAN1 should discuss aspects related to potential scheduled transmission of CB-Msg3, e.g., whether new DCI format is needed.

Proposal 1: Send LS to RAN1 informing on the potential RAN1 work pertaining to detailed physical resource mapping of PUSCH payload data to the configured PUSCH resource and potential other RAN1 topics.

There should be an upper limit on the total data volume that UE is allowed to transmit via CB-Msg3 procedure. However, the UE should be allowed to transmit the given total data volume irrespective of the specific coverage situation it finds itself in. Specifically, this means that if the UE is not able to transmit its payload in a single shot transmission due to bad coverage situation the UE is allowed to request a scheduled transmission for the rest of the payload bits. This can be implemented e.g. by adding one bit/Boolean to CCCH message of Msg3 or allowing UE to include a BSR MAC-CE in the CCCH message.

Proposal 2: The total data volume per CB-Msg3 session per UE is upper limited.

Proposal 3: UE is allowed to start transmission of its payload using CB-Msg3 if its payload is smaller or equal to the configured maximum TBS, and the UE may request a scheduled transmission for the rest of the payload via CCCH of Msg3, if the whole payload does not fit into a configured CB-Msg3 resource.

At the network side, if base station detects a request by UE to continue the transmission after initial CB-Msg3 transmission, it may move UE to RRC-connected mode by responding the UE with an RRCConnectionSetup(-NB)-message. In this case the Msg4 contains a contention resolution information which indicates to UE that the “initial” transmission via CB-Msg3 was successful and there is no need to transmit it again in the RRC-Connected mode. Otherwise, if the contention resolution is not successful the UE is allowed to stay in RRC-Idle state and are allowed to re-attempt the “initial” transmission.

Proposal 4: Upon the request by UE, the base station may move UE to RRC-connected mode by responding the UE with an RRCConnectionSetup(-NB)-message, if the contention resolution for the UE was successful, and thereafter UE does not need to transmit the “initial” transmission again in RRC-Connected state.

Overall, this helps to improve the network capacity as the preconfigured PUSCH resources do not need to be dimensioned according to worst case coverage levels but, instead, the UEs in worst coverage situation are allowed to segment their payload data and transmit part of it via scheduled transmissions in RRC-Connected state, similarly as is done in Rel-17 SDT in NR.

# Conclusions

In this contribution we discussed issues related to uplink capacity enhancement for NB-IoT-based NTN system, and we have the following proposals:

Proposal 1: Send LS to RAN1 informing on the potential RAN1 work pertaining to detailed physical resource mapping of PUSCH payload data to the configured PUSCH resource and potential other RAN1 topics.

Proposal 2: The total data volume per CB-Msg3 session per UE is upper limited.

Proposal 3: UE is allowed to start transmission of its payload using CB-Msg3 if its payload is smaller or equal to the configured maximum TBS, and the UE may request a scheduled transmission for the rest of the payload via CCCH of Msg3, if the whole payload does not fit into a configured CB-Msg3 resource.

Proposal 4: Upon the request by UE, the base station may move UE to RRC-connected mode by responding the UE with an RRCConnectionSetup(-NB)-message, if the contention resolution for the UE was successful, and thereafter UE does not need to transmit the “initial” transmission again in RRC-Connected state.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)