3GPP TSG-RAN Meeting #130	R2-2504341

# St. Julians, Malta, 19th – 23rd May 2025

Agenda Item:	8.7.3

Source:	Ericsson

Title:	Discussion on UE Assistance Information (UAI)

Document for:	Discussion, Decision

# 1	Introduction

The measurement gap enhancement is a RAN1-led objective with related impacts in RAN2 and RAN4. The goal is to minimize the impact of measurements gaps on XR capacity while having a minimal effect on the mobility performance. Achieving this impact minimization involves removing scheduling restrictions i.e., enabling data transmission/reception during the measurement gaps. With RAN1 at the helm and after many discussions, they concluded on a DCI-based dynamic skipping solution where the network can inform the UE to skip/cancel a particular measurement gap occasions to make way for scheduling. The alternative is a semi-static solution however already ruled out in RAN1/RAN2 but RAN4 would continue to discuss.

In addition to the solution to enable transmission/reception during gaps/restrictions, another aspect discussed in RAN1 was related to the information that the UE can send to the network to assist in skipping/cancellation of the measurement gaps but was unable to reach a consensus on the UAI support. An LS [1] was sent to RAN4 for their evaluation and RAN4 has responded to this LS in [2]. They agreed to introduce a ratio of gap occasions that is recommended for cancellation during a period of 1 second. It should be straightforward to implement this ratio in the UAI. But there are a couple of open issues related to the triggering of this UAI as described below.

Further, another aspect of UAI to assist with skipping/cancellation that we believe is important is the UE mobility information. Using this information, the network can for example simply not configure measurement gaps for stationary UEs. Thus, in this contribution, in addition to the ratio of gap occasions, we also propose the signaling of the UE mobility reusing the framework as defined for (e)Redcap.

# 2	Discussion

## 2.1	Ratio of Gap Occasions

For the ratio of gap occasions, there are currently two open issues as captured by the RRC rapporteur:

- Issue5: FFS when the UE should trigger UAI for assistance information for measurement occasion

- Issue6: FFS what are the configurations for controlling UAI reporting assistance information for measurement occasion ratio

On Issue 6, to prevent that the UE sends the updates to the ratio of gap occasions often, a prohibit timer can be configured. No additional information needs to be captured at this point.

Prohibit timer shall be configured for controlling the UAI assistance information for ratio of gap occasions.

On Issue 5, when the UE should trigger UAI or provide an update on the ratio of gap occasions can be left up to UE implementation. No further work is required to be done in RAN2.

It is up to UE implementation when to trigger or provide an update in the UAI for the ratio of gap occasions.

## 2.2	UE Stationarity

UE assistance information related whether the UE is stationary is potentially useful for the network in deciding the measurement gap configuration/skipping occasions.

- Information on stationarity of the UE could potentially be useful to (or not) configure measurement gaps and indicate skipping occasions.

We could also simply reuse the framework of the relaxed measurement criterion for a stationary (e)RedCap UE. In the current procedure, the network provides an RRM-MeasRelaxationReportingConfig for determination of stationarity and the UE can report its rrm-MeasRelaxationFulfilment status in the UAI on whether the stationary criterion is met. The other advantage of reusing the relaxed measurement criterion for a stationary UE is that there are no corresponding RAN4 requirements.

Consider reusing the RRM measurement relaxation criterion for a stationary (e)RedCap UE as framework for providing the UE stationarity in the UAI.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	Information on stationarity of the UE could potentially be useful to (or not) configure measurement gaps and indicate skipping occasions.

The observations lead to the following proposals:

Proposal 1	Prohibit timer shall be configured for controlling the UAI assistance information for ratio of gap occasions.

Proposal 2	It is up to UE implementation when to trigger or provide an update in the UAI for the ratio of gap occasions.

Proposal 3	Consider reusing the RRM measurement relaxation criterion for a stationary (e)RedCap UE as framework for providing the UE stationarity in the UAI.

# 4 	References

- R1-2405736, LS on UE Assistance Information, 3GPP RAN WG1 #117, Fukuoka, Japan.

- R4-2504972, Reply LS on UE assistance information, RAN WG4#114bis, Wuhan, China.

# 5	Annex

The following are the agreements from RAN2#127bis on measurement gap enhancements:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution.

Further, the following agreements taken in the RAN2#128:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

Furthermore, the agreements from RAN2#129:

Agreements on MG skipping - From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion. - “Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC. - RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1