3GPP TSG-RAN WG2 Meeting #130	 R2-2504362

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.4

Source: 	CATT

Title: 	Remaining issue on support of PWS for NB-IoT NTN UE

Document for: 	Discussion and Decision

Introduction

This contribution further discusses the remaining issue for PWS for NB-IoT NTN UEs, i.e. whether SIB1-NB needs to be first acquired upon PWS indication.

Whether SIB1-NB acquisition is needed

About acquisition on PWS immediately, RAN2 achieved the agreement “We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed” in RAN2 #128. It is FFS whether SIB1-NB acquisition upon PWS indication is needed; the intention of avoiding acquiring the SIB1-NB was to reduce the latency for acquisition of PWS.

In LTE, UE is not required to periodically check schedulingInfoList contained in SIB1, and the eNB may not send paging message to indicate to the UE the update of the schedulingInfoList due to the start or stop of the scheduling for PWS. Hence the UE should first read the SIB1 to acquire the scheduling information for the corresponding PWS message, and then acquire the PWS message accordingly, as follows [1]:

NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled.

If the similar mechanism is reused by the NB-IoT NTN UE, it also requires the UE to acquire the SIB1-NB to get the scheduling information for the corresponding PWS message upon reception of the indication of PWS.

However, the periodicity of SIB1-NB (2560 ms) is much longer than that of SIB1 in LTE (80 ms); acquiring SIB1-NB will thus introduce more latency which may increase the possibility that the requirement for PWS acquisition is not met. If this were regarded as an issue to be solved, RAN2 could consider a solution to avoid UE having to acquire SIB1-NB right before acquisition of PWS message. Specifically, the NW which supports PWS can reserve the position of SI window for PWS in the schedulingInfoList, and the NW really broadcasts the corresponding PWS in the reserved position only when the NW starts sending the indication of PWS. Otherwise, the resources located in the SI window can be scheduled for other data transmission.

Proposal 1: RAN2 down-selects from the following way forwards regarding whether SIB1(-NB) needs to be first acquired upon PWS indication:

- WF 1: Reuse the same mechanism as in LTE, i.e. the PWS indication triggers the UE to re-acquire schedulingInfoList contained in SIB1-NB to get scheduling info for corresponding PWS message.

- WF 2: The schedulingInfoList contained in SIB1-NB includes the scheduling information for PWS message even when eNB does not broadcast PWS message, and the PWS indication triggers UE to acquire the corresponding PWS message immediately based on the stored schedulingInfoList.

Conclusion

In this contribution, the remaining issue on support of PWS for NB-IoT UE is discussed with the following proposal:

Proposal 1: RAN2 down-selects from the following way forwards regarding whether SIB1(-NB) needs to be first acquired upon PWS indication:

- WF 1: Reuse the same mechanism as in LTE, i.e. the PWS indication triggers the UE to re-acquire schedulingInfoList contained in SIB1-NB to get scheduling info for corresponding PWS message.

- WF 2: The schedulingInfoList contained in SIB1-NB includes the scheduling information for PWS message even when eNB does not broadcast PWS message, and the PWS indication triggers UE to acquire the corresponding PWS message immediately based on the stored schedulingInfoList.

Reference

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"