3GPP TSG-RAN2 Meeting #130			R2- 2504372

Location, Country, Date

Agenda item:	8.2.3

Title:	Further consideration on A-IoT random access

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting [1], RAN2 discusses some issues related to Ambient IoT random access and formulates agreements as follows.

Agreements on msg1 - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. Agreements on new R2D message - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. Agreements on msg2 - A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. - Same Msg2 format is used for initial transmission and retransmission of Msg2. Agreements on NACK - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). - For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging. - For CFRA, NACK feedback and re-access is not supported. FFS how to achieve - FFS on end of procedure

This contribution discusses some further details on A-IoT random access.

Discussion

Regarding the RA framework for Ambient IoT, it becomes apparent that strict byte-alignment of the R2D message is neither observed in practice nor necessary for efficient transmission. In scenarios where high device density is expected, the CBRA R2D trigger message and A-IoT Msg2 may be sent repeatedly to a large population of devices.

For R2D messages that will be massively sent, imposing a rigid byte-alignment requirement would therefore introduce singling overhead for sending at reader side and energy overhead for receiving at device side. It might also enlarge the time for all device being identified or successfully accessed. From this perspective, at least R2D messages that are frequently sent are not mandatory to be byte-aligned.

Observation 1: It’s beneficial to keep R2D messages which are supposed to be sent massively, e.g., CBRA R2D trigger message and A-IoT Msg2 short and compact to reduce the signaling and energy overhead and accelerate random access procedure.

Proposal 1: Byte alignment is not mandatory at least for R2D message that will be frequently transmitted, e.g., R2D trigger message and A-IoT Msg2.

The other common issue is the D2R grant indicated in A-IoT MAC PDU, which mainly depends on RAN1. However, RAN2 can make some assumptions for the ease of discussion and endorsement. At a glance, the A-IoT paging message can serve to preconfigure a set of D2R resources for A-IoT Msg1 to all selected devices in advance and the R2D trigger massage can be utilized to activate the preconfigured A-IoT Msg1 resource, which is very similar to the Type 1 or 2 Configured Grant mechanism but it’s a one-off resource allocation.

How the D2R resource for A-IoT Msg3 is indicated, i.e., whether through an explicit grant in A-IoT Msg2 or by reusing the A-IoT paging message as A-IoT Msg1, is deferred to RAN1 specifications and whether the length of device ID is constant. Moreover, the D2R grant for the first D2R message in CFRA can likewise reuse the CBRA Msg1 mechanism (indicated in A-IoT paging message), reinforcing a unified approach.

Proposal 2: RAN2 agrees the following WA on CBRA D2R grant, which can be revisited if RAN1 does otherwise:

- D2R resource indication for A-IoT Msg1 is preconfigured in A-IoT paging message and activated by R2D trigger message.

- D2R resource indication for A-IoT Msg3 is indicated in A-IoT Msg2 by default.

  - Details on CFRA

    - CFRA for multiple devices

In certain service scenarios, for example, to confirm whether a group of known devices is still within the coverage of the reader or to count the number of devices with known IDs, the ability to trigger CFRA for multiple devices not merely convenient but necessary. When CFRA is extended to address multiple devices, the device ID related information and resource allocation indication for each selected device should be carried in the paging message, which can result in a substantially longer R2D transmission.

However, with regarding to the RAN1 agreement that device may not monitoring PRDCH after a time threshold, this enlargement forces each device to remain in monitoring mode for an extended period, thereby increasing energy consumption. Also, a very long A-IoT paging message is not preferred as it has higher change to fail in transmission, considering the length of multiple device IDs.

Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device.

Considering the upcoming Rel-19 endorsement, it is suggested to postpone this feature to the next release, allowing RAN2 to prioritize the work on fundamental yet critical design for procedure and functionality.

Proposal 3: CFRA for multiple devices is not supported in Rel-19.

    - CFRA trigger message

In CBRA, a separate trigger message helps to leave enough time for devices to randomly select an access occasion and generate the random ID. In CFRA, however, D2R resources are pre‐allocated, random ID is not necessary, and no randomization is required. Since the paging message inherently selects the intended device and can indicate the time‐domain starting point of the D2R resource, introducing an additional CFRA trigger message provides no new feature or any benefits for resource indication.

Proposal 4: As CFRA does not need randomization, i.e., device to random select resource and generate random ID, the A-IoT paging message can be reused as the CFRA trigger message.

A further motivation occasionally cited for a separate CFRA trigger is to deliver higher energy to awaken low‐energy devices. For now, the reader has no mechanism to ascertain an individual device’s energy state without device signalling, especially without any D2R message. Consequently, a separate CFRA trigger message can hardly help.

    - Failure detection and handling in CFRA

When the CFRA paging message from the reader fails, due to, e.g., the device being momentarily unavailable or failing to detect trigger message or send the response within a defined time window, the reader must ensure that the failed device can be detected and triggered for a new round for CFRA.

Considering other failures in CFRA, for example, D2R message failure, which can be detected by reader via CRC or energy detection, the reader can also reuse the A-IoT paging message to resend the command.

Observation 2: In CFRA, reader can use a D2R time threshold, e.g., TD2R_max, CRC and energy detection to detect any D2R or R2D failure.

Therefore, the reusage of the original CFRA paging message as a trigger for new access is both practical and effective. Since the paging message already identifies the target device and indicates the resource for CFRA.

Proposal 5a: In case reader detects a failure in CFRA, reader can resend the CFRA paging message to initiate a new round of CFRA procedure unless CN indicates otherwise.

For the maximum number of resending a CFRA paging message, if the CN does not configure it, it is recommended to be determined by the reader implementation, for example, 5 times retransmissions can be considered appropriate for reliability.

Proposal 5b: The maximum number of resending for the CFRA paging message, if not configured by CN, can be determined by implementation.

Proposal 5c: As a baseline, it is recommended to allow at least 5 times of paging message resending in case of CFRA failure. FFS it can also be applied to A-IoT Msg3 retransmission triggered by A-IoT Msg2.

For A-IoT Msg3, if device is low in energy, multiple retransmissions may cause device exhaust all the energy, maybe device can skip some D2R transmission in case of low energy.

  - Details on CBRA

    - CBRA trigger message

Form the device’s perspective, upon reception of a CBRA paging message, a device requires some time to randomly select one access occasion and generate its random ID, and if the random seed is used, the random seed of selecting occasion and random ID had better to be different to avoid potential A-IoT Msg3 failure.

This behavior is different read and write which merely need. Therefore, the first CBRA trigger message should not to be too close to the paging message to leave some time for device operations.

If the first CBRA trigger were merged into the paging payload, the device’s processing timeline would be compressed or a longer time threshold for device responding should be specified. As the CBRA trigger message is anyway inevitable, merging CBRA trigger message to paging has more specification impact.

Observation 3a: Devices in CBRA need some time to randomly select an access occasion and generate a random ID. Merging the trigger message into the paging message may require a longer response time threshold.

Observation 3b: RAN1 may need to define a new time threshold specifying the interval of paging message and A-IoT Msg1 if paging message is also the first CBRA trigger message.

Proposal 6: To minimize specification impact, the first CBRA R2D trigger message is suggested not to merge into the paging message.

In practical, some devices may fail to detect one or more CBRA trigger messages due to fading or interference. Hence, RAN needs to consider how to address CBRA trigger message failure.

Rather than introducing complex acknowledgement or feedback signalling, the reader can simply transmit additional trigger messages that map to extra access occasions. This best‐effort repetition ensures that devices which miss an earlier trigger still have further opportunities to commence their random‐access procedures, without penalizing the overall system with excessive control overhead.

Observation 3c: Devices that miss their access occasions due to CBRA R2D trigger message failure can be addressed by reader sending additional trigger messages and offering more access occasions.

Given the CBRA trigger message can be massive and repetitive, it is neither necessary nor efficient to enforce byte alignment or include per‐message CRC protection by mandatory. A minimal, short trigger message minimizes resource occupation and processing burden on both network and device. Occasional missed triggers have negligible impact on aggregate access performance, so omitting alignment padding and CRC fields reduces overhead without compromising overall reliability.

Proposal 7: The CBRA R2D trigger message should be short and simple for efficiency.

Consider the following RAN1 agreement, when two access occasions are granted, the indication of how many access occasions are available could reside either in the paging message or within the dedicated trigger itself.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

From our perspective, embedding this indication in the paging message slightly reduces repetitive overhead, since it only needs to signal once per paging round. However, should the NW wish to support both X=1 and X=2 in the same paging round, it’s preferred to introduce a specialized, lightweight trigger message for X=2.

Proposal 8: For simplicity, reader should indicate X=1 and X=2 for time domain resources in the A-IoT paging message, making it effective for the entire paging round.

We don’t have strong preference and we fail to see the necessity to change the value of X within a paging round. However, with the upcoming endorsement, RAN2 should consider this issue

    - A-IoT Msg1 and related device behaviour

Prior to transmitting the A-IoT Msg1, a device should both randomly select one of the indicated candidate access occasions provided by the reader and generate its 16-bit random ID for contention resolution. Therefore, A-IoT shall implement a (pseudo-)random number generator (RNG).

In practice, the specification generally leaves implantation details of RNG to device vendors but specifies the requirements on the RNG, e.g., uniformity within the given random number range, independence to avoid devices generating the same random number, period length before RNG generating the same random number sequence, and even the efficiency of RNG, etc. The detail can be further discussed, and it may be up to RAN5.

Proposal 9a: A-IoT shall implement a (pseudo-)random number generator (RNG) to randomly select access occasion and generate the 16-bit random ID, RAN2 may specify some requirements, e.g., uniformity and independence later.

However, we deem one issue notably worthwhile. Supposing multiple devices generate the same random number, and the random number is reused in access occasion selection, for example, the random number is Y and there are N access occasions, devices then select access occasion M, where M ≡ Y (mod N), then the A-IoT Msg3 is a definitely failure. To avoid this issue, random number used in resource selection and random ID must be different.

Proposal 9b: To avoid the devices that generate the same random ID also select the same access occasion and thus A-IoT Msg3 collides, device should generate 2 independent random numbers for access occasion selection and random ID.

Also, in order to avoid, the device failed in the last attempt still fails in the next round, device should refresh its random ID by mandatory.

Proposal 9c: Device shall generate a new random ID for every selected paging round to avoid random ID collision.

In CBRA, the A-IoT Msg2 performs two core functions, i.e., echoing back the 16-bit random ID of each device that successfully resolved contention and indicating device to further send A-IoT Msg3. Some multiplexing proposals suggest to include an explicit number of the echoed random ID. However, as the length or TB size of A-IoT Msg2 is known by devices, indicating the number of echoed random ID redundant.

For random ID collision, If A-IoT Msg2 is multiplexed and random ID collisions occur, especially in FDM or TDM with X=2, a device may not recognize whether the echoed random ID in Msg2 refers to itself or not. Making matters worse, the reader also remains unaware of any random ID collision until device sends A-IoT Msg3 and fails. Consequently, random ID collisions are ultimately resolved through re-access that initiated by the NACK message after the A-IoT Msg3 failure.

Observation 4: In FDM or TDM with X=2, random ID collision in A-IoT Msg1 can be identified by A-IoT Msg3 failure and reader can use the NACK message to initiate re-access for random ID collided devices.

Proposal 10a: The random ID collision can be addressed by existing re-access mechanism triggered by NACK message after A-IoT Msg3.

Proposal 10b: For endorsement, any further enhancement or optimization to address random ID collision should be postponed to Rel-20.

    - A-IoT Msg2

By contrast, including the AS ID in A-IoT Msg2 offers a clear benefit, i.e., indicating of AS ID in A-IoT Msg2 doesn’t additional signalling exchanges. This binding can be achieved with minimal overhead by appending a one-bit indicator immediately after the echoed RN16. Such an approach avoids reserving unused bit-fields while still providing enough information.

Regarding the A-IoT Msg3 grant which is still under discussion in RAN1, it should be captured in the A-IoT MAC RAR. Hence, the A-IoT MAC RAR should consist of echoed random ID, R2D grant and an optional AS ID.

Whether the length of A-IoT Msg3 grant is fixed or not is not clear yet as in FDM or TDM when X=2, i.e., when A-IoT Msg2 is multiplexed, some indication in grant may be shared and therefore be omitted. Hence, we choose to include an L field to indicate the length of the Msg3 D2R grant.

Similarly, the A field is to indicate the present of AS ID, if A is set to 0, it means that the random ID is reused as the AS ID.

Proposal 11a: A-IoT Msg2 may consist of multiple A-IoT MAC RARs.

Proposal 11b: A-IoT MAC RAR may consist of an A filed indicating the presence of AS ID, echoed Random ID, and D2R grant for A-IoT Msg3 whose details is up to RAN1.

The following are the examples of proposed A-IoT Msg2 formats.

Figure 2.2.3-1 An example of A-IoT MAC Random Access Response

Figure 2.2.3-2 An example of A-IoT MAC PDU consisting of A-IoT MAC RARs

As agreed before, resending A-IoT Msg3 can trigger A-IoT Msg3 retransmission,

    - A-IoT Msg3

Since A-IoT Msg3 does not support multiplexing or segmentation, its format is simplified and only needs to include the Device ID as its resource is indicated inclusively.

Proposal 12a: A-IoT Msg3 should consist of device ID only.

Another issue is the validity of the AS ID, as analysis before, devices that generate the same random ID and select the same access occasion can only be distinguished by A-IoT Msg3 collision. With regarding to the former agreements that device consider its access attempt as success when receiving A-IoT Msg2 echoed with the same random ID and the AS ID is assigned also in A-IoT Msg2, it’s afraid that even if A-IoT Msg3 fails, the device may still consider its AS ID as valid.

Observation 5: For current RAN2 agreements, the devices that generate the same random ID and select the same access occasion will also be assigned with the same AS ID. Only the NACK message can indicate A-IoT Msg3 failure and AS ID invalidity to these devices.

Proposal 12b: Device should consider the AS ID is invalid if it receives the NACK message echoed the same AS ID.

    - A-IoT Msg4, or the NACK message

For the NACK message, or A-IoT Msg4, its main purpose is to indicate the A-IoT Msg3 transmission failure and trigger re-access, it’s suggested to be sent before the next CBRA triggering message to reduce the time that device needs to monitoring the PRDCH.

Proposal 13: To shorten the time device monitoring PRDCH, the NACK message, or A-IoT Msg4, should be sent before next CBRA trigger message.

And in FDMA, similar to multiplexed A-IoT Msg2, multiplexed NACK is also helpful to reduce some signalling overhead. However, for FDMA, how to reduce the NACK signaling overhead can be further discussed. There are two options for multiplexing of NACK message, both are only sent in case A-IoT Msg3 transmission failure.

- Option 1: only NACK information is sent. Then to identify which device is intended, the AS ID need to be included. If N Msg3s are miss detected, the multiplexed payload size is no smaller than 16*N.

- Option 2: bitmap is used with both NACK and ACK. The number of bits for bitmap can be,

  - Option 2-1: X bits, where X equals to the number of multiplexed Msg1 resources in one access occasion. Devices that send Msg3 exactly know which Msg1 resource it chooses, then it can know which bit among X bits to check. For example, if the corresponding bit is “0”, it means NACK for its Msg3, if the corresponding bit is “1”, it means ACK for its Msg3.

  - Option 2-2: Y bits, where Y equals to the number of devices to be responded in Msg2. Devices receive Msg2 and check the number of successful IDs, and get the location of its bit among Y bits.

For Option 2, since it is likely that the number of multiplexed A-IoT Msg1s is no larger than 8, the overhead of option 2 is smaller than option1, since no AS ID is included.

Figure 2.2.6-1: Two options for A-IoT Msg4 NACK feedback of multiplexed A-IoT Msg3

Proposal 14a: Similar to A-IoT msg2, the NACK message can also be multiplexed if A-IoT Msg3 is FDMed or TDMed with X=2.

Proposal 14b: For multiplexed A-IoT Msg3 transmission, two options for NACK feedback can be studied, both are only sent when NACK exists for Msg3,

Option 1: only NACK information is sent with AS ID to identify device

Option 2: bitmap is used with both NACK and ACK information.

    - CBRA Failure detection

When a CBRA attempt fails, it most often stems from one of three root causes:

- The A-IoT Msg1 failure results by either collision with other device(s) or reception failure at reader side. Reader will not send A-IoT Msg2 for both situations, hence device is aware of its access failure and waits for re-access.

- A-IoT Msg2 reception failure. A-IoT Msg2 may simply not reach the device. Device naturally consider its access attempt as failure and waits for re-access.

- A-IoT Msg3 failure, including reader fails to receive A-IoT Msg3, A-IoT Msg3 collision due to multiple devices generates the same random ID. Reader can use NACK message to indicate device for re-access.

How the reader determining that an access attempt has indeed failed or not can likewise be handled without excessive signalling. From the device’s perspective, a time threshold and energy detection technique used for SR in PUCCH can be reused to determine whether device has sent any D2R message or not. And CRC can be used to check whether the R2D message is correct.

Proposal 15a: RAN2 agree the WA that reader should consider the access attempt as failure and initiate re-access when:

- A-IoT Msg1 collision, where reader detects the energy of the D2R message but cannot decode it.

- A-IoT Msg2 reception failure at device side, where reader detects no energy on the D2R resource for A-IoT Msg3. Reader can resend the same A-IoT Msg2, but if it’s not improving, reader should initiate re-access and consider the AS ID as invalid.

- A-IoT Msg3 failure caused by random ID collision, where reader detect energy on the D2R resource for A-IoT Msg3 but cannot decode it or CRC is wrong.

- A-IoT Msg3 failure caused by reception and transmission failure, where reader detects no energy on the indicated resource or detects energy but fails to decode it.

Similar, device can also use the time threshold and energy detection technique to determine whether access attempt is failure or not.

Proposal 15b: RAN2 agree the WA that device should consider the access attempt as failure and wait for re-access when:

- A-IoT Msg1 collision, or A-IoT Msg1 reception failure at reader side, or A-IoT Msg2 reception at device side, where device receives no A-IoT Msg2 or the received Msg2 is not reflecting its random ID.

- A-IoT Msg3 failure caused by random ID collision, or reception failure, or transmission failure, where device receives the NACK message.

Conclusion

Based on the abovementioned analysis and observations, the following is proposed.

Overall:

Observation 1: It’s beneficial to keep R2D messages which are supposed to be sent massively, e.g., CBRA R2D trigger message and A-IoT Msg2 short and compact to reduce the signaling and energy overhead and accelerate random access procedure.

Proposal 1: Byte alignment is not mandatory at least for R2D message that will be frequently transmitted, e.g., R2D trigger message and A-IoT Msg2.

Proposal 2: RAN2 agrees the following WA on CBRA D2R grant, which can be revisited if RAN1 does otherwise:

- D2R resource indication for A-IoT Msg1 is preconfigured in A-IoT paging message and activated by R2D trigger message.

- D2R resource indication for A-IoT Msg3 is indicated in A-IoT Msg2 by default.

CFRA for multiple devices:

Proposal 3: CFRA for multiple devices is not supported in Rel-19.

CFRA trigger message:

Proposal 4: As CFRA does not need randomization, i.e., device to random select resource and generate random ID, the A-IoT paging message can be reused as the CFRA trigger message.

CFRA failure detection:

Observation 2: In CFRA, reader can use a D2R time threshold, e.g., TD2R_max, CRC and energy detection to detect any D2R or R2D failure.

Proposal 5a: In case reader detects a failure in CFRA, reader can resend the CFRA paging message to initiate a new round of CFRA procedure unless CN indicates otherwise.

Proposal 5b: The maximum number of resending for the CFRA paging message, if not configured by CN, can be determined by implementation.

Proposal 5c: As a baseline, it is recommended to allow at least 5 times of paging message resending in case of CFRA failure. FFS it can also be applied to A-IoT Msg3 retransmission triggered by A-IoT Msg2.

CBRA trigger message:

Observation 3a: Devices in CBRA need some time to randomly select an access occasion and generate a random ID. Merging the trigger message into the paging message may require a longer response time threshold.

Observation 3b: RAN1 may need to define a new time threshold specifying the interval of paging message and A-IoT Msg1 if paging message is also the first CBRA trigger message.

Proposal 6: To minimize specification impact, the first CBRA R2D trigger message is suggested not to merge into the paging message.

Observation 3c: Devices that miss their access occasions due to CBRA R2D trigger message failure can be addressed by reader sending additional trigger messages and offering more access occasions.

Proposal 7: The CBRA R2D trigger message should be short and simple for efficiency.

Proposal 8: For simplicity, reader should indicate X=1 and X=2 for time domain resources in the A-IoT paging message, making it effective for the entire paging round.

A-IoT Msg1:

Proposal 9a: A-IoT shall implement a (pseudo-)random number generator (RNG) to randomly select access occasion and generate the 16-bit random ID, RAN2 may specify some requirements, e.g., uniformity and independence later.

Proposal 9b: To avoid the devices that generate the same random ID also select the same access occasion and thus A-IoT Msg3 collides, device should generate 2 independent random numbers for access occasion selection and random ID.

Proposal 9c: Device shall generate a new random ID for every selected paging round to avoid random ID collision.

Observation 4: In FDM or TDM with X=2, random ID collision in A-IoT Msg1 can be identified by A-IoT Msg3 failure and reader can use the NACK message to initiate re-access for random ID collided devices.

Proposal 10a: The random ID collision can be addressed by existing re-access mechanism triggered by NACK message after A-IoT Msg3.

Proposal 10b: For endorsement, any further enhancement or optimization to address random ID collision should be postponed to Rel-20.

A-IoT Msg2:

Proposal 11a: A-IoT Msg2 may consist of multiple A-IoT MAC RARs.

Proposal 11b: A-IoT MAC RAR may consist of an A filed indicating the presence of AS ID, echoed Random ID, and D2R grant for A-IoT Msg3 whose details is up to RAN1.

A-IoT Msg3:

Proposal 12a: A-IoT Msg3 should consist of device ID only.

Observation 5: For current RAN2 agreements, the devices that generate the same random ID and select the same access occasion will also be assigned with the same AS ID. Only the NACK message can indicate A-IoT Msg3 failure and AS ID invalidity to these devices.

Proposal 12b: Device should consider the AS ID is invalid if it receives the NACK message echoed the same AS ID.

A-IoT Msg4:

Proposal 13: To shorten the time device monitoring PRDCH, the NACK message, or A-IoT Msg4, should be sent before next CBRA trigger message.

Proposal 14a: Similar to A-IoT msg2, the NACK message can also be multiplexed if A-IoT Msg3 is FDMed or TDMed with X=2.

Proposal 14b: For multiplexed A-IoT Msg3 transmission, two options for NACK feedback can be studied, both are only sent when NACK exists for Msg3,

Option 1: only NACK information is sent with AS ID to identify device

Option 2: bitmap is used with both NACK and ACK information.

CBRA failure detection:

Proposal 15a: RAN2 agree the WA that reader should consider the access attempt as failure and initiate re-access when:

- A-IoT Msg1 collision, where reader detects the energy of the D2R message but cannot decode it.

- A-IoT Msg2 reception failure at device side, where reader detects no energy on the D2R resource for A-IoT Msg3. Reader can resend the same A-IoT Msg2, but if it’s not improving, reader should initiate re-access and consider the AS ID as invalid.

- A-IoT Msg3 failure caused by random ID collision, where reader detect energy on the D2R resource for A-IoT Msg3 but cannot decode it or CRC is wrong.

- A-IoT Msg3 failure caused by reception and transmission failure, where reader detects no energy on the indicated resource or detects energy but fails to decode it.

Proposal 15b: RAN2 agree the WA that device should consider the access attempt as failure and wait for re-access when:

- A-IoT Msg1 collision, or A-IoT Msg1 reception failure at reader side, or A-IoT Msg2 reception at device side, where device receives no A-IoT Msg2 or the received Msg2 is not reflecting its random ID.

- A-IoT Msg3 failure caused by random ID collision, or reception failure, or transmission failure, where device receives the NACK message.

References

[1]	R2_129b_ChairNotes_25-04-11_16-45 final