3GPP TSG-RAN WG2 Meeting #130        	                       R2-2504377

Malta, MT, 19th-23rd May 2025

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on time info for NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed whether to include the time info for NW side data collection and achieved the following agreements.

For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

In RAN#129 meeting [2], the following agreement was achieved.

As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

This contribution will discuss the remaining issues on time info for NW side data collection.

- Discussion

In legacy logged MDT reporting, the absolute time info absoluteTimeStamp (48bits, as specified in [3]) when the logged measurement is provided and the relative time info relativeTimeStamp (as specified in [3]) which indicates the time of logging measurement results would be included in UEInformationResponse message. And the time info is used for the network to analyze when the issue occurred and optimize the configuration.

Time domain information, especially periodicity information, is one of the most important parameters for time domain model training for beam prediction. And the absolute time stamp is also important to figure out the suspend of logging due to event not satisfied. In addition, absolute time stamp is also one of the network side information for model training to distinguish the different traffic load at daytime or nighttime.

During last two meetings, RAN2 discussed to include time info for NW side data collection and agreed for temporal domain, the network is made aware whether there is a gap between two consecutive samples, and the UEInformationResponse contains one or more logged measurement entries in chronological order. Some companies indicated that the network can be aware of the time info because the UE is in connected mode, and the continuous samples without exact time info are enough for model training. However, we don’t think so considering the following aspects:

Firstly, RAN2 has agreed to support event-triggered data collection. In this case, the network cannot be aware of when the event is satisfied since it depends on UE’s measurements. Furthermore, the gap between two consecutive samples cannot be known by the network. Therefore, the time info indicating when the event triggered is necessary.

Secondly, even for periodic data collection, the time info is needed since the periodicity can be changed, or the target gNB cannot be aware of the periodicity configuration within source gNB during handover procedure.

Thirdly, the NW-sided model could be trained centrally by OAM using datasets from multiple gNBs, rather than being trained locally at individual gNBs. In such scenario, OAM would lack awareness of the specific time information.

Lastly, if the UE reports the logged data to a different gNB than the one providing the data collection configuration to the UE, e.g. at handover procedure, the target gNB will not be aware of the configuration and time info of logged data.

Observation 1: The time info is necessary for model training considering the following aspects:

- For event-triggered data collection, the network cannot be aware of when the event is satisfied and the gap between two consecutive samples

- For periodic data collection, the periodicity can be changed, or the target gNB cannot be aware of the periodicity configuration within source gNB during handover

- For model training at OAM using datasets from multiple gNBs, OAM cannot be aware of the specific time information

- For the scenario that UE reports the data collected within serving gNB to target gNB, the target gNB does not have the data collection configuration and cannot be aware of the time info

Based on the above analysis, we believe that the time info is necessary for model training and should be included for data collection for model training.

Proposal 1: The time info of logging measurement results should be included for NW side data collection.

Furthermore, considering the gap may be caused by event-triggered logging, handover, full buffer and low power cases, the following options can be considered to indicate the gap between two consecutive samples:

- Option 1: Similar mechanism as logged MDT, i.e. the absolute time stamp which indicates when the logged measurement is provided, and the relative time stamp which indicates the time of logging measurement results

- Option 2: The absolute time stamp which indicates when the logged measurement is provided, the relative time stamp which indicates the start time of logging measurement results, and periodicity info which indicates the period of logging measurement results

Figure 1. An example for data logging

Taking the Fig.1 as an example, if option 1 is adopted, the UE will indicate the absolute time stamp t0 and the relative time stamp for each logging measurement result from t1 to t2 (i.e. 7 relative time stamps) for blue part, from t3 to t4 for green part, and from t5 to t6 for yellow part. If we follow the legacy mechanism, the size of absolute time stamp is 48bits, and the size of relative time stamp is 13 bits, the UE needs to report 308 bits (48bits + 13bits *(8+6+6)) to indicate the time info.

AbsoluteTimeInfo-r16 ::= BIT STRING (SIZE (48))

relativeTimeStamp-r16 INTEGER (0..7200)

If option 2 is adopted, the UE will indicate the absolute time stamp t0, the relative time stamp t1 indicating the time of starting logging, and periodicity1 (e.g. 40ms) for blue part, the relative time stamp t3 and periodicity1 (e.g. 40ms) for green part, the absolute time stamp t5 and periodicity2 (e.g. 80ms) for yellow part. If the size of periodicity info is 5bits, the UE needs to report 102 bits (48bits + 13bits*3 + 5bits*3).

From the perspective of signaling overhead, option 2 has smaller overhead than option 1, but from the perspective of complexity, option 1 is simpler than option 2 and aligned with legacy logged MDT. RAN2 can further down-select option1 or option 2 to indicate the gap between the two consecutive samples.

Proposal 2: RAN2 to discuss the following options to indicate the gap between the two consecutive samples:

- Option 1: Similar mechanism as logged MDT, i.e. the absolute time stamp which indicates when the logged measurement is provided, and the relative time stamp which indicates the time of logging measurement results

- Option 2: The absolute time stamp which indicates when the logged measurement is provided, the relative time stamp which indicates the start time of logging measurement results, and periodicity info which indicates the period of logging measurement results

# 3	Conclusion

Here are the proposals for NW side data collection.

Observation 1: The time info is necessary for model training considering the following aspects:

- For event-triggered data collection, the network cannot be aware of when the event is satisfied

- For periodic data collection, the periodicity can be changed, or the target gNB cannot be aware of the periodicity configuration within source gNB during handover

- For model training at OAM using datasets from multiple gNBs, OAM cannot be aware of the specific the time information

- For the scenario that UE reports the data collected within serving gNB to target gNB, the target gNB does not have the data collection configuration and cannot be aware of the time info

Proposal 1: The time info of logging measurement results should be included for NW side data collection.

Proposal 2: RAN2 to discuss the following options to indicate the gap between the two consecutive samples:

- Option 1: Similar mechanism as logged MDT, i.e. the absolute time stamp which indicates when the logged measurement is provided, and the relative time stamp which indicates the time of logging measurement results

- Option 2: The absolute time stamp which indicates when the logged measurement is provided, the relative time stamp which indicates the start time of logging measurement results, and periodicity info which indicates the period of logging measurement results

# 4	Reference

- Draft_RAN2_129bis_Meeting_Report_v2

- R2-2501702 RAN2#129 Meeting Report

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification