3GPP TSG-RAN WG2 Meeting #130       	                      R2-2504381

Malta, MT, 19th-23rd May 2025

Agenda item:	8.3.5

Source:	CMCC

Title:	Discussion on performance monitoring for AI Mob

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 had the following agreements on performance monitoring for AI Mob:

For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

This contribution will discuss the performance monitoring metrics for RRM measurement prediction with UE-sided model and NW-sided model, and for measurement event prediction with UE-sided model.

- Discussion

- 2.1 NW-side performance monitoring for UE-sided model

During the discussion on performance monitoring for beam management in AI/ML PHY, NW-side monitoring for UE-sided model means that the network calculates the performance KPI(s) of the model. The specific KPI to be used will be up to NW implementation, which is transparent to the UE. Of course, some assistance information from UE is needed for NW to calculate the metrics. For example, if NW would like to derive Top 1 or Top K beam prediction accuracy, the UE can be configured to report the actual Top 1 or Top K beam IDs, or if the NW would like to derive RSRP difference, the UE can be configured to report the actual beam measurement results. For AI Mob, the same principle can be also applied.

Proposal 1: For NW-side performance monitoring, the specific KPI to be used is up to NW implementation, and assistance information (e.g. actual measurement results) from the UE is needed for NW to calculate the performance metrics.

For RRM measurement prediction, RAN2 discussed temporal domain case A/case B, frequency domain and spatial domain prediction, as well as cell-based and cluster-based option. And for each scenario, the similar sub-use cases are considered for L3 beam level and cell level measurement prediction, where the metrics should be discussed per sub-use case.

- For L3 cell level measurement prediction:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Case 2: To directly predict cell level results based on cell level results

- Case 3: To directly predict cell level results based on beam level results

- For L3 beam level measurement prediction:

- Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results

- Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results

- Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results

For case 1, model output is L1 beam level results. To monitor the model performance, the predicted and actual L1 beam RSRP(s) as assistance information can be provided to the network.

For case 2 and case 3, model output is L3 cell/beam level results. To monitor the model performance, the predicted and actual L3 cell or beam RSRP(s) as assistance information can be provided to the network.

Proposal 2: For RRM measurement prediction, the following assistance information can be provided to the network for NW-side performance monitoring:

- For case 1: predicted and actual L1 beam RSRP(s)

- For case 2 and case 3: predicted and actual L3 cell or beam RSRP(s)

For measurement event prediction, RAN2 studied direct prediction and indirect prediction. During the simulation phase, F1 score (for indirect and direct) and time difference between true event and predicted event reporting (for indirect) are used as performance metrics.

RAN2#127bis Agreements - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection.

In our understanding, for the above metrics, the UE can report the true event occurrence time and predicted event time to the network for calculating F1 score or time difference, and the info can be applied for indirect and direct prediction.

Proposal 3: For measurement event prediction, the UE can report the true event occurrence time and predicted event time/ to the network for NW-side performance monitoring.

- 2.2 UE-side performance monitoring for UE-sided model

UE-side monitoring means that the UE calculates the performance KPI(s) of the UE-sided model.

For RRM measurement prediction, L1 RSRP difference and/or L3 RSRP difference are used as performance metrics during the simulation evaluation. For now, RAN2 can use them as starting point, and additional metrics can be further discussed.

Proposal 4: For RRM measurement prediction, L1 RSRP difference and/or L3 RSRP difference calculated by UE can be reported to the network as performance metrics for UE-side performance monitoring.

For measurement event prediction, some companies proposed that the calculation of F1 score may be complicated for the UE during the online discussion in last meeting. In fact, there may not too many available truth labels for the calculation of performance metrics, for example, the network can handover the UE to target cell once a single measurement event is satisfied. Therefore, we think the UE can also report the true event occurrence time and predicted event time to the network, which is similar to NW-side performance monitoring. For time difference between true event and predicted event reporting (for indirect prediction), we think it is easy for the UE to calculate and report to the network.

Proposal 5: For measurement event prediction, the UE can report the following information to the network:

- The true event occurrence time

- Time difference between true event occurrence time and predicted event time

- 2.3 Performance monitoring for NW-sided model

For NW-sided model, the agreements on beam management in AI/ML PHY can be followed that gNB is responsible for monitoring its own performance. That is, the performance metrics should be calculated by gNB, and assistance information (e.g. actual measurement results) from the UE can be provided to the network, which is similar to the NW-side performance monitoring for UE-sided model.

Proposal 6: For NW-sided model, the network is responsible for monitoring its own performance, and assistance information (e.g. actual measurement results) from the UE can be provided to the network.

# 3	Conclusion

Here are the proposals on performance monitoring for AI Mob.

NW-side performance monitoring for UE-sided model

Proposal 1: For NW-side performance monitoring, the specific KPI to be used is up to NW implementation, and assistance information (e.g. actual measurement results) from the UE is needed for NW to calculate the performance metrics.

Proposal 2: For RRM measurement prediction, the following assistance information can be provided to the network for NW-side performance monitoring:

- For case 1: predicted and actual L1 beam RSRP(s)

- For case 2 and case 3: predicted and actual L3 cell or beam RSRP(s)

Proposal 3: For measurement event prediction, the UE can report the true event occurrence time and predicted event time to the network for NW-side performance monitoring.

UE-side performance monitoring for UE-sided model

Proposal 4: For RRM measurement prediction, L1 RSRP difference and/or L3 RSRP difference calculated by UE can be reported to the network as performance metrics for UE-side performance monitoring.

Proposal 5: For measurement event prediction, the UE can report the following information to the network:

- The true event occurrence time

- Time difference between true event occurrence time and predicted event time

Performance monitoring for NW-sided model

Proposal 6: For NW-sided model, the network is responsible for monitoring its own performance, and assistance information (e.g. actual measurement results) from the UE can be provided to the network.

# 4	Reference

- Draft_RAN2_129bis_Meeting_Report_v2