3GPP TSG RAN WG2 Meeting #130	R2-2504392

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.4

Source:	CMCC

Title:	Considerations on broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

5.	UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

FFS whether and how the association between intended service area and frequencies is acquired.

In this contribution, we would like to provide some further considerations on the open issues for broadcast service in NTN system.

Discussion

Service continuity

In last RAN2 meeting, we have discussed on service continuity and reached a agreement that UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. FFS whether and how the association between intended service area and frequencies is acquired.

Some companies may have the concern about signaling overhead to provide association between intended service area and frequencies, and also indicate that with the mapping between the concerned frequency and one or more MBS FSAIs in the SIB21 is enough to keep service continuity.

However, we should not forget that the reason for enhancing the NTN MBS broadcast is that the intended service area is expected to be smaller than the coverage of a Uu cell. If we do not provide a further association between intended service area and frequencies, UE only has a per cell acknowledgement about the concerned frequency which may go against the original intention of WI scope. Therefore it is better to provide the association between intended service area and frequencies. And we could just consider to provide intended service area ID in the SIB21 instead of intended service area information for signaling overhead reduction.

Proposal 1: It is proposed to provide the association between intended service area ID and frequencies in SIB21, and an example mapping option is as following:

SIB21 information element

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

...,

[[

mbs-AreaInfoList-r19                      MBS-AreaInfoList-r19            OPTIONAL,  -- Need R

mbs-AreaInfo-InterFreqList-r19           MBS-AreaInfo-InterFreqList-r19       OPTIONAL  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-AreaInfoList-r19 ::= SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19

MBS-AreaInfo-InterFreqList-r19 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-AreaInfo-InterFreq-r19

MBS-AreaInfo-InterFreq-r19 ::= SEQUENCE {

dl-CarrierFreq-r19         ARFCN-ValueNR,

mbs-AreaInfoList-r19       MBS-AreaInfoList-r19

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

-- TAG-SIB21-STOP

-- ASN1STOP

MBS interest indication

We discussed in RAN2 #127bis meeting that whether a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area without convergence.

As current specification, the MBS interest indication procedure is to inform the network that the UE in RRC_CONNECTED is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation. And the trigger condition is described in section 5.9.4.2 of TS 38.331.

- 5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

In the contribution [2], it is mentioned that initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area could help NW to choose a suitable cell providing the broadcast service for this UE. However, per the exiting mechanism, UE already supports to initiate the MII procedure upon handover as highlighted above. And in NTN, UE is able to determine whether in the intended service area based its own GNSS and service area information provided by network. Further, network may have no acknowledge of the UE location, i.e., UE is in which service area may not be clear at network side. Then whether initiate the MII procedure could be just up to UE itself. Therefore, further enhancement for MBS Interest Indication initiation is not needed.

Observation 1: UE is already able to initiate the MBS Interest Indication procedure in several cases as in existing specification mentioned above.

Proposal 2: For the MBS Interest Indication initiation, we could just follow the legacy mechanism, further enhancement is not needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

Proposal 1: It is proposed to provide the association between intended service area ID and frequencies in SIB21, and an example mapping option is as following:

SIB21 information element

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

...,

[[

mbs-AreaInfoList-r19                      MBS-AreaInfoList-r19            OPTIONAL,  -- Need R

mbs-AreaInfo-InterFreqList-r19           MBS-AreaInfo-InterFreqList-r19       OPTIONAL  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-AreaInfoList-r19 ::= SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19

MBS-AreaInfo-InterFreqList-r19 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-AreaInfo-InterFreq-r19

MBS-AreaInfo-InterFreq-r19 ::= SEQUENCE {

dl-CarrierFreq-r19         ARFCN-ValueNR,

mbs-AreaInfoList-r19       MBS-AreaInfoList-r19

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

-- TAG-SIB21-STOP

-- ASN1STOP

Observation 1: UE is already able to initiate the MBS Interest Indication procedure in several cases as in existing specification mentioned above.

Proposal 2: For the MBS Interest Indication initiation, we could just follow the legacy mechanism, further enhancement is not needed.

Reference

- R2-129bis NR-NTN-IoT-NTN (Sergio)_ EOM

- R2-2408016	Further Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core