3GPP TSG-RAN WG2 Meeting #130                               	                                R2-2504401

St.Julians, Malta,  May 19th – 23rd, 2025

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the left issue of RLC enhancements

WID/SID:	          NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

In the last RAN2 #129bis meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this contribution, we provide our views on the RLC enhancements.

Discussion

Autonomous retransmissions

It is agreed in RAN2 129 meeting that only a single autonomous retransmission will be triggered per RLC SDU.  Therefore one shot autonomous retransmission is considered as the last attempt to retransmit the RLC SDU. The round time for the next RLC SDU retransmission and its status report after one shot autonomous retransmission may exceed the PDB of this SDU. After one shot autonomous retransmission is submitted to lower layer, the RLC transmitting side may receive the discard indication for this RLC SDU from PDCP or the ACK/NACK for single autonomous retransmission in the STATUS report. If the RLC transmitting side receives the NACK for single autonomous retransmission in the STATUS report, the retranmsission couter RETX_COUNT could be incremented by one. If the RETX_COUNT reaches maxRetxThreshold and RLC indicates to upper layers that max retransmission has been reached as legacy.

Proposal 1: If the RLC transmitting side receives the NACK for the single autonomous retransmission in the STATUS report, the retranmsission couter RETX_COUNT could be incremented by one. If the RETX_COUNT reaches maxRetxThreshold and RLC indicates to upper layers that max retransmission has been reached as legacy.

Enhanced polling mechanisms

One left FFS is how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.

The purpose of polling enhancement is to fasten the polling mechanism for delay-critical data. The polling should be triggered as soon as possible without waiting the accumulated number of PDU or Bytes exceed the threshold pollPDU or pollByte when the remaining time of an RLC SDU or an RLC SDU segment falls below a threshold as agreed last meeting. Therefore the multiple pollings may be included in one transmission opportunity and no enhancement is need to avoid excessive polling.

Observation 1: The purpose of polling enhancement is to fasten the polling mechanism for delay-critical data. The polling should be triggered immediately without waiting the accumulated number of PDU or Bytes exceed the threshold pollPDU or pollByte when the remaining time of an RLC SDU or an RLC SDU segment falls below a threshold as agreed last meeting.

Proposal 2: The multiple pollings may be included in one transmission opportunity and no enhancement is need to avoid excessive polling.

Avoidance of unnecessary retransmission

The RLC failure is indicated from RLC to upper layer when max retransmission has been reached, which may lead Radio Link Failure (RLF) and bring the interruption. If the RLC AM transmitting entity discards an obsolete RLC SDU and the maximum number of retransmissions is reached, the indication from RLC to upper layer, which indicates max retransmission has been reached as RLC failure, shall not be triggered due to the retransmissions discarding.

Proposal 3: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Delay-reporting data volume

Considering the implementation complexity, the control PDU, e.g. STATUS PDU and data to be retransmitted should be reported in the shortest reporting threshold in DSR.

Proposal 4: The control PDU, e.g. STATUS PDU and data to be retransmitted should be reported with the shortest reporting threshold in DSR.

PDCP discard

In the congestion case, the gNB may perform the PDCP SDU discarding based on PSI. These SDUs considered as low importance may be configured with a dedicated timer discardTimerForLowImportance [1] which may be set shorter than the timer discardTimer for higher priority SDU, therefore SDUs considered as low importance may be discarded firstly in the case of congestion. Automatic retransmission and polling enhancement is typically to ensure reliable delivery for the delay-critical data but not for the less critical data. Since the loss of low-importance SDUs does not significantly affect the overall system performance, it is not necessary for automatic retransmission and polling enhancement to apply to discard for PDUs with low importance.

Proposal 5:  It is unnecessary for automatic retransmission and polling enhancement to apply to discard for PDUs with low importance.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Proposal 1: If the RLC transmitting side receives the NACK for the single autonomous retransmission in the STATUS report, the retranmsission couter RETX_COUNT could be incremented by one. If the RETX_COUNT reaches maxRetxThreshold and RLC indicates to upper layers that max retransmission has been reached as legacy.

Observation 1: The purpose of polling enhancement is to fasten the polling mechanism for delay-critical data. The polling should be triggered immediately without waiting the accumulated number of PDU or Bytes exceed the threshold pollPDU or pollByte when the remaining time of an RLC SDU or an RLC SDU segment falls below a threshold as agreed last meeting.

Proposal 2: The multiple pollings may be included in one transmission opportunity and no enhancement is need to avoid excessive polling.

Proposal 3: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Proposal 4: The control PDU, e.g. STATUS PDU and data to be retransmitted should be reported with the shortest reporting threshold in DSR.

Proposal 5:  It is unnecessary for automatic retransmission and polling enhancement to apply to discard for PDUs with low importance.

Reference

- 3GPP TS 38.323 V18.4.0 (2024-12), Packet Data Convergence Protocol (PDCP) specification(Release 18)

- 3GPP TS 38.322 V18.2.0 (2024-12), Radio Link Control (RLC) protocol specification (Release 18).