3GPP TSG-RAN WG2 Meeting #130	R2-2504413

St. Julian’s, Malta,19th  – 23rd  May,  2025

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-side Beam Management

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we discuss open issues as well as FFS items from previous meeting related to functionality-based LCM for UE-side AI/ML beam management regarding applicability reporting, management of poorly performing configurations, and an indication of active data collection options for UE requests for data collection, and performance monitoring. This document concludes with our Stage 2 text proposal concerning active, inapplicable periodic beam prediction configurations in the Applicability Reporting Clause detailed in Section 7.

# 2	Open Issues

The rapporteur provided a list of open issues for each agenda item prior to this meeting, RAN2#130, here. We will discuss those relevant to 8.1.2.2 in this section.

## 2.1	(RRC-1) Cause of inapplicability

RRC-1 Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec.

An AI/ML-enabled configuration could be determined to be inapplicable in three main cases. In all cases, the UE considering the configuration, NW-side additional conditions (or associated IDs), if provided, and UE-side additional conditions.

- The UE was unable to identify a model in the UE or in a model store available to the UE, which could enable the configuration.

- The UE was unable to identify a model in the UE but was able to identify a model in a model store available to the UE, which could enable the configuration.

- The UE was able to identify a model in the UE or in a model store available to the UE, but due to performance constraints, it cannot run the model.

In the first case, a model is not available in the UE or for transfer to the UE. We could consider this permanent inapplicability. In this context, permanent means in the time of an RRC session.

Observation 1: An AI/ML-enabled configuration could be considered permanently inapplicable, in the context of an RRC session, if an applicable model neither exists in the UE nor in a model store available to the UE.

In the second and third cases, the inapplicability is temporary, either resolved by downloading a model or by waiting for performance constraints to dissipate, e.g., by charging the device’s battery. To the NW, there is no actionable difference between the UE reporting inapplicability due to needing to download a model or due to performance constraints since in the future, for both cases, the configuration could become applicable.

Observation 2: An AI/ML-enabled configuration could be considered temporarily inapplicable in two cases: if the UE has access to a model store with an applicable model or if the reason for inapplicability is related to UE performance constraints such as processing capacity or battery capacity.

Proposal 1: Support two cause values for inapplicability: permanent inapplicability and temporary inapplicability. Setting the appropriate value is up to UE implementation.

## 2.2	(RRC-2) Content of otherConfig to enable applicability reporting in UAI

RRC-2 Issue description: It is not yet clarified what the content (if any) of the UAI configuration should be, to enable the UE to report applicability in UAI, e.g. applicability updates/changes as agreed for option A. For instance, this configuration could be just a flag or could contain further information.

We do not see the need to differentiate between reporting the applicability of configurations per use case. Therefore, there is no need to future-proof by creating use case-dependent configurations for enabling applicability reporting with UAI. We do not have any further agreements to make other aspects optional, such as reporting a cause. Therefore, otherConfig can simply include a flag enabling or disabling the reporting of updates to applicability of AI/ML configurations.

Proposal 2: Support only a flag to enable or disable the reporting of updates to applicability of AI/ML configurations. The absence of the configuration implies that reporting of updates to applicability is disabled.

## 2.3	(RRC-5) Reporting behaviour of inapplicable, active periodic inference configuration

In RAN2#129, it was agreed “if option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5)” [1].  Moreover, according to the RAN2#128 agreement “when a functionality becomes non-applicable the UE doesn’t autonomously deactivate, the NW is expected to deactivate an activated functionality when it receives a report from UE that it is inapplicable” [2]. Additionally, in RAN2#129bis, it was agreed that the UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly upon receiving one or more full inference configuration(s) via RRCReconfiguration message.

However, even if a full inference configuration for beam prediction is provided with reportConfigType=’periodic’, the UE can report when the applicability of the configuration becomes inapplicable using UAI. Because the UE cannot deactivate the inference reporting autonomously, a potential ambiguity is introduced because the quality and usefulness of the inference reports are unknown. See Figure 3.1-1 for details.

Figure 3.1-1. Ambiguity of UE behaviour after an active periodic inference configuration becomes inapplicable

Observation 3: An active beam prediction configuration may become inapplicable after it is activated. During this time, the outputs of the model enabling the configuration could be unreliable or ambiguous.

An approach to resolve the above issue is to define a new UE reporting behaviour. Instead of reporting Set A beam predictions, the UE would report Set B beam measurements after the UE reports the inapplicability of a beam prediction reporting configuration. This avoids wasting configured air interface resources but also supports performance monitoring at the NW (e.g., in case of Type-1 Option 1).

Proposal 3: When an active periodic beam prediction configuration becomes inapplicable, the UE shall suspend the transmission of Set A beam predictions and reporting of Set B measurements corresponding to the inapplicable configuration until the UE is reconfigured by the NW.

Proposal 4: RAN2 to endorse the text proposal within Applicability Reporting Clause for Stage 2 running CR (changes are highlighted in orange in Section 7).

## 2.4	(RRC-7/8) Structure and Content of the Applicability Report

RRC-7 Issue description: It is FFS whether the applicability report for option B (sets of inference related parameters) can be included in RRCReconfigurationComplete (or if it can only be included in UAI).

RRC-8 Issue description: Considering that both Option A and B are to be specified, we need to discuss how the 2 options co-exist, whether there is any interaction between them etc. For instance, it is not clear whether both option A and option B can be configured or not. This can affect signaling design of ApplicabilityReportList.

The RRC applicability report will be used to report the applicability of any AI/ML-enabled configuration, or any configuration subject to the applicability determination procedure, for all RRC-based use cases, e.g., CSI feedback enhancement, Beam Prediction, and Mobility, in the future. Every configuration which we have discussed thus far in the use cases has an ID, and that ID, e.g., CSI-ReportConfigId, can be used to identify AI/ML-enabled configurations and to report their applicability, and updates thereto. The same assumption can be made for sets of inference-related parameters.

Below, we provide a possible ApplicabilityReport-r19, which we find to be extensible to future use cases. Lastly, we provide ASN.1 for context, which closely matches the running CR for RRC for AI/ML Phy: ApplicabilityReportList-r19; UEAssistanceInformation-v1900-IEs; and RRCReconfigurationComplete-v1900-IEs.

In Figure 3.3-1, the applicabilityIdType is an ENUMERATED value which identifies the type of ID, e.g., csi-ReportConfigId which is pointed to by applicabilityId, which is an INTEGER with a range to be determined, which can hold any ID type. The benefit of this option is that we can include spare values in the ENUMERATED, which can be updated in the future instead of additional ID types.

The inapplicabilityCause is an ENUMERATED value which identifies the type of cause, which is either permanent or temporary for each full inference configuration and set of inference-related parameters.

Additionally, the serving cell index is included and applicable to full inference configurations and sets of inference-related parameters. For the full inference configuration, the serving cell index deconflicts the overlapping configuration IDs, which are unique per cell, and for the sets of inference-related parameters, the serving cell index is used to report whether the parameters are applicable to each serving cell.

ApplicabilityReport-r19 ::= SEQUENCE { configIdType ENUMERATED { csi-ReportConfigId, use-case-X-Rel-19, use-case-Y-Rel-19, spare3, spare4, spare5, spare6, spare 7 }, configId INTEGER (0..255), applicability ENUMERATED {applicable, inapplicable}, inapplicabilityCause ENUMERATED {temporary, permanent} OPTIONAL, -- Cond inapplicable servingCellIndex ServCellindex }

Figure 3.3-1: ApplicabilityReport-r19

The supporting elements for signalling the ApplicabilityReport are shown in Figure 3.3-2 below, and are captured similarly in the running CR. The same ApplicabilityReportList container would be used in UEAssistanceInformation and in RRCReconfigurationComplete, where applicable.

ApplicabilityReportList-r19 ::= SEQUENCE (SIZE(1..maxNrOfApplicabilityReports)) OF ApplicabilityReport-r19 UEAssistanceInformation-v1900-IEs ::= SEQUENCE { applicabilityReportList ApplicabilityReportList-r19 OPTIONAL -- Need R nonCriticalExtensions SEQUENCE {} } RRCReconfigurationComplete-v1900-IEs ::= SEQUENCE { applicabilityReportList ApplicabilityReportList-r19 OPTIONAL -- Need R nonCriticalExtensions SEQUENCE {} }

Figure 3.3-2: ApplicabilityReportList-r19, UEAssistanceInformation-v1900-IEs, and RRCReconfigurationComplete-v1900-IEs Adapted from the running CR.

Observation 4: (RRC-7) The same structure, i.e., ApplicablityReportList, can be used for reporting applicability in RRCReconfigurationComplete and in UEAssistanceInformation. By using the same structure, the applicability of  sets of inference related parameters, or Option B, can be reported the same way as full inference configurations.

Observation 5: (RRC-8) Full inference configurations and sets of inference-related parameters would be configured separately and have their own IDs. Therefore, they can be reported simultaneously using the same ApplicabilityReport.

Proposal 5: To allow future-proof extensibility, adopt the structure of Figure 3.1-1of the ApplicabilityReport-r19 and its inclusion in UEAssistanceInformation-v1900-IEs and RRCReconfigurationComplete-v1900-IEs, including the reporting for a combination of full inference configurations and sets of inference-related parameters with the ability to indicate the configuration ID type (e.g., CSI-ReportConfigId, InferenceRelatedParametersId) and the configuration ID for which applicability is being reported.

## 2.4	(RRC-9/10) Definition of ‘applicable AI/ML functionality’

This section will address the open issues RRC-4 (Definition of ‘applicable AI/ML functionality’ and RRC-6 (Terminology throughout RRC specs) provided in the RRC open issues list for RAN2#130. The scope of the open issue will be expanded to include how the applicability determination procedure could be implemented in a way that does not require the terminology.

RRC-9 Issue description: How to update the definition of ‘applicable AI/ML functionality’ in clause 3.1, e.g. replace 'functionality', and align it with RAN1 specs and with TS 38.300.

RRC-10 Issue description: It is FFS how to consistently update the AIML related terminology throughout the document (e.g. whether to adopt the terms 'measurement prediction', 'prediction configuration', etc.).

The following agreements are relevant to the discussions on terminology in this section and the next.

RAN2#129b: Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

RAN2#127b: For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

RAN2#127b: For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

While it is clear that throughout the SI and WI on AI/ML, we have used a mixture of terms related to applicability and functionality, the terms have only served to divide AI/ML-based operations from legacy operations. Therefore, we should focus on the main objective with having used these terms, which is to differentiate procedures which relate to AI/ML-based configurations from those which do not. Given the time spent on terminology and not having reached consensus, we propose a way forward to generically describe new procedures developed for AI/ML.

In the agreements replicated above, we have agreed not to define AI/ML specific terminology and to reuse existing procedures and terminology when possible. In an agreement from the last meeting, we even refer to “full inference configurations” instead of “full inference functionalities”. Clearly, it is natural to describe AI/ML-based configurations instead of forcing the term functionality.

Observation 6: In our latest agreements, we have been able to describe AI/ML-related behaviours without using the term functionality, and in previous meetings, we have agreed to avoid the use of new AI/ML-specific terminology when legacy terminology is sufficient.

Proposal 6: Do not use the term “functionality” in 38.331 and 38.300.

We think that for the purpose of describing the procedures related to applicability determination and reporting, we can describe aspects of a configuration which would make it subject to the applicability determination procedure. That is, for each configuration IE type, for which there is only one in the case of Beam Management, the CSI-ReportConfig, we can identify a set of parameters which would surely identify a configuration as one subject to applicability determination and reporting.

For example, if a CSI-ReportConfig contains the field typeOfBMCase1andBMCase2, being discussed by RAN1 in their running parameter list, found here, a procedure could detect the field and require reporting of applicability. Note that there is no mention of AI/ML, models, or functionality in the resulting procedure.

1> for each csi-ReportConfig included within csi-ReportConfigToAddMod 2> if the field typeOfBMCase1andBMCase2 is included: 3> determine the applicability of the csi-ReportConfig 3> include an entry in the applicability report list: 4> if the csi-ReportConfig corresponds to an SCell: 5> include the SCellIndex 4> set the configuration ID to the CSI-ReportConfigId 4> set the configuration type to CSI-ReportConfigId 4> if the applicability of the csi-ReportConfig has been determined to be applicable 5> set the applicability to applicable 4> else 5> set the applicability to inapplicable 5> if configured to provide a cause of inapplicability 6> determine the cause of inapplicability 6> set the cause to temporary or permanent inapplicability

Observation 7: Some fields in CSI-ReportConfig will only be present in AI/ML-enabled configurations, such as typeOfBMCase1andBMCase2. These fields can be used to uniquely identify beam prediction AI/ML-enabled configurations based on CSI-ReportConfig.

Proposal 7: Use a field or list of fields only available in AI/ML-enabled configurations, e.g., typeOfBMCase1andBMCase2, to identify configurations which are subject to the applicability determination procedure.

Issue RRC-6 mentions a “measurement prediction” and “prediction configuration”, but it isn’t clear why these terms are required outside of Stage 2. In Stage 3, we can be more implicit. If a reportQuantity is set to a predicted value, then it is clear that the configuration is for reporting predictions. Further description is not required.

Observation 8: Stage 2 can describe the nature of an AI/ML or prediction-based configuration. In Stage 3, the field names can describe the purpose of a configuration without the supporting text redescribing what is clear from the field names.

Proposal 8: Outside of field names, there is no need in 38.331 to explicitly specify whether a configuration is for prediction or whether reported measurements are predictions.

# 3	Other Open Issues

## 3.1	Repeated indication of applicability for poorly performing configurations

RAN2 agreed that the UE is the entity responsible for determination of the applicability of an AI/ML-enabled configuration, enabled by a UE-side model. The applicability depends in part on the evaluation of conditions involved in the consistency between training and inference. Some conditions are based on a comparison between model meta data and the configuration provided to the UE and others are evaluated based on aspects related to the UE, e.g., its velocity or the inputs from various sensors. What has not been considered thus far, however, is the consideration of historical performance of a configuration enabled by AI/ML models.

A UE could frequent a cell, each time reporting that a configuration is applicable. However, due to a change in conditions, such as the construction of a new building or the presence of a large vehicle in front of the cell, or the lack of consideration for different weather conditions, a configuration reported as applicable consistently performs poorly.

Observation 9: A poorly performing inference configuration could consistently perform poorly because the applicability determination procedure is unable to account for unforeseen factors which could impact model performance.

To prevent the UE from repeatedly reporting a poorly performing functionality/configuration as applicable, we propose that the NW should be able to provide feedback to the UE while reconfiguring the UE due to a poorly performing AI/ML-enabled configuration. The UE could use the feedback to augment its applicability determination criteria.

Observation 10: An indication of poor performance when reconfiguring a UE configured with an inference configuration could be used by the UE to help determine applicability of a configuration.

Proposal 9: The NW can provide a cause with reconfiguration, e.g., modifying or releasing an inference configuration, indicating that the configuration performed poorly.

# 4 Remaining aspects on UE data collection request

In this section, the remaining aspects concerning RRC-3 (open issue being discussed here on UE data collection request) are detailed.

## 4.1	Indication of active data collection option

Many UEs will likely have the same data collection needs, especially those of the same hardware model or vendor. Therefore, it could be that the reference signals for data collection are already transmitting when the UE indicates its preference to the NW. Therefore, we think it could be useful to indicate to the UE which of the candidate UE-side data collection configurations are already actively transmitting (e.g., CSI-ResourceConfigId=1 in Figure 4.1).

We propose the following ASN.1 structure in Figure 4.1-2 based on the RAN1 agreement from RAN1#120 stating that “for UE-sided model, for configuring the resource for data collection purpose, support CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report”, with the addition of indicating which configuration candidates are active. We should keep in mind that other use cases will be supported in the future when designing this configuration and propose a similar flexible structure for providing IDs as we proposed in Section 3.3 on the Applicability Report.

UE-DataCollectionPrefConfig-r19 ::= SEQUENCE (SIZE(1..maxNrUE-DataCollectionPrefConfigs)) OF UE-DataCollectionPrefConfigItem-r19

UE-DataCollectionPrefConfigItem-r19 ::= SEQUENCE {

ue-DataCollectionConfigIdType        ENUMERATED { csi-ReportConfigId, use-case-X-Rel-19, use-case-Y-Rel-19, spare3, spare4, spare5, spare6, spare 7 },

ue-DataCollectionConfigId            OCTET STRING,

ue-DataCollectionConfigStatus        ENUMERATED { active } OPTIONAL

}

Figure 4.1-2 indication of data collection for UE-DataCollectionPrefConfig-r19

Observation 11: Other UEs may have already requested the same preference for a UE data collection configuration as another UE and the configuration may already be active.

Proposal 10: Include in the configuration for data collection configuration candidates, e.g., UE-DataCollectionPrefConfig, an indication whether a data collection configuration candidate is already active, e.g., for other UEs which already indicated the same preference.

Proposal 11: A UE can immediately begin performing measurements on a data collection configuration candidate which is already active and can indicate its preference for an active data collection configuration candidate.

# 5	UE- sided model performance monitoring

UE is configured with L1-RSRP measurements for inference by the network. This configuration includes measurements required as inputs to the beam prediction configuration, e.g., measurements of Set B for beam prediction Case-1 or measurements of Set A within the observation window for beam prediction Case-2. The beam prediction inference, e.g., L1-RSRP and/or RS index, are reported to the network.

If a UE is configured with Type-1 Option-2 monitoring, then UE is configured with L1-RSRP measurements for monitoring, which are used for UE-based KPI calculation. UE-based KPIs are calculated and reported to the network and network makes LCM decisions based on these reports. The following was agreed in RAN1#120bis regarding the KPI calculation:

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

In Type1-Option 2, we can categorize the reporting Top-M beams to different temporal options, such as periodical, semi-persistent and event-triggered. In the periodical or semi-persistent, network can configure UE with periodical reporting or semi-persistent reporting (e.g., via a MAC-CE trigger from the network)  of monitoring metrics. Pertaining to the configuration, UE will report the monitoring metrics (based on L1-RSRP measurements of resource set(s) for monitoring) to the network. On the other hand, for event-triggered reporting, any L1-based event can be considered. What kind of L1 based events could be considered for Top-M beam reporting is up to RAN1. Along with the monitoring of events, reporting a failure event that is associated with multiple performance failure of a configuration related with ML models when a failure happens for an ongoing inference configuration is also beneficial. Monitoring a beam failure event including Set A beams reference signals with reporting early feedback of a failure event is essential specially in case of not receiving fallback configuration from the NW in time before the failure occurs.

Proposal 12: For beam prediction-Case1 and beam prediction-Case2, Type 1 Option 2 performance monitoring for beam prediction related CSI reporting

- Configure each UE with dedicated reference signals from Set A beams for early feedback of a failure event

- Trigger beam failure event reporting via performance monitoring KPI evaluation

# 6	Conclusion

This document has made the following observations:

Observation 1: An AI/ML-enabled configuration could be considered permanently inapplicable, in the context of an RRC session, if an applicable model neither exists in the UE nor in a model store available to the UE.

Observation 2: An AI/ML-enabled configuration could be considered temporarily inapplicable in two cases: if the UE has access to a model store with an applicable model or if the reason for inapplicability is related to UE performance constraints such as processing capacity or battery capacity.

Observation 3: An active beam prediction configuration may become inapplicable after it is activated. During this time, the outputs of the model enabling the configuration could be unreliable or ambiguous.

Observation 4: (RRC-7) The same structure, i.e., ApplicablityReportList, can be used for reporting applicability in RRCReconfigurationComplete and in UEAssistanceInformation. By using the same structure, the applicability of  sets of inference related parameters, or Option B, can be reported the same way as full inference configurations.

Observation 5: (RRC-8) Full inference configurations and sets of inference-related parameters would be configured separately and have their own IDs. Therefore, they can be reported simultaneously using the same ApplicabilityReport.

Observation 6: In our latest agreements, we have been able to describe AI/ML-related behaviours without using the term functionality, and in previous meetings, we have agreed to avoid the use of new AI/ML-specific terminology when legacy terminology is sufficient.

Observation 7: Some fields in CSI-ReportConfig will only be present in AI/ML-enabled configurations, such as typeOfBMCase1andBMCase2. These fields can be used to uniquely identify beam prediction AI/ML-enabled configurations based on CSI-ReportConfig.

Observation 8: Stage 2 can describe the nature of an AI/ML or prediction-based configuration. In Stage 3, the field names can describe the purpose of a configuration without the supporting text redescribing what is clear from the field names.

Observation 9: A poorly performing inference configuration could consistently perform poorly because the applicability determination procedure is unable to account for unforeseen factors which could impact model performance.

Observation 10: An indication of poor performance when reconfiguring a UE configured with an inference configuration could be used by the UE to help determine applicability of a configuration.

Observation 11: Other UEs may have already requested the same preference for a UE data collection configuration as another UE and the configuration may already be active.

And the following proposals:

Proposal 1: Support two cause values for inapplicability: permanent inapplicability and temporary inapplicability. Setting the appropriate value is up to UE implementation.

Proposal 2: Support only a flag to enable or disable the reporting of updates to applicability of AI/ML configurations. The absence of the configuration implies that reporting of updates to applicability is disabled.

Proposal 3: When an active periodic beam prediction configuration becomes inapplicable, the UE shall suspend the transmission of Set A beam predictions and reporting of Set B measurements corresponding to the inapplicable configuration until the UE is reconfigured by the NW.

Proposal 4: RAN2 to endorse the text proposal within Applicability Reporting Clause for Stage 2 running CR (changes are highlighted in orange in Section 7).

Proposal 5: To allow future-proof extensibility, adopt the structure of Figure 3.1-1of the ApplicabilityReport-r19 and its inclusion in UEAssistanceInformation-v1900-IEs and RRCReconfigurationComplete-v1900-IEs, including the reporting for a combination of full inference configurations and sets of inference-related parameters with the ability to indicate the configuration ID type (e.g., CSI-ReportConfigId, InferenceRelatedParametersId) and the configuration ID for which applicability is being reported.

Proposal 6: Do not use the term “functionality” in 38.331 and 38.300.

Proposal 7: Use a field or list of fields only available in AI/ML-enabled configurations, e.g., typeOfBMCase1andBMCase2, to identify configurations which are subject to the applicability determination procedure.

Proposal 8: Outside of field names, there is no need in 38.331 to explicitly specify whether a configuration is for prediction or whether reported measurements are predictions.

Proposal 9: The NW can provide a cause with reconfiguration, e.g., modifying or releasing an inference configuration, indicating that the configuration performed poorly.

Proposal 10: Include in the configuration for data collection configuration candidates, e.g., UE-DataCollectionPrefConfig, an indication whether a data collection configuration candidate is already active, e.g., for other UEs which already indicated the same preference.

Proposal 11: A UE can immediately begin performing measurements on a data collection configuration candidate which is already active and can indicate its preference for an active data collection configuration candidate.

Proposal 12: For beam prediction-Case1 and beam prediction-Case2, Type 1 Option 2 performance monitoring for beam prediction related CSI reporting

- Configure each UE with dedicated reference signals from Set A beams for early feedback of a failure event

- Trigger beam failure event reporting via performance monitoring KPI evaluation

# 7	Text Proposal for 38.300 Applicability Reporting Procedure

Note: The changes in the section below are on top of the latest available version of the running CR [3].

#### X.Y.2.3	Applicability Reporting

For UE side model, the Network provide inference configuration based on UE supported functionalities. The UE can report its applicable functionalities, non-applicable functionalities with relevant cause and their subsequent change to Network. The basic procedure of applicability reporting is described as in Figure X.Y.2.3-1.

Figure X.Y.2.3-1: Initial applicability and applicability status change reporting for Option A

1. The network inquiries about the UE capability information.

2. UE indicates its supported functionalities to the network via UE capability information.

3. The network may provide inference configuration with NW-side additional conditions to UE via CSI report configuration or via otherconfig.

Editor's note: Whether to specify anything for NW-side additional conditions should be up to RAN1.

4. UE determines the applicable AI/ML functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

5. The UE reports its initial functionality applicability in RRCReconfigurationComplete message.

6.  When provided with periodic CSI configuration consistent with reported UE capabilities in CSI report configuration, upon reporting the applicable functionalities, the UE autonomously activates the applicable AI/ML functionalities. When provided with semi-persistent CSI and/or aperiodic CSI configuration, upon reporting the applicable AI/ML functionalities, applicable AI/ML functionality activation follows the CSI measurement and reporting, i.e., semi-persistent reporting can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI.

NOTE 1:  The network may transmit an RRCReconfiguration message to the UE including the inference configuration in case inference configuration is not provided in step 3. Upon receiving one or more inference configuration(s), UE shall maintain all the inference configuration(s) no matter the inference configuration is applicable or non-applicable until the network releases it explicitly.

7. When the network has configured applicability reporting via otherconfig, and applicability of the functionality changes, the UE can report updated functionality applicability and non-applicability with relevant cause in UEAssistanceInformation message. When an activated AI/ML functionality becomes non-applicable, the UE does not autonomously deactivate it, but the UE indicates to the network the change in the applicability. Upon reception of UE indication of the functionality becoming non-applicable, the network should deactivate or release this activated functionality. When an activated applicable AI/ML configuration with periodic CSI configuration becomes inapplicable, the UE reports only the measured beams configured in the inapplicable configuration until the network deactivates or release this activated configuration.

Editor's note: FFS signaling details for option B (e.g., whether it is signaling in CSI-Report Config or otherconfig) and activation.

# 8	References

[1] R1-2410898, “RAN2 Chair’s Notes”, 3GPP TSG RAN WG2 #129, Athens, Greece, February 17 – 21, 2025.

[2] R1-2407554, “RAN2 Chair’s Notes”, 3GPP TSG RAN WG2 #128, Orlando, US, November 18 – 22, 2024.

[3] [POST129][024][AI PHY] Stage 2 running CR (vivo) R2-25xxxxx_Introduction of AI for Air interface feature in 38300