3GPP TSG-RAN WG2 Meeting #130	R2-2504423

St. Julians, Malta, May. 19th – 23rd, 2025

# Agenda item:	8.6.2

# Source:	ETRI

# Title:	Remaining open issues on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#129-bis meeting, agreements on inter-CU LTM design have been made as the followings [1]:

Agreements on inter-CU LTM - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included. - NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG. - From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4. - MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement. - If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA. - An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

In this paper, we discuss remaining open issues regarding inter -CU LTM.

- Discussion

  - NCC value in Enhanced LTM CSC MAC CE

According to the MAC running CR, the Enhanced LTM Cell Switch Command MAC CE includes a NCC value field as an optional field, but there is no indication bit to indicate the presence of the NCC value filed. During the email discussion, some companies raised an issue regarding this problem, and Rapporteur described that the UE can identify whether NCC value is included based on the R19 set ID: NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Although it is possible to decode the optional field in the Enhanced LTM CSC MAC CE based on the R19 set ID information, it cannot be considered an efficient or simple approach. Instead, using one of the following methods would simplify the implementation.

- Introduce a 1-bit field in the enhanced CSC MAC CE to indicate whether a NCC value is included.

- Use the enhanced CSC MAC CE when the UE is required to perform security update. (i.e., a NCC value is always included in the enhanced CSC MAC CE)

The key difference between the legacy CSC MAC CE and the enhanced CSC MAC CE is the presence of the NCC value field. If a 1-bit field is added to the enhanced CSC MAC CE to indicate the presence of the NCC value, there would be no benefits in using a separate MAC CE for R19 LTM. The legacy LTM CSC MAC CE could be reused by simply adding the indication bit and the optional NCC value field.

The simplest approach would be to use the new MAC CE when the R19 set IDs of the target cell and source cell are different, and to use the legacy MAC CE otherwise. Additionally, in the case of intra-cell scenarios where a security update is required, the legacy method can still be used.

Proposal 1: The Enhanced LTM CSC MAC CE always contains an NCC value field

Proposal 2: The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

  - Fast recovery after LTM cell switch failure

Regarding fast recovery after LTM cell switch failure (i.e., due to the expiry of T304), the following 4 cases can be considered.

- Case 1: Intra-CU cell switch failure & the selected candidate cell has the same Rel-19 ID as source cell

- Case 2: Inter-CU cell switch failure & the selected candidate cell has the same Rel-19 ID as target cell

- Case 3: Intra-CU cell switch failure & the selected candidate cell has the different Rel-19 ID from source cell

- Case 4: Inter-CU cell switch failure & the selected candidate cell has the different Rel-19 ID from target cell

The first case corresponds to the R18 LTM fast recovery and has already been agreed to be supported. Case 2 was discussed during the previous RAN2 meeting and was agreed to be supported as follows:

At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - If reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

In Case 3, where the UE selects a candidate cell whose Rel-19 ID is different from that of the source cell, the UE is required to perform a security key update for fast recovery. However, the UE cannot perform the security key update because a new NCC was not included in the LTM CSC MAC CE transmitted by the source cell for intra-CU. As a result, the UE must perform the RRC re-establishment procedure in Case 3.

Case 4 was also discussed during the previous RAN2 meeting, but no conclusion was reached. There are two sub-cases in this case. In the first sub-case, the UE selects a candidate cell whose Rel-19 is different from the target cell but same as source cell. In this case, the UE shall revert back to the source cell configuration and then performs fast LTM recovery with the selected candidate cell without security update. However, the UE cannot revert back to the source cell configuration after PDCP reestablishment and it is aligned with the current RRC running CR. According to the current RRC running CR, the UE is allowed to revert back to the source cell configuration only when the triggered LTM cell switch procedure does not require a security key refresh.

Case 4 was also discussed during the previous RAN2 meeting, but no conclusion was reached. There are two sub-cases within this case. In the first sub-case, the UE selects a candidate cell whose Rel-19 set ID is different from the target cell but the same as the source cell. In this case, the UE shall revert back to the source cell configuration and then perform fast LTM recovery with the selected candidate cell without a security update. However, the UE cannot revert back to the source cell configuration after PDCP re-establishment, which is aligned with the current RRC running CR. According to the current RRC running CR, the UE is allowed to revert back to the source cell configuration only when the triggered LTM cell switch procedure does not require a security key refresh.

5.3.5.8.3 T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution as described in 5.3.5.18.6 and 5.3.5.18.x: 4> if the LTM cell switch execution procedure does not require a security key refresh: 5> revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG; 3> else if the T304 of the MCG expiry is caused by an LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3: 4> revert back to the UE configuration used in the source PCell (i.e., the source Pcell prior to the starting of timer T311) except for the PDCP state variables for SRB(s) associated to the MCG;

In the second sub-case, the UE selects a candidate cell whose Rel-19 set ID is different from both the target cell and the source cell. In this case, the UE does not need to revert back to the source cell configuration but requires an additional NCC value to perform a security key update during fast recovery with the selected candidate cell. However, the Enhanced LTM CSC MAC CE contains only a single NCC value, and that value has already been used for the security key update required for the cell switch to the target cell. As a result, the UE cannot perform the necessary security key update procedure for fast recovery to the source cell. Therefore, in Case 4, the UE should perform the RRC re-establishment procedure rather than fast recovery.

Proposal 3: if the UE selects a candidate cell whose Rel-19 ID is different from the source cell after intra-CU LTM, the UE performs RRC reestablishment procedure. (i.e., the fast recovery is not supported)

Proposal 4: if the UE selects a candidate cell whose Rel-19 ID is different from the target cell after inter-CU LTM, the UE performs RRC reestablishment procedure. (i.e., the fast recovery is not supported)

- Conclusions

In this paper, we discuss CLTM related issue. Following are proposals made in this paper:

Proposal 1: The Enhanced LTM CSC MAC CE always contains an NCC value field

Proposal 2: The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

Proposal 3: if the UE selects a candidate cell whose Rel-19 ID is different from the source cell after intra-CU LTM, the UE performs RRC reestablishment procedure. (i.e., the fast recovery is not supported)

Proposal 4: if the UE selects a candidate cell whose Rel-19 ID is different from the target cell after inter-CU LTM, the UE performs RRC reestablishment procedure. (i.e., the fast recovery is not supported)

- References

- Chair notes RAN2#129-bis

- Running MAC CR for Mob Ph4

- Running RRC CR for inter-CU and conditional LTM