# 3GPP TSG-WG2 Meeting #130                                            R2-2504439

St.Julians, Malta, May 19th – 23rd, 2025

Source: 			CMCC

Title:  		    Discussion on data transmission for A-IoT

Agenda item:	8.2.4

Document for: 	Discussion

Introduction

In RAN2#129bis meeting, we discussed the data transmission for A-IoT and achieved the following agreements.

For AS ID - AS ID is applied for Inventory + command case; - AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed). - For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits. - Do not specify the reader behaviour on how exactly the ASID is generated. - The device releases the AS ID upon power off (no stage 3 specification impact); - The device only keeps one AS ID at a time. - For CFRA, command message is used for AS ID assignment. - For CBRA, Msg 2 is used for AS ID assignment. - The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new Msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely. For segmentation - For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. - FFS whether the reader always includes the command for retransmission of segments. 1-bit indication is sufficient to indicate whether more D2R data will be sent - For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption. PDU format - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message - RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1. - The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message - The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. - In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS.

In this contribution, we will discuss the data transmission functionalities and MAC PDU format of A-IoT.

Discussion

Segmentation

With progress in past meetings, majority opening issues on segmentation have been agreed. We further discuss the remaining issues.

Length of offset

As agreed about the PDUs format, the MAC PDU should be byte-aligned with the assumption that the allocated TBS value is in the unit of byte. The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message. In addition, RAN2 assumes that the upper layer data SDU is byte-aligned. For segment retransmission, reader explicitly indicates an offset in the MAC layer. Considering the maximum data size of D2R is about 1000 bits, So, we propose the length of offset indicator can be 7 bits.

Proposal 1: The length of offset indicator can be 7 bits.

Command message in R2D for retransmission of segment

For the first segment and the unsegmented packet, the reader can not identify the D2R transmission failure is caused by R2D receiving failure or D2R transmission failure. The command message is needed to be included in the R2D message to indicate device the star address. However, for the subsequent segment retransmission, it is obviously that the device has successfully receive the command message and forwards that information to higher layer to store the start address in the VM. The following D2R scheduling message does not need to contain the command message. In addition, considering the message size of command, it will increase the demodulation complexity of the A-IoT device.

Proposal 2: The start address in command message needs to be stored in the VM of device.

Proposal 3: In case subsequent segment retransmission, the command message does not need to be re-contained in the R2D message.

Offset zero for first segment and Unsegment packet

For scheduling the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. From the device’s perspective, it implicitly indicates that the latest R2D transmission, including the first segment or unsegmented packet, is failure. The offset indicator field is not needed, avoiding R2D overhead.

Proposal 4: For the first segment and unsegmented packet retransmission, the “offset” indicator in R2D is not needed.

Offset in D2R

As we discussed and agreed in the SI, sequence-number-like indicator is not considered in A-IoT system. Wait-and-stop mode (1-1 waiting mechanism) is used to reduce the complexity of A-IoT in R19. So, “offset” in D2R message is not needed to indicator the sequence to reader.

Proposal 5: The “offset” indicator is not needed in D2R message.

AS ID

Agreements on AS ID validation been reached. when receiving a Paging message with new transaction ID for that device, or trigger new Msg1 transmission as a result of receiving paging message trigger, or power-off, the device releases its AS ID.

The power-off case is straightforward. The paging message with new transaction ID is addressed to the new A-IoT service request. However, the “new Msg1 transmission as a result of receiving paging message trigger” is no clear description for us. We understand that the intention is to say if the device detects CBRA failure, device will trigger re-access in subsequent access occasion. For the device that detected CBRA failure, it can release the AS ID, no matter there is subsequent paging. So, the condition of release the AS ID should be when the CBRA failure, instead of new Msg1 transmission. In addition, as we proposed in Paging paper, for the CFRA case, the transaction ID is not needed. This case is still not be included in the AS ID validation discussion. We propose to add new condition as when CFRA is triggered targeting this device.

Proposal 6: the AS ID should be released in the following cases:

- Power off (no change)

- Upon receiving paging message with new transaction ID (no change)

- Upon CBRA failure (update)

- Upon receiving paging message of CFRA (new add)

Write operation response

In SA2 LS S2-2501241, the service data response for write command is “operation result indication response for 'Write'”. There may be an uncertain writing time for devices with different kinds of capabilities. Some device may not finish “write” when it has to respond to reader in the D2R resource as indicated in R2D message. Whether and how to respond the write command should be discussed to specify device behaviour.

For the core network, it needs to clearly know whether the write operation has been successfully completed, and there is no need to receive a response message about whether the device has already received the command but has not been successfully written. If the reader does not receive a response in the initial D2R resource assigned for command feedback, it needs to reallocate resources for scheduling the successful write result. Therefore, we believe that at least from the perspective of RAN2, the reader can obtain the results sent by devices with longer processing times through multiple D2R scheduling. There is no need for signaling enhancement between the reader and the core network.

The remaining issue is how the device can respond to the reader when it cannot process the write command in time. We understand that this can be summarized as the device having no data transmission at present, but there will be coming data later. That is to say, MAC PDU can only include padding, but the indication of the remaining data to be transmitted is valid. The network needs to send scheduling information again to allocate D2R resources. Another solution for the device is avoid not responding to this scheduling and wait for the reader to retransmit the command message. Based on implementation, the device completed the last write operation and feedback with the subsequent D2R resource. However, we understand that the second method is not as rigorous as the first one.

Proposal 7: From the perspective of RAN2, the reader can obtain the “write” results from the devices with longer processing times through multiple D2R scheduling.

Proposal 8: If the device has not finished the “write” operation, it feedbacks padding and informs reader about the remaining data.

It needs to be clarified that the format of the aforementioned MAC PDU is different from other data PDU. There may be cases where the remaining data is not zero and padding is included. To avoid the reader misinterpreting, we can restrict the reader to read the padding length field, no matter if there is remaining data.

Proposal 9: The reader shall read the padding length field, no matter if there is remaining data.

PDU format

General

RAN2 has support multiplexing of information for multiple devices in R2D message for msg2. For the subsequent R2D message, e.g., Msg4 containing NACK message, read/write command, there are not conclusion on whether multiplex can be introduced. In our understanding, the NACK message as indication of Msg3 transmission failure only include the AS ID or random ID. The length of a concatenated Msg4 is not too large to bring complexity and latency to the device compared to multiplexing Msg2. So, NACK message for multiple devices can be multiple in one Msg4.

Proposal 10: NACK message for multiple devices can be multiplexed in one Msg4.

However, the subsequent R2D message, for example the command message, especially the write command, may contain large amounts of data. Multiplexing commands from multiple devices into a single R2D data packet can impose significant energy overhead on the devices. Meanwhile, if R2D messages include different types of command messages, it may require a certain header field to indicate the type of command. This brings greater challenges to the design of the R2D format. At the same time, we need to consider whether there are security issues with multiplexing commands for different devices. For that reason, we suggest RAN2 not to consider multiplex subsequent R2D message towards multiple devices within one R2D message except for Msg2 and Msg4.

Proposal 11: RAN2 is asked not to consider multiplex command message of different devices within one R2D message.

D2R format

RAN2 has agree that the MAC PDU should be byte-aligned with assuming the allocated TBS value is in the unit of byte. FFS for R2D trigger message. The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message.

Considering the conclusion of last meeting, only Msg1 and Msg3/data are considered as D2R message type. For Msg1, it contains Random value RN16 for CBRA and acknowledges of paging message for CFRA. The MAC header is not required for reader to identify such D2R message. For Msg3 and data, they are scheduled by the response to different R2D messages. The data transmission is used to acknowledge the command and will not start before Msg3 transmission. So, the message type is not needed for D2R.

The following is an example of A-IoT Msg1:

Fig.1: An example of Msg1 of CBRA

Fig.2: An example of Msg3 of CBRA and first D2R message of CFRA

Proposal 12: The MAC header is not required for Msg1 of CBRA and first D2R message of CFRA.

Proposal 13: Message type is not needed for Msg3 and D2R data.

RAN2 has agreed MAC padding is supported at least for D2R. When the data volume is less than the resources indicated in R2D, padding is used to fill the gap. In most cases, the size of the padding is less than that of the data. It is more concise and efficient to indicate the length of the padding instead of data. But the reader cannot allocate precise resource size, the size of the length field of SDU and padding are always the same. Considering the case that the resources allocated by the reader may be 1000 bits (128 bytes), but the D2R data can still be very small, the length of the padding may still need to be indicated by 7 bits.

The possible cases are listed as following:

- The remaining data indicator is set valid, and if the padding length field is zero, it is addressed to data segmentation, the PDU may not include the padding.

- The remaining data indicator is set valid, and if the padding length field is not zero, it is addressed to write command respond, the PDU may include the padding only.

- The remaining data indicator is set invalid, and the padding length field is not zero, it is addressed to unsegment and last segment packet.

Fig.3: An example of D2R data

Proposal 14: Suggest introducing length field of the padding in D2R message.

Proposal 15: Suggest considering 7 bits length field of the padding.

Proposal 16: RAN2 is suggested to agree the following cases for D2R MAC PDU format:

- The remaining data indicator is set valid, and if the padding length field is zero, it is addressed to data segmentation, the PDU may not include the padding.

- The remaining data indicator is set valid, and if the padding length field is not zero, it is addressed to response to unfinished “write” operation, the PDU may include the padding only.

- The remaining data indicator is set invalid, and the padding length field is not zero, it is addressed to unsegment or last segment packet.

Conclusion

According to the above discussion, the following proposals are given:

Segmentation

Length of offset

Proposal 1: The length of offset indicator can be 7 bits.

Command message in R2D for retransmission of segment

Proposal 2: The start address in command message needs to be stored in the VM of device.

Proposal 3: In case subsequent segment retransmission, the command message does not need to be re-contained in the R2D message.

Offset zero for first segment and Unsegment packet

Proposal 4: For the first segment and unsegmented packet retransmission, the “offset” indicator in R2D is not needed.

Offset in D2R

Proposal 5: The “offset” indicator is not needed in D2R message.

AS ID

Proposal 6: the AS ID should be released in the following cases:

- Power off (no change)

- Upon receiving paging message with new transaction ID (no change)

- Upon CBRA failure (update)

- Upon receiving paging message of CFRA (new add)

Write operation response

Proposal 7: From the perspective of RAN2, the reader can obtain the “write” results from the devices with longer processing times through multiple D2R scheduling.

Proposal 8: If the device has not finished the “write” operation, it feedbacks padding and informs reader about the remaining data.

Proposal 9: The reader shall read the padding length field, no matter if there is remaining data.

PDU format

General

Proposal 10: NACK message for multiple devices can be multiplexed in one Msg4.

Proposal 11: RAN2 is asked not to consider multiplex command message of different devices within one R2D message.

D2R format

Proposal 12: The MAC header is not required for Msg1 of CBRA and first D2R message of CFRA.

Proposal 13: Message type is not needed for Msg3 and D2R data.

Padding

Proposal 14: Suggest introducing length field of the padding in D2R message.

Proposal 15: Suggest considering 7 bits length field of the padding.

Proposal 16: RAN2 is suggested to agree the following cases for D2R MAC PDU format:

- The remaining data indicator is set valid, and if the padding length field is zero, it is addressed to data segmentation, the PDU may not include the padding.

- The remaining data indicator is set valid, and if the padding length field is not zero, it is addressed to response to unfinished “write” operation, the PDU may include the padding only.

- The remaining data indicator is set invalid, and the padding length field is not zero, it is addressed to unsegment or last segment packet.

Reference

[1] RP-243326, New work item: Solutions for Ambient IoT (Internet of Things) in NR

[2] R2-2502717, Further consideration on A-IoT Random access, CMCC