3GPP TSG-RAN WG3 Meeting #130	R2-2504443

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.10.3

Source:	CMCC

Title:	Discussion on SON/MDT for Slicing and NTN

Document for:	Discussion

# 1 Introduction

SON/MDT for Slicing

In previous meeting, RAN3 sent a LS [1] to RAN2 on slice-based cell reselection.

RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario: - A UE supporting slice-based cell reselection may not find any suitable cell\ in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell. RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

In last meeting [2], RAN2 discussed the SON/MDT for slicing and had the following agreements:

FFS if/what RAN2 enhancements we will do to address the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

SON/MDT for NTN

In the incoming LS [3] from RAN3, the following information about the MRO enhancement for location-based CHO in NTN are included:

- Overall Description:

RAN3 discussed the MRO enhancement for location-based CHO in NTN for earth moving scenario. In order to optimize the distance threshold for location-based CHO, RAN3 thinks it is beneficial to support either option provided below.

- Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens.

RAN3 would like to check with RAN2 which option to be adopted in the UE RLF report.

- Actions:

To RAN2 group:

ACTION:	RAN3 kindly asks RAN2 to provide the feedback on the above question.

In this contribution, we intend to discuss the potential SON/MDT enhancements for network slicing and analysis the feasibility of two options provided in the LS for location-based CHO in NTN and provide our views on the suitable solution for resolving this issue.

# 2 Discussion

2.1 SON/MDT for network slicing

For slice-based cell re-selection, the UE shall derive reselection priorities based on the rules specified in TS 38.304 [4], considering the NSAG priority received by NAS and the slice information received in SIB16 or RRCRelease message, e.g. nsag-CellReselectionPriority, sliceAllowedCellListNR, and sliceExcludedCellListNR.

The UE shall derive reselection priorities for slice-based cell reselection according to the following rules: - Frequencies that support at least one prioritized NSAG received from NAS have higher reselection priority than frequencies that support none of the NSAG(s) received from NAS. - Frequencies that support at least one NSAG provided by NAS are prioritised in the order of the NAS-provided priority for the NSAG with highest priority supported on the frequency. - Among the frequencies (one or multiple) that support the highest prioritised NSAG(s) with the same NAS-provided priorities, the frequencies are prioritized in the order of their highest nsag-CellReselectionPriority given for these NSAG(s). If no nsag-CellReselectionPriority is given for a NSAG at a frequency, the lowest priority value is used (i.e, lower than any of the network configured values for these frequencies). - Frequencies that support none of the NSAG(s) provided by NAS are prioritized in the order of their cellReselectionPriority.

Furthermore, if the cell is either listed in the sliceAllowedCellListNR or is not listed in the sliceExcludedCellListNR, or neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured, the UE will consider the cell to support the NSAG.

The UE considers a cell on an NR frequency to support all slices of an NSAG if - the nsag-ID and TA of the NSAG indicated for the NR frequency in dedicated signalling but not in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; or - the nsag-ID and TA of the NSAG indicated for the NR frequency in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; and - the cell is either listed in the sliceAllowedCellListNR (if provided in the sliceInfoList) or the cell is not listed in the sliceExcludedCellListNR (if provided in the sliceInfoList); or - Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the sliceInfoList.

On the other hand, it is also specified that if the highest ranked cell or best cell does not support the NSAG, the UE will re-derive the re-selection priority based the NSAG supported by this cell or legacy cell priority. Then the UE continues to evaluate based on the re-derived re-selection priority, and will select a cell which supports the next priority NSAG within UE.

For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause: - if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency); - Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

On the other hand, if the sliceAllowedCellListNR or sliceExcludedCellListNR is not configured, the slice based cell re-selection may be impacted. For example, the following slice information is broadcast in SIB16, including frequency info, supported NSAG ID, NSAG based cell re-selection priority, slice allowed cell list and slice excluded cell list. It is assumed that the priority of NSAG#1 is higher than NSAG#2 within UE, and the larger number for cell re-selection priority means the higher priority.

| Frequency | NSAG ID | NSAG based cell re-selection priority | Legacy cell re-selection priority | Slice allowed cell list | Slice excluded cell list |
| --- | --- | --- | --- | --- | --- |
| F1 | NSAG#1 | 4 | 3 | Cell#1, Cell#2 | - |
|  | NSAG#2 | 5 |  | - | Cell#4 |
| F2 | NSAG#1 | 6 | 5 | - | - |
| F3 | NSAG#2 | 4 | - | - | - |
| F4 | N/A | - | 4 | - | - |

Based on the slice based cell re-selection mechanism specified in TS 38.304 [3], the UE derives that the reselection priority for the frequency is F2>F1>F3>F4. And the procedure is as follows:

- Firstly, considering the highest NSAG (i.e. NSAG#1), both F1 and F2 support NSAG#1, and the NSAG based cell re-selection priority for F1 and F2 are 4 and 6 respectively, so the priority of F2 is higher than F1, i.e. F2>F1;

- Secondly, considering the next highest priority NSAG (i.e. NSAG#2), both F1 and F3 support NSAG#1, since F1 has been considered in the first step, so the priority of F3 is lower than F1, i.e. F2>F1>F3;

- Lastly, all NSAGs supported by UE have been considered, then the frequency doesn’t support any NSAG will be considered. Since there is only one frequency supports none NSAG (i.e. F4), the priority is lowest, i.e. F2>F1>F3>F4.

Then the UE will evaluate from the highest priority frequency, i.e. F2. For F2, neither slice allowed cell list nor slice excluded cell list is configured, which means all cells on F2 support NSAG#1. The UE will camp on the best cell on F2 (e.g. cell#3).

However, it may happen that after the UE camps on the cell (e.g. cell#3), the UE is aware of the cell#3 does not support the NSAG#1 via SIB16, especially when the UE resides at the boundary of different TA, since different NSAG(s) can be supported in different TA. That is the scenario RAN3 indicated that a UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG (i.e. NSAG#1), thereby UE have to camp on cells belonging to the lower ranked NSAG (i.e. NSAG#2) or eventually use legacy cell reselection procedure to find a suitable cell.

Observation 1: The scenario identified by RAN3 may occur especially when the UE resides at the boundary of different TA, since different NSAG(s) can be supported in different TA.

In this case, when the UE would like to initiate the highest priority NSAG (i.e. NSAG#1), but the camped cell#3 cannot satisfy the requirements on NSAG#1, a handover procedure may be needed to let the UE access a target cell supporting the NSAG#1, which increase the latency and complexity. On the other hand, if only sliceExcludedCellListNR is configured and the highest ranked cell or best cell is not within the sliceExcludedCellListNR, the UE will consider the cell to support the NSAG. However, after the UE camps on the cell, it may find the cell does not support the NSAG via SIB16, which is similar to the above case.

Observation 2: Due to sub-optimal configuration of sliceAllowedCellListNR or sliceExcludedCellListNR in SIB16, the UE may find the best cell or highest ranked cell does not support the highest NSAG after the UE camps on, which may lead to unnecessary handover.

If the UE can log and report this info to the network, the network can optimize the configuration of sliceAllowedCellListNR or sliceExcludedCellListNR, for example, configure the sliceExcludedCellListNR to include cell#3 which means the cell#3 does not support the highest NSAG#1, and other cells on F2 support NSAG#1. With the information, the UE can re-derive the cell re-selection priority and can camp on F1 which also supports the highest NSAG#1.

Observation 3: If the UE can log the NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) and priority information supported by the UE (received from NAS), the network can optimize the configuration of sliceAllowedCellListNR or sliceExcludedCellListNR, e.g. include the highest ranked cell or best cell in sliceExcludedCellListNR, then the UE will camp on the other suitable cell which supports the highest priority NSAG.

Based on the above analysis and observations, we propose the following proposal.

Proposal 1: It is proposed to log the NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) and priority information supported by the UE (received from NAS), which is helpful for NW to optimize the configuration of sliceAllowedCellListNR and/or sliceExcludedCellListNR in SIB16.

2.2 SON/MDT for NTN

For location-based CHO in NTN, there are CondEvent D1 and CondEvent D2 specified for earth fixed case and earth moving case, respectively. If the distance between UE and a reference location/moving reference location for the serving cell becomes larger than configured threshold distanceThreshFromReference1 in CondEvent D1/D2 and the distance between UE and a reference location/moving reference location for the candidate cell becomes shorter than configured threshold distanceThreshFromReference2 in CondEvent D1/D2, CHO execution condition is satisfied and handover is executed.

We could observe that the potential configuration issues of the distance thresholds in trigger conditions would affect HO performance. And we think the distance information between the UE and the moving reference location in the serving cell and in the candidate cell at the time of failure could help to optimize whether the configured distance thresholds may contribute to the failure.

Observation 4: For location-based trigger condition, the distance information between UE and the moving reference location in serving cell and in candidate cell respectively when the failure occurs is beneficial to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

In the LS [3], RAN3 provides two potential solutions on how to obtain the distance between UE and the moving reference location in serving cell and in candidate cell respectively when the failure occurs for earth moving scenario. The related information in either option below would be reported from UE in the RLF report for the location-based CHO in NTN.

- Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens.

For Option 1, we observe that a few information related to the absolute UTC time (e.g., timeInfoUTC in SIB9 or t1-Threshold for CondEventT1 in ReportConfigNR) are specified in TS 38.331. As per the definition of t1-Threshold shown below, we can see that 39 bits may be required to represent this parameter. Therefore, the information related to the absolute UTC time are only provided from Network to UE by now. From our point of view, we should avoid to directly introduce the absolute UTC time in RLF report from UE which would cause UE complexity.

Furthermore, for location-based CHO, we need to obtain distance value between UE and the moving reference location in serving cell and in candidate cell to optimize configuration issues of the distance thresholds in trigger condition. Option 1 is only used to get the time of when RLF/HOF happens. Network still has to perform the additional calculation to derive the distance between UE and the reference location in serving cell and in candidate cell when RLF occurred based on UE location information. For UE location information, we observe that the locationInfo IE in RLF report is optional, that means UE location information may be not available when the failure occurrs and network could not derive the distance between UE and the referenceLocation.

Observation 5: For Option 1 (the absolute UTC time), the following issues need to be considered:

- The absolute UTC time which may be represented by 39 bits are only provided from Network to UE by now.

- Reporting the absolute UTC time when RLF occurred in the RLF report would cause UE complexity.

- Option 1 is only used to get the time of when RLF/HOF happens. The additional calculation is still needed for Network to derive the distance between UE and the reference location in serving cell and in candidate cell when RLF occurred based on UE location information.

- Since the IE locationInfo in RLF report is optional, UE location information may be not available when RLF occurred and network could not derive the distance between UE and the referenceLocation in serving cell and in candidate cell.

For Option 2, it seems a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information. Actually, UE also needs to measure the distance between UE and the moving reference location in serving cell and in candidate cell respectively to evaluate whether execution condition is satisfied for location-based CHO. Therefore, there is no additional complexity for UE to measure the distance between UE and the moving reference location in serving cell and in candidate cell when RLF occurred.

Moreover, as per the definition of distanceThreshFromReference1 and distanceThreshFromReference2 in Event D2, we could roughly get to know that the size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the size of the absolute UTC time IE. The IE size of two measured distances, the distance between UE and the moving reference location in serving cell and in candidate cell respectively, may be represented by 32 bits which is still less than the IE size of the absolute UTC time.

Based on the above analysis, we think Option 2 is a more reasonable solution than Option 1 to get the distance information for location-based CHO with the following reasons:

- Option 2 is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information.

- There is no additional complexity for UE to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell when RLF occurred. Anyway, UE needs to measure the distance between UE and the moving reference location in serving cell and in candidate cell respectively to evaluate whether location-based CHO execution condition is satisfied.

- The IE size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the IE size of the absolute UTC time.

Observation 6: Option 2 is a more reasonable solution to get the distance information for location-based CHO with the following reasons:

- Option 2 is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information.

- There is no additional complexity for UE to measure the distance between UE and the moving reference location in serving cell and in candidate cell when RLF occurred. Anyway, UE needs to measure the distance between UE and the moving reference location in serving cell and in candidate cell respectively to evaluate whether location-based CHO execution condition is satisfied.

- The  IE size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the IE size of the absolute UTC time.

Based on the above comparation, the pros and cons of Option 1 and Option 2 are summarized as below:

|  | Option 1 | Option 2 |
| --- | --- | --- |
| The IE size of reporting information | Big | Small |
| Additional calculation for Network | Yes | No |
| Additional UE complexity | Yes | No |

Proposal 2: RAN2 to select Option 2 in RAN3 LS to optimize the distance threshold for location-based CHO in NTN for earth moving scenario, i.e., to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens in RLF report.

Proposal 3: To inform RAN3 with RAN2 decision in the reply LS.

Based on the above proposals, a draft LS is provided in the appendix for review.

# 3 Conclusion

In this paper, we discuss on the potential SON/MDT enhancements for network slicing and MRO enhancement for location-based CHO in NTN for earth moving scenario, we have the following observations and proposals:

SON/MDT for network slicing

Observation 1: The scenario identified by RAN3 may occur especially when the UE resides at the boundary of different TA, since different NSAG(s) can be supported in different TA.

Observation 2: Due to sub-optimal configuration of sliceAllowedCellListNR or sliceExcludedCellListNR in SIB16, the UE may find the best cell or highest ranked cell does not support the highest NSAG after the UE camps on, which may lead to unnecessary handover.

Observation 3: If the UE can log the NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) and priority information supported by the UE (received from NAS), the network can optimize the configuration of sliceAllowedCellListNR or sliceExcludedCellListNR, e.g. include the highest ranked cell or best cell in sliceExcludedCellListNR, then the UE will camp on the other suitable cell which supports the highest priority NSAG.

Proposal 1: It is proposed to log the NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) and priority information supported by the UE (received from NAS), which is helpful for NW to optimize the configuration of sliceAllowedCellListNR and/or sliceExcludedCellListNR in SIB16.

SON/MDT for NTN

Observation 4: For location-based trigger condition, the distance information between UE and the moving reference location in serving cell and in candidate cell respectively when the failure occurs is beneficial to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Observation 5: For Option 1 (the absolute UTC time), the following issues need to be considered:

- The absolute UTC time which may be represented by 39 bits are only provided from Network to UE by now.

- Reporting the absolute UTC time when RLF occurred in the RLF report would cause UE complexity.

- Option 1 is only used to get the time of when RLF/HOF happens. The additional calculation is still needed for Network to derive the distance between UE and the reference location in serving cell and in candidate cell when RLF occurred based on UE location information.

- Since the IE locationInfo in RLF report is optional, UE location information may be not available when RLF occurred and network could not derive the distance between UE and the referenceLocation in serving cell and in candidate cell.

Observation 6: Option 2 is a more reasonable solution to get the distance information for location-based CHO with the following reasons:

- Option 2 is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information.

- There is no additional complexity for UE to measure the distance between UE and the moving reference location in serving cell and in candidate cell when RLF occurred. Anyway, UE needs to measure the distance between UE and the moving reference location in serving cell and in candidate cell respectively to evaluate whether location-based CHO execution condition is satisfied.

- The  IE size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the IE size of the absolute UTC time.

Proposal 2: RAN2 to select Option 2 in RAN3 LS to optimize the distance threshold for location-based CHO in NTN for earth moving scenario, i.e., to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens in RLF report.

Proposal 3: To inform RAN3 with RAN2 decision in the reply LS.

# Reference

- R3-250914 LS on SON for Network Slicing

- Draft_RAN2_129bis_Meeting_Report_v3

- R2-2503317- R3-252382, LS on SONMDT for NTN, Apr, 2025.

- TS 38.304 NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state

# Appendix

Title:	[Draft] Reply LS on SONMDT for NTN

Response to:	R2-2503317- R3-252382

Release:	Release 19

Work Items:	NR_ENDC_SON_MDT_Ph4-Core

Source:	CMCC

To:	RAN3

CC:

Contact Person:

Name:		          Fang Xie

E-mail Address:	xiefang@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN2 would like to thank RAN3 for the LS on the two options to optimize the distance threshold for location-based CHO in NTN. RAN2 has evaluated the pros and cons of two options, and select below Option 2 for location-based CHO optimization in NTN, RAN2 will continue the work on signalling support for Option 2.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens.

- Actions

To RAN3 group:

ACTION: RAN2 kindly asks RAN3 to take the above information into account for the further work.

- Dates of next TSG RAN WG2 meetings

TSG-RAN WG3 Meeting #131	25-29 August  2025	Bangalore, India

TSG-RAN WG3 Meeting #131bis           13-17 October 2025	Prague , CZ