3GPP TSG-RAN WG2 Meeting #130                                    R2-2504454

St. Julien, Malta, 19th – 23rd May 2025

Agenda Item	: 8.2.3 (Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Contention resolution failure

As agreed in RAN2#126 meeting, the A-IOT device considers the contention resolution as successful when the Msg2 containing the same random number transmitted in Msg1 is received. In other words, if the A-IOT device receives the Msg2 including a random number different from the random number transmitted in Msg1, the A-IOT device should consider the contention resolution as failed, and the A-IOT device performs the re-access procedure.

In order to determine the contention resolution failure, the simple solution is to introduce a new timer for Msg2 reception. However, in the RAN2#129bis meeting, it was discussed whether a new timer for NACK feedback is needed or not, and the conclusion is that the timer for NACK feedback is not needed. Since it is assumed that the A-IOT device always monitors the PRDCH, the A-IOT device should monitor the PRDCH regardless of whether the timer for NACK feedback is running or not.

In addition, in RAN2#128 meeting, it was agreed that “RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID”. Considering this, it may hard to define the duration of the new timer for Msg2 reception. Thus, we propose not to introduce the new timer for Msg2 reception to determine the contention resolution failure.

Proposal 1. A new timer for Msg2 reception is not introduced to determine the contention resolution failure

Then, RAN2 need to discuss how to determine the contention resolution failure without introducing the timer for Msg2 reception.

In our view, there are two cases when the A-IOT device does not receive the Msg2. The first case is that the Msg1 is not successfully transmitted. The second case is that the A-IOT device does not successfully receive the Msg2. For the second case, the reader can indicate the re-access procedure to the A-IOT device by transmitting the NACK feedback. However, for the first case, from the reader point of view, if the reader does not receive any Msg1 after transmitting the access occasion trigger message, the reader considers that any A-IOT device does not transmit the Msg1. In this case, the reader may transmit the next access occasion trigger message. However, there is a case where the A-IOT device can receive the Msg2 with the same random number transmitted in Msg1 after the reception of the new access occasion trigger message.

For example, the A-IOT device#1 transmits the Msg1 containing the random number = 10, but the Msg1 is not transmitted successfully. After that, A-IOT device#1 and #2 receive the access occasion trigger message. The A-IOT device#2 transmits the Msg1 containing the random number = 10. Then, A-IOT device#1 and #2 receive the Msg2 containing random number = 10. In this case, both A-IOT device#1 and #2 consider the contention resolution as successful.

In order to prevent the above case, the A-IOT device considers the contention resolution as failed if the access occasion trigger message is received after transmitting the Msg1. The following figure shows an example of this case.

Proposal 2. If the access occasion trigger message is received after transmitting the Msg1, the A-IOT device considers the contention resolution as failed.

  - Msg1 resource indication

In the RAN1#120bis meeting, the following agreement is made for Msg1 transmission for TDMA.

For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

In addition, since it was agreed that the FDMA is supported for Msg1 transmission in RAN1, the following scenario can be considered for the case where the number of time domain resources, i.e., X value, is 2, and the number of frequency domain resources, i.e., Y value, is 4.

Then, RAN2 need to discuss how to indicate the number of time/frequency domain resources, and we think that there are two options as follows.

- Option 1. The number of time/frequency domain resources are indicated in the access occasion trigger message. It means that the number of time/frequency domain resources can be dynamically changed within the paging round.

- Option 2. The number of time/frequency domain resources are indicated in the A-IOT paging message. It means that the number of time/frequency domain resources is fixed within the paging round

In our view, Option 1 causes a large signalling overhead because the information on the number of time/frequency domain resources should be contained in every access occasion trigger message. In addition, we do not see the motivation to dynamically change the number of time/frequency domain resources within the paging round. Thus, we prefer to go option 2.

Proposal 3. The number of time/frequency domain resources for Msg1 is indicated in the A-IOT paging message in a static manner within the paging round.

  - The time/frequency information in Msg2

In RAN2#129 meeting, it was agreed that the multiple random numbers can be multiplexed in Msg2. Since the multiple A-IOT devices can transmit the Msg1 containing random number in different time/frequency domain, the reader transmits the Msg2 containing the multiple random numbers received in Msg1.

If the Msg2 includes multiple random numbers for multiple A-IOT devices, the A-IOT device needs to know which random number is associated with itself. This is because there is a case where the A-IOT device wrongly determines the contention resolution completion. For example, the A-IOT device#1 transmits the RN=10 in frequency#1, and the device#2 transmits the RN=10 in frequency#2. However, the reader receives the RN=10 on the frequency#1 but does not receive the RN=10 on the frequency#2. Then, if the A-IOT device#1 and A-IOT device#2 receive the Msg2 containing RN=10 without time/frequency information, both A-IOT devices consider the contention resolution successfully completed. In this case, the A-IOT device#2 does not perform the re-access procedure.

Therefore, we think that the time/frequency information should be contained in the Msg2 to prevent the above random number collision case.

Proposal 4. The time/frequency information should be contained in Msg2.

If the above proposal is agreeable, RAN2 discuss how to indicate the time/frequency information.

In order to reduce the signalling overhead in Msg2, each Msg1 resource should be associated with the unique index within the access occasion. To make an association between the Msg1 resource and index within the access occasion, the simple way is to pre-define the index based on the time/frequency domain resources for Msg1. The following figure shows an example to allocate the index for Msg1 resource within the access occasion.

Proposal 5. The index is pre-defined for each Msg1 resource within one access occasion.

If there is an association between the index and the Msg1 resources, it would be better to use the index to indicate the presence/absence of the random number associated with the Msg1 resource. For this, we think that the simple solution is to introduce the bitmap associated with the index in the Msg2, e.g., each bit of the bitmap is associated with an index. The following figure shows an example of the Msg2 format. For example, if the ID0 field is set to 1, the ID0 field indicates that the random number associated with the index0 is contained.

Proposal 6. In Msg2, the bitmap field should be introduced to indicate the presence/absence of the random number.

  - NACK feedback

In the RAN2#129 meeting, the following agreement was made.

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback

As shown above, the NACK feedback is used after the contention resolution is completed, and the NACK feedback is used to indicate the re-access purpose. It means that the reader does not need to transmit the NACK feedback when the Msg3 is received. Thus, the absence of the NACK feedback is considered as a successful transmission of the Msg3.

Proposal 7. The absence of the NACK feedback is considered as successful transmission of the Msg3.

Then, RAN2 need to discuss the NACK feedback format.

In the RAN2#129 meeting, the message type and contents for MAC PDU format were discussed, and the following agreements were made.

- Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

In our view, the NACK feedback should not be piggybacked in the Msg2 because the NACK feedback indicates the re-access procedure by terminating the Msg3 transmission in the current paging round. Thus, since it is assumed that the NACK feedback is always used standalone, a new R2D MAC PDU format for NACK feedback should be defined.

Proposal 8. A new R2D MAC PDU format for NACK feedback should be defined.

During the access procedure, some A-IOT devices can successfully transmit the Msg3 and some A-IOT devices may not successfully transmit Msg3. Thus, the NACK feedback can be indicated per A-IOT device. However, if the NACK feedback is indicated per A-IOT device, the signalling overhead is increased because each NACK feedback should contain the R2D MAC header. Thus, similar to the Msg2, the NACK feedback contains the information on multiple A-IOT devices requiring the re-access procedure to reduce the signalling overhead.

Proposal 9. The NACK feedback should contain the information on at least one A-IOT device requiring the re-access procedure.

Then, RAN2 need to discuss what information should be contained in the NACK feedback. For this, two options were proposed during the email discussion.

- Option 1. The A-IOT device’s AS ID is contained in the NACK feedback.

- Option 2. Only time/frequency information used in Msg1 is contained in NACK feedback.

It was agreed that the NACK feedback is used only for Msg3. It means that the reader indicates the NACK feedback to the A-IOT device if the Msg3 is not received after transmitting the Msg2. If the reader does not receive the Msg3 reception due to the Msg2 failure, the transmission of the AS ID in the NACK feedback is useless because the A-IOT device may not know the AS ID due to the Msg2 failure.

In addition, considering that the maximum number of time resources for Msg1 is 2 and the maximum number of frequency resources for Msg1 is 4, Option 2 requires only 8bit to indicate the NACK feedback. However, since the AS ID is 16bit, the maximum size of the NACK feedback is 128bit which causes the signalling overhead.

Considering above, we think that the Option 2 is better than Option 1.

Proposal 10. Only time/frequency information should be contained in NACK feedback.

The following is an example of the NACK feedback format.

Example of NACK format

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. A new timer for Msg2 reception is not introduced to determine the contention resolution failure

Proposal 2. If the access occasion trigger message is received after transmitting the Msg1, the A-IOT device considers the contention resolution as failed.

Proposal 3. The number of time/frequency domain resources for Msg1 is indicated in the A-IOT paging message in a static manner within the paging round.

Proposal 4. The time/frequency information should be contained in Msg2.

Proposal 5. The index is pre-defined for each Msg1 resource within one access occasion.

Proposal 6. In Msg2, the bitmap field should be introduced to indicate the presence/absence of the random number.

Proposal 7. The absence of the NACK feedback is considered as successful transmission of the Msg3.

Proposal 8. A new R2D MAC PDU format for NACK feedback should be defined.

Proposal 9. The NACK feedback should contain the information on at least one A-IOT device requiring the re-access procedure.

Proposal 10. Only time/frequency information should be contained in NACK feedback.