3GPP TSG-RAN WG2 Meeting #130	R2-2504471

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:   8.5.4

Source:	HONOR

Title:	Discussion on adaptation of common signal/channel transmissions

Document for:	Discussion and decision

1. Introduction

In Rel-19 NES WID [1], the objective of adaptation of common signal/channel transmissions is captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

By RAN2#129bis [2], most of essential issues has been closed. However, there are some open issues to be closed. In this contribution, we discuss the following open issues on paging adaptation and RACH adaptation in time domain according to open issue list:

- Open issues in 38.331:

- DownlinkConfigCommonSIB:

  - Issue3: FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

- RACH adaptation:

  - Issue1: FFS: How to configure additional RACH resource for RACH adaptation in RRC, with consideration of RAN1 input.

- Open issues in MAC:

- Issue2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

- Discussion

In this section, we will discuss the following remaining issues:

- Whether to introduce a separate pei-ConfigBWP

- PEI-O clustering/bundling

- Adjustment of backoff time

- Transmission/retransmission power

- Additional PRACH resources configuration

- Provisions of receiving an indication during the validity duration

- Availability of the legacy PRACH resources

2.1 Paging adaptation in time domain

Whether to introduce a separate pei-ConfigBWP

One remaining issue in the RRC running CR is whether to introduce a separate pei-ConfigBWP for paging adaptation [2]:

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

pei-ConfigBWP contains pei-searchSpace and firstPDCCH-MonitoringOccasionOfPEI-O parameters. A separate pei-ConfigBWP for paging adaptation means introducing a separate pei-searchSpace and a separate firstPDCCH-MonitoringOccasionOfPEI-O for NES UE. However, a separate firstPDCCH-MonitoringOccasionofPEI-O has been introduced for Rel-19 NES UE. In this way, the key issue is whether a separate pei-searchSpace can be introduced. We think it is up to RAN1 to determine whether a separate pei-searchSpace can be introduced, and RAN2 can send a LS to RAN1, if necessary.

Proposal 1: (38.331-DownlinkConfigCommonSIB-Issue3) Whether to introduce a separate pei-ConfigBWP including a separate pei-searchSpace is up to RAN1.

PEI-O clustering/bundling

In the RAN2#128 meeting and RAN2#129 meeting, we have following agreement [3][4]:

Agreements on paging adaptation 2. Introduce value for N=T/32. FFS on T/64, T/128, T/256. Agreements on paging adaptation 1. For N, values smaller than T/32 are not supported.

With the extension of the N value to T/32, the interval between PFs will be increased for NES UEs, resulting in an extended network sleep time. The PEI-O distribution between PFs will become a critical factor affecting NW sleep time. pei-FrameOffset and firstPDCCH-MonitoringOccasionOfPEI-O affect the PEI-O distribution. The current pei-FrameOffset and firstPDCCH-MonitoringOccasionOfPEI-O configurations make the PEI-O scattered distribution throughout the whole paging cycle. This leads NW to wake up frequently between PFs to send PEIs, reducing the continuous sleep time.

Observation 1: NW wakes up at each PEI-O between PFs to send PEIs, reducing the continuous sleep time that should have been extended due to the extended PF interval.

If NW configures PEI-O clustering/bundling in a paging cycle, the frequency of NW wake-up due to sending PEI between PFs can be reduced.

Proposal 2: (38.331-DownlinkConfigCommonSIB-Issue3) PEI-O clustering/bundling of NES UEs can be considered.

2.2	RACH adaptation in time domain

Adjustment of backoff time

When NES UE initiates the access on legacy PRACH resources and receives a BI, it indicates a conflict in accessing these resources. If the nearest resource is additional PRACH resource, there is no need to wait for the backoff time to retransmit Msg1. Therefore, NES UE can adjust the backoff time value received for legacy PRACH resources, e.g., by ignoring or reducing the value of backoff time, to use PRACH resources as soon as possible.

Proposal 3: (MAC-Issue2) If the additional PRACH resources is valid, NES UE can adjust the backoff time value received for legacy PRACH resources, e.g., by ignoring or reducing the value of backoff time.

Transmission/retransmission power

In our understanding, for NES UE, additional PRACH resources and legacy PRACH resources hold equal status. Therefore, there is no need to configure separate initial transmission power and separate power ramping value for additional PRACH resources. When NES UE performs initial access or retransmission on additional PRACH resources, it is reasonable to reuse the legacy initial transmission power preambleReceivedTargetPower and legacy power ramping value powerRampingStep. If the resource type used when retransmitting Msg1 is different from the resource type selected last time, NES UE will not clear the ramping power value of the retransmission and can continue to ramp. Therefore, there is no need to introduce separate power-related parameters for transmission and retransmission for NES UE. That is,  legacy power-related parameters (i.e., preambleReceivedTargetPower and powerRampingStep) can be reused for NES UE.

Proposal 4: (38.331-RACH adaptation-Issue1) Legacy power-related parameters (i.e., preambleReceivedTargetPower and powerRampingStep) can be reused for NES UE.

Additional PRACH resources configuration

RAN1#120-bis meeting has achieved the following agreement [5]:

Agreement For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling.

RAN1 agreed to configure the validity duration of additional PRACH resources via high layer signalling. The validity duration configuration should be in the additional PRACH resources configuration. Additional PRACH resources configuration is under discussion. Additional PRACH resources configuration should be known not only by NES UE in RRC_CONNECTED state, but also by NES UE in RRC_IDLE and RRC_INACTIVE state. Therefore, the additional PRACH resources configuration should be in SIB1.

Proposal 5: (38.331-RACH adaptation-Issue1) The additional PRACH resources configuration should be in SIB1.

Provisions of receiving an indication during the validity duration

In the RAN1#120 meeting, we have the following agreement [6]:

Agreement Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE, - Option 1: SFN of the first PF from the next I-DRX cycle - Option 2: SFN of the first PF from the current I-DRX cycle - Option 3: From the first frame of the first PRACH association period after UE receives the DCI - Option 4: From the first frame of the current SI modification period - Option 5: From the first frame of the next SI modification period

Based on the above agreement, during a validity duration of additional PRACH resources in the NES cell, an NES UE may receive a validity indication for these resources. Since the validity duration of additional PRACH resources has a unified starting reference point, this indication does not imply that the NES UE can use the additional resources within the validity duration this time; rather, it indicates that the NES UE can use the additional PRACH resources from the next starting reference point. That is, the validity indication should indicate the validity from the next starting reference point.

Proposal 6: (MAC-Issue2) The validity indication should indicate the validity from the next starting reference point.

Availability of the legacy PRACH resources

During the validity duration of additional PRACH resources, NES UE can use both additional PRACH resources and legacy PRACH resources, and legacy UE can only use legacy PRACH resources. When the collision on legacy resources is already unacceptable, if NES UE continues to access on legacy PRACH resources, it will lead to an access failure.

Observation 2: When the collision on legacy resources is already unacceptable, if NES UE continues to access on legacy PRACH resources, it will lead to an access failure.

Therefore, during the validity duration of additional PRACH resources, NW can indicate whether the NES UE can use the legacy PRACH resources when the additional PRACH resources is valid. when initial RA and retransmitting Msg1.

Proposal 7: (MAC-Issue2) NW can indicate whether the NES UE can use the legacy PRACH resources when the additional PRACH resources is valid.

- Conclusions

In this contribution, we made the following observations and proposals:

Observations

Observation 1: NW wakes up at each PEI-O between PFs to send PEIs, reducing the continuous sleep time that should have been extended due to the extended PF interval.

Observation 2: When the collision on legacy resources is already unacceptable, if NES UE continues to access on legacy PRACH resources, it will lead to an access failure.

Proposals

- Paging adaptation in time-domain

Proposal 1: (38.331-DownlinkConfigCommonSIB-Issue3) Whether to introduce a separate pei-ConfigBWP including a separate pei-searchSpace is up to RAN1.

Proposal 2: (38.331-DownlinkConfigCommonSIB-Issue3) PEI-O clustering/bundling of NES UEs can be considered.

- RACH adaptation in time-domain

Proposal 3: (MAC-Issue2) If the additional PRACH resources is valid, NES UE can adjust the backoff time value received for legacy PRACH resources, e.g., by ignoring or reducing the value of backoff time.

Proposal 4: (38.331-RACH adaptation-Issue1) Legacy Power-related parameters (i.e., preambleReceivedTargetPower and  powerRampingStep) can be reused for NES UE.

Proposal 5: (38.331-RACH adaptation-Issue1) The additional PRACH resources configuration should be in SIB1.

Proposal 6: (MAC-Issue2) The validity indication should indicate the validity from the next starting reference point.

Proposal 7: (MAC-Issue2) NW can indicate whether the NES UE can use the legacy PRACH resources when the additional PRACH resources is valid.

- References

- RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

- RAN2#129bis, Chair Notes.

- R2-2501462, Running RRC CR for enhancements for network energy efficiency, Ericsson.

- RAN2#129, Chair Notes.

- RAN2#128, Chair Notes.

- RAN1#120bis, Chair Notes.

- RAN1#120, Chair Notes.