3GPP TSG-RAN WG2 Meeting #130	R2-2504473

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion conditional LTM

Document for:	Discussion and decision

1	Introduction

For the conditional LTM, UE evaluates conditions for triggering LTM and execute cell switch upon satisfying the event. RAN2#129 bis meeting come to several agreements as following.

Agreements on C-LTM:

1. Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

2. C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

3. RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

4. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this contribution, we discuss the general concept of the conditional LTM like execution trigger events and the early UL synchronization. Then we propose some solutions/way-forwards accordingly.

- Discussion

Since there is no L1 measurement and event evaluation int the L3-based C-LTM, the beam selection could not be based on the L1 measurement result of the selected beam. In the legacy RACH-based procedure, the UE select the beam above the legacy threshold. Same principle could be applied for L3 based C-LTM.

Proposal 1: (MAC-10) For L3 based C-LTM, follow the legacy beam selection procedure (e.g., UE select the beam above the legacy threshold)

With the early UL synchronization designed in Rel-18, UE could perform the RACH-less LTM on the triggered candidate. To avoid signalling and power overhead resulted from frequent TA acquisition, TA could be maintained in the UE after pre-acquisition before the execution and UE could use the corresponding TA when executing the switch to the candidate if the TA of this candidate is still valid. Furthermore, whether the former received TA could be reused after the cell switch is another question and needs to be aligned between the UE and NW. If the TA could be maintained, the validity of the TA could be monitored based on the legacy mechanism like TAT and DL RSRP threshold.

Proposal 2: In addition to the new timer for candidate’s TA, TA validity could also be monitored based on the legacy DL RSRP solution at the UE side.

During previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization. This enables that the early synchronization could be obtained earlier than the cell switch by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition. The TA of a candidate cell is sent from the serving cell before the execution of the CLTM could enable the RACH-less LTM. However, the L1 measurement is performed by UE and sent to the NW by MR to provide the information to trigger the TA acquisition for the candidate cell with one/several good enough beams which may cause long latency. To increase the possibility of the RACH less, it is beneficial if the UE detects the TA is not valid and directly report the explicit information to the NW.

Proposal 3: UE could request the TA acquisition to the NW when it detects the TA of one candidate is invalid/needed.

3	Conclusions

In this contribution, we discussed the conditional LTM for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: (MAC-10) For L3 based C-LTM, follow the legacy procedure for beam selection.

Proposal 2: In addition to the new timer for candidate’s TA, TA validity could also be monitored based on the legacy DL RSRP solution at the UE side.

Proposal 3: UE could request the TA acquisition to the NW when it detects the TA of one candidate is invalid/needed.

4	References