3GPP RAN WG2 Meeting #130	R2-2504476

# St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.6

Source:	HONOR

Title:	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

In RAN#105 meeting, the following objective of Rel-19 XR work item was agreed [1]:

Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2#129bis meeting, the following agreements on XR rate control have been achieved [2].

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In this contribution, we will further discuss the remaining issues of XR rate control.

2. Discussion

2.1. [MAC-04] Which type of ID should be used to identify a QoS flow

Since QoS flow ID is only unique within a certain PDU session/DRB/Logical channel, the new MAC CE should comprise PDU session ID/DRB ID/LCID to identify the certain QoS flow. From our point of view, DRB ID/LCID is better for AS layer procedure, so PDU session ID should be excluded. A certain QoS flow may be mapped to different logic channels in DC scenario, which increases the complexity for UE to identify QFI. On the contrary, QoS flow ID can only be mapped to one DRB ID, which is straightforward for UE to identify certain QoS flow.

Proposal 1: (MAC-04) Use DRB ID + QFI to identify a QoS flow that need to be throttled.

2.2. [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows

As XR service usually comprise multiple QoS flows, gNB may relieve congestion by decreasing the rate of multiple QoS flows at the same time. Similarly, gNB could indicate multiple QoS flows to increase rate at the same time when congestion is relieved. Consequently, it is flexible for gNB to take multiple QoS flows rate in the XR rate control MAC CE.

Proposal 2: (MAC-05) XR rate control MAC CE can comprise multiple QoS flows.

2.3. [MAC-07] Format of the Rate Control MAC CE

gNB would configure DRB ID and its mapped QoS flow ID sequence in SDAP-Config when UE enters RRC_CONNECT mode. For XR rate control MAC CE design, we suggest using bitmap to indicate which QoS flows need to be throttled, with each bit associating to the index of one QoS flow in SDAP-Config. Compared with carrying QoS flow IDs, MAC CE payload sizes can be significantly reduced in this way. QoS flows corresponding to those bits which set to 1 have the recommended available bitrates.

Proposal 3: (MAC-07) For the MAC CE design, use bitmap to indicate which QoS flows need to be throttled.

Theoretically, the maximum number of the mapped QoS flow is 64, and the corresponding bitmap occupies 8 octets. However, the typical number of the mapped QoS flows is much less than 64. We suggest defining two MAC CEs with different octets of the bitmap field (e.g. one octet, eight octets).

The following figure1 illustrates the rate control MAC CE that has one octet bitmap. Assume that the mapped QoS flow ID are QFI2(with index Q0), QFI3(with index Q1), QFI4(with index Q2), QFI5(with index Q3). gNB needs to throttle QFI3 and QFI5.

Figure 1 XR rate control MAC CE with one octet bitmap field

Proposal 4: (MAC-07) Define two MAC CEs with different octet of the bitmap field (e.g. one octet, eight octets) for XR rate control.

2.4. [MAC-10] Behaviour of bitRateQueryProhibitTimer

As to the granularity of the prohibit timer, we think it is related to number of QoS flows that rate control MAC CE can comprise.

Based on the analysis in section 2.1, we prefer rate control MAC CE can support multiple QoS flows. We suggest configuring prohibit timer in DRB granularity for the following reasons. QoS flows that mapped to the same DRB usually have similar QoS. In our understanding, UE should query all these QoS flows rate for at one time. Furthermore, compare to configure prohibit timer in QoS flow granularity, Configuring prohibit timer in DRB granularity could significantly reduce the frequency of the query.

Proposal 5: (MAC-10) Support to configure prohibit timer in DRB granularity.

3. Conclusions

This contribution provides the following proposals.

Proposal 1: (MAC-04) Use DRB ID + QFI to identify a QoS flow that need to be throttled.

Proposal 2: (MAC-05) XR rate control MAC CE can comprise multiple QoS flows.

Proposal 3: (MAC-07) For the MAC CE design, use bitmap to indicate which QoS flows need to be throttled.

Proposal 4: (MAC-07) Define two MAC CEs with different octet of the bitmap field (e.g. one octet, eight octets) for XR rate control.

Proposal 5: (MAC-10) Support to configure prohibit timer in DRB granularity.

4. References

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #129bis