3GPP TSG-RAN WG2 Meeting #130	R2-2504477

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.4

Source:	HONOR

Title: 	Discussions on the support of broadcast service

Document for:	Discussion and decision

1	Introduction

During the previous meetings for Rel-19 NTN, RAN2 clarified use cases of the broadcast service in NTN system and made some basic agreements during RAN2#129 bis meeting as follows.

Agreements: 1. RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) 2. If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH 3. If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. 4. The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

In this contribution, we would further discuss how to provide the broadcast service via NTN based on the previous agreements.

- Discussion

In the current specification, UE could avoid acquiring MCCH for the local broad cast service as shown in the below. Based on the assumption that the ISA may be quite static, it is also beneficial that the UE use the intended service area list related to one specific broadcast service in the USD to avoid acquiring the MCCH when UE is outside the MBS service area of this MBS broadcast service or to deprioritize the frequency which doesn’t provide its interest MBS service. Furthermore, the NW could also provide the information about the association between the FSAI and intended service areas by the legacy mechanism (i.e., SIB21).

Proposal 1a: (idle/inactive-3) UE could use the intended service area list related to one specific broadcast service in the USD to avoid acquiring the MCCH when UE is outside the MBS service area of this MBS broadcast service.

Proposal 1b: (idle/inactive-3) UE could use the association between the FSAI and intended service areas in SIB21 for cell reselection.

The MBS Interest Indication (MII) is introduced to provide the information about the receiving/interested frequencies/services and the priority related to the MBS. The information provided in the MII may be used to assist the NW to determine the MBS related configuration or the RB management. As shown in the previous local MRB service, the UE could enjoy the service as long as it could receive the data since the service is only provided to all UEs in a geographical area. However, for the intended service area, the UE able to receive the broadcast may be located out of the intended area. Thus, assistance information about the UE’s ISA could be reported along with the MII to assist the NW.

Proposal 2: The MII mechanism could be reused to report the ISA ID of the UE.

Although the issue is more severe in the NTN scenario, the intended area is defined since the NTN cell may be too big to provide MBS service within the proper area. However, since the TN cell is related to the deployment of the gNB and the intended service area is based on the policy provided by the service provider. This means the border of the intended service area may be not aligned with the border of the TN cell. Thus, TN UEs could also utilize the info to understand whether it can obtain the specific MBS service based on its location.

Proposal 3: The MBS intended service area mechanism should not be restricted to neither NTN cells nor NTN-capable UEs.

3	Conclusions

In this contribution, we discussed the issues related to intended service area in NTN. Based on the discussion, the following proposals are concluded:

Proposal 1a: (idle/inactive-3) UE could use the intended service area list related to one specific broadcast service in the USD to avoid acquiring the MCCH when UE is outside the MBS service area of this MBS broadcast service.

Proposal 1b: (idle/inactive-3) UE could use the association between the FSAI and intended service areas in SIB21 for cell reselection.

Proposal 2: The MII mechanism could be reused to report the ISA ID of the UE.

Proposal 3: The MBS intended service area mechanism should not be restricted to neither NTN cells nor NTN-capable UEs.

4	References