3GPP TSG-RAN WG2 Meeting #130		R2-2504493

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:		8.5.3

Source:				ASUSTeK

Title:		Broadcasting status for OD-SIB1 request

Document for: 	Discussion and Decision

- Introduction

In RAN2#127bis meeting, RAN2 agreed to adopt broadcasting status check for OD-SIB1 similar as legacy OD-SIB before sending a OD-SIB1 request:

Agreements on OD-SIB1

…

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

RAN1 discussed related issues for several meetings and they concluded that it would be left up to UE’s implementation to determine a broadcasting status of OD-SIB1.

In this contribution, we discuss issues related to how to finalize the design with respect to above agreement.

- Discussion

Regarding the broadcasting status/state of OD-SIB1 originated from above RAN2 agreement, there has been a very long discussion in RAN1 for several meetings. And the view is very diverse regarding the purpose of RAN2’s agreement and whether signalling support from RAN1 is necessary or it’s purely up to RAN2 to design. After a very long debate, the following is agreed in Athens [1]:

FL Proposal 6-1-9 (from vivo2)

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following:

  - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH

    - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified.

  - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission

    - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified.

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

  - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission.

  - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH

Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy.

Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements

Note: The above cases are for SSB on the sync raster.

In RAN1#112bis, RAN1 further down-select to Alt.3 in the above agreement [3]:

Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following:

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

After above decision, there is no concrete criteria for the UE to determine if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1, which means UE is allowed to request SIB1 without checking SIB1’s broadcasting status. It is actually equivalent to overturn the above agreement. Therefore, it seems proper to further discuss/confirm whether to stick to the original agreement or give it up. One possibility to keep the original agreement is to allow indication of broadcasting status via MIB, an example could be to utilize K_SSB as suggested by Alt.1 of the agreement from RAN1#120. We propose RAN2 to choose one of the alternatives in below proposal to finalize this issue.

Observation 1: Based on the agreement made by RAN1 in RAN1120bis, there is no specified way for UE to determine broadcasting status of SIB1.

Observation 2: RAN2 needs to further confirm whether to conduct a broadcasting status check prior to SIB1 request as agreed in RAN2#127.

Proposal: RAN2 decides one of the following alternatives:

Alt. 1: RAN2 sticks to conduct a broadcasting status check by checking K_SSB in MIB of that cell before requesting SIB1of that cell.

Alt.2: RAN2 reverts the following agreement:

Agreements on OD-SIB1

…

8.	If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Alt. 3: RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1of that cell.

- Conclusion

In this contribution, we discuss issues related to OD-SIB1 broadcasting status check and have the following observations/proposals:

Observation 1: Based on the agreement made by RAN1 in RAN1120bis, there is no specified way for UE to determine broadcasting status of SIB1.

Observation 2: RAN2 needs to further confirm whether to conduct a broadcasting status check prior to SIB1 request as agreed in RAN2#127.

Proposal: RAN2 decides one of the following alternatives:

Alt. 1: RAN2 sticks to conduct a broadcasting status check by checking K_SSB in MIB of that cell before requesting SIB1of that cell.

Alt.2: RAN2 reverts the following agreement:

Agreements on OD-SIB1

…

8.	If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Alt. 3: RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1of that cell.

Reference

- R2-2407571, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung).

- RAN1#120 Chair’s note.

- RAN1#120bis Chair’s note.