3GPP TSG-RAN WG2 Meeting #130	R2-2504494

St. Julians, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.5.4

Source:			ASUSTeK

Title:	Discussion on PRACH adaptation

Document for: 	Discussion and Decision

- Introduction

In RAN2 #129 meeting [1], following agreements are made for RACH adaptation:

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

6.  Will follow legacy mechanism regarding how to select RACH resource.

In this contribution, we would like to further discuss some remaining issues in RACH adaptation.

- Discussion

  - RA-RNTI calculation

In RAN1 #120 meeting [2], it has been agreed that the additional RO is invalid when the additional RO is overlapped with legacy valid RO in both time-domain and frequency-domain.

Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs

However, this agreement does not rule out the situation that the additional RO and legacy RO are overlapped in time-domain and not overlapped in frequency domain. On the other hand, note that the variables in RA-RNTI calculation include the index of the first OFDM symbol of the PRACH occasion (s_id), the index of the first slot of the PRACH occasion in a system frame (t_id), the index of the PRACH occasion in the frequency domain (f_id) and the UL carrier used for Random Access Preamble transmission (ul_carrier_id). That is to say, RA-RNTI collision could occur when the additional RO and legacy RO are overlapped in time-domain.

Observation 1: RA-RNTI collision could occur when the additional RO and legacy RO are overlapped in time-domain.

To solve the issue, there may be some alternatives to be considered.

Option 1: Rely on network implementation.

Option 2: Revise the RA-RNTI formula.

For Option 1, the network could schedule time domain resources to forbit legacy RO and additional RO overlapping. However, such implementation limits the flexibility and reduce the efficiency of resource allocation.

For Option 2, it could be done by either changing the definition of frequency domain index or introducing a new offset to separate the two ROs. Note that when 2-step RA was implemented, an offset was also introduced in the formula to tell the difference between MSGB-RNTI and RA-RNTI (14×80×8×2). Similarly, another offset could be utilized for separating the type of the selected RO. That is,

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2 × 2 × nes_id, where nes_id = 0 for legacy RO, nes_id = 1 for additional RO.

Proposal 1: Revise the RA-RNTI formula for RACH adaptation, e.g., introducing a new offset in RA-RNTI formula to separate the case that whether the selected RO is additional RO or legacy RO.

  - Backoff time differentiation

Another issue is that given the different accessibility of legacy ROs and additional ROs, there may be a lack of efficiency with a single backoff time applied solely. For example, in Figure 1, a legacy UE (UE1) selects a legacy RO (L1), and a NES UE (UE2) selects an additional RO (A1) to perform Msg1 transmission. If both UEs don’t receive RAR corresponding to their RAPID upon the expiry of RAR window, and there is a BI factor included in the received RAR respectively, they select a backoff time respectively. The backoff time is determined from selecting a random value with equal probability between 0 and PREAMBLE_BACKOFF, of which the value is “BI field (in each RAR)” multiplied by “(the same) scaling factor”. The distribution/periodicity of legacy ROs and additional ROs might be different, and thus the opportunity for retransmission when selecting different ROs should vary. If the BI field for UE1 (which selected L1) and the BI field for UE2 (which selected A1) are with the same value, UE1 and UE2 might have similar backoff time, which leads to a possibly same retransmission occasion (L3). Ideally, the backoff time for UE2 could be shorter for the case that additional ROs are denser, since UE2 would have more opportunity (L2, A3, A4) to perform re-access. To avoid the same (long) backoff time, UE2 (accessed to A1) should derive a BI field different from UE1 does.

Figure 1: Illustration of similar backoff issue

Proposal 2: BI field corresponding to the additional RO should have a different value compared with BI field corresponding to the legacy RO.

- Conclusion

Observation 1: RA-RNTI collision could occur when the additional RO and legacy RO are overlapped in time-domain.

Proposal 1: Revise the RA-RNTI formula for RACH adaptation, e.g., introducing a new offset in RA-RNTI formula to separate the case that whether the selected RO is additional RO or legacy RO.

Proposal 2: BI field corresponding to the additional RO should have a different value compared with BI field corresponding to the legacy RO.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

- Final Report of 3GPP TSG RAN WG1 #120 v1.0.0	(Athens, Greece, February 17th – 21st, 2025)