3GPP TSG-RAN WG2 Meeting #130	R2-2504506

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.8.2

Source:	NERCDTV

Title:			  Further consideration on downlink coverage enhancements

WI Code:       NR_NTN_Ph3-Core

Document for:	Discussion and Decision

- Introduction

According to the updated WID for R19 NTN, the objective of downlink coverage enhancements are as follows [1]:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

Further, in the RAN2 129 meeting, it has been agreed for downlink coverage enhancements that [2]:

Agreements: - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - The number of SMTC/gaps a UE needs to consider at any time will not be increased further - From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. - RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). - RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

And in the meeting of RAN2 129bis, the following agreements were made for downlink coverage enhancements [3]:

Agreements: 1. From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits 2. We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring 3. RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). 4. We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) 5. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution, we will further discuss the downlink coverage enhancements including possible SMTC enhancements and access control.

- Discussion

Possible SMTC enhancements

One objective of downlink coverage enhancements for R19 NTN is to define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. And in the RAN2 129 meeting, it was agreed to support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC, and if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient.

Although different SMTCs may be applied for different cells, the UE only needs to consider some of the SMTCs according to its location considering the aspect of power saving. In our point of view, UE may only need to select the SMTCs of some neighbour cells close to the UE and then perform measurements using the SMTCs selected.

And an approach of SSB based SMTC selection is shown as the following figure.

Figure 1 SSB based SMTC selection

Observation 1: UE only needs to consider some of the SMTCs according to its location.

Further, in the last meeting, it was agreed that network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. However, in our opinion, it is not enough only to provide the association between SMTC and location, as UE only needs to perform measurement for active cells.

We think there may be three options to solve this problem:

Option1: Network only provides the association between location and SMTCs, the cells associated to which are in active state at that time.

Option 2: Network provides in the assistance information not only the association between SMTC and location but also the active/inactive states of the cells associated to the SMTCs.

Option 3: Network broadcasts the active/inactive states of the neighbour cells in the serving cell periodically, which is independent of the assistance information.

Considering the three options above, we think option 3 is the most effective, as the active/inactive states of the neighbour cells is not only useful for UE SMTC selection but also necessary for UE cell reselection. Therefore, the active/inactive states of the neighbour cells should be broadcasted in the serving cell periodically.

Proposal 1: When gNB provides the association between location and SMTCs, the active/inactive states of the neighbor cells associated should also be considered.

Cell barring

It was agreed in the RAN2 129bis meeting that RAN2 assumes there is no need to introduce new barring bits from SSB extension point of view and will wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring. We think that legacy UEs should not be barred from accessing the DL CE cells if RAN1 concludes there is no impact on legacy UEs considering the link level enhancements.

Proposal 2: Legacy UEs should not be barred from accessing the DL CE cells if RAN1 concludes there is no impact on legacy UEs considering the link level enhancements.

3. Conclusion

Based on the above discussion, we give the following observation and proposals:

Observation 1: UE only needs to consider some of the SMTCs according to its location.

Proposal 1: When gNB provides the association between location and SMTCs, the active/inactive states of the neighbor cells associated should also be considered.

Proposal 2: Legacy UEs should not be barred from accessing the DL CE cells if RAN1 concludes there is no impact on legacy UEs considering the link level enhancements.

References

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2] RAN2#129 chairman’s notes.

[3] RAN2#129bis chairman’s notes.