3GPP TSG-RAN WG2 Meeting #130	R2-2504510

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on support of TDD mode for IoT-NTN

WID/SID:	IoT_NTN_TDD - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN #106 the work item on NB-IoT TDD NTN mode [1] was agreed with the following objective:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

At RAN2 #129 the following was agreed:

Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

At RAN2 #129bis the following was agreed:

Agreements: 1. Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. 2. In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell. 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes 4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe 6. For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

In this contribution we discuss aspects of TDD NTN mode, which impacts RAN2.

# 2	Discussion

According to the WID and RAN1 discussions the NB-IoT TDD NTN frame structure is as illustrated in Figure 1.

Figure 1 NB-IoT TDD NTN frame structure.

Furthermore, RAN1 has agreed that the UL and DL occasions are paired, i.e. if UE has received scheduling information in DL1 the corresponding UL occasion is UL1.

Agreement (RAN1 #119) Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraint

In general, the sparsity of DL and UL subframes impacts the network capacity, especially when considering the mandatory signalling (NPSS, NSSS, NPBCH, SIB1, SIB31). According to RAN1 conclusion the overhead of transmitting the aforementioned, excluding SIB31, is about 40 % of the anchor carrier.

Conclusion (RAN1 #119) For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits. - When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16: - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier. - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier. - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included. - The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB

Thus the number of subframes, which can carry control- and user-plane data is limited. For example, the NB-IoT PRACH preamble has a high capacity due to the use of 3.75 kHz subcarrier. However, the RAR/Msg2 will require a larger bandwidth and thus it may be challenging for the eNB to respond timely to all UEs if many preambles were transmitted in the preceding UL occasion.

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Based on the RAN1 agreements, if the UE has transmitted in UL1 the UE will have to monitor for a response in DL1. According to the frame structure definition, the time-separation is 3*8.28 ms + 3 guard periods which approximates 25 ms (see also Figure 1).

The WID assumptions define LEO 600 km and 1200 km orbits and thus the RTT is in the range 4-12.88 ms for a regenerative LEO600 deployment and 8-25.76 ms for a transparent LEO600 deployment depending on the UE’s elevation angle to the satellite. This means that the UL-DL occasion gap in some scenarios is significantly longer than the RTT and in other scenarios the RTT and the gap is quite comparable.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

The large UL-DL time separation impacts the RAN2 specification, because some MAC procedures are based on the UE-eNB RTT. For example, the 4-step Random Access procedure utilize the UE-eNB RTT when defining the start of the RAR window after Msg1 transmission and for start of the Contention Resolution timer after Msg3 transmission.

An example is given for the RAR window (TS 36.321), which starts based on X subframes + UE-eNB RTT.

### 5.1.4 Random Access Response reception … If the UE is an NB-IoT UE: - if the random access preamble was transmitted in a non-terrestrial network: - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; … Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT TDD/FDD mode; Preamble format; Number of NPRACH repetitions; X. FDD; 0 or 1; >= 64; 41. FDD; 0 or 1; < 64; 4. FDD; 2; >= 16; 41. FDD; 2; < 16; 4. TDD; Any; Any; 4.

Starting the RAR window based on X (of table 5.1.4-1) and the UE-eNB RTT may result in most of the window occurring during the UL-DL occasions gap. This is problematic from UE energy consumption point of view and also limits the eNB scheduling flexibility, because it may reduce the overlap between the RAR window and the DL subframes in the DL occasion.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

A similar issue also exists for the Contention Resolution timer, because it is started based on the UE-eNB RTT as well.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

Based on this issue we propose that RAN2 discuss how to better align the UE’s PDCCH monitoring, based on the RAR window and Contention Resolution timer, with the DL occasions. We consider a feasible solution is to ensure the RAR window and Contention Resolution timer does not start before the next DL occasion. This can be achieved by adjusting the  “starts at the subframe that contains the end of the last … repetition plus X subframes plus UE-eNB RTT” to “starts at the subframe that contains the end of the last … repetition plus max(X subframes plus UE-eNB RTT, distance from last UL transmission repetition to next DL subframe)”. Using this update, the RAR window and Contention Resolution timer will either start at the first DL subframe of the next DL occasion or start later if the UE-eNB RTT is long.

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Within the 90ms, for one cell, only one pair of UL and DL occasions can be used for the UEs served by the cell. For the other occasions and the gaps between the occasions, it can be regarded as unavailable subframes. For UE in RRC idle state, the UE may however still be able to measure neighbour cells in the subframes, which are unavailable on the serving cell, because the neighbour cell(s) may use different pair(s) of UL and DL.

At RAN4 #114bis it was agreed that cell reselection requirements apply if the UE is provided with the active periods of the neighbour cells configured for the measurements. A similar statement was also discussed for RRC Connected mode measurements.

Agreement: For the cell reselection for TDD NB-IoT in NTN, - The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements. - For earth fixed cell and earth moving cell: - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern] - Further discuss the values if -1dB will be used. Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite

Observation 5: RAN4 has agreed measurement requirements are applicable if the UE is provided with information on the active periods of the neighbour cells configured for the measurements.

Therefore, RAN2 shall discuss how to provide assistance information concerning the neighbor cells in IoT-NTN TDD mode operation. In our understanding, the straight-forward solution is to define the measurements including an indication of the active period/offset of neighbour cells in SIB.

Proposal 2: RAN2 to discuss how to provide assistance information on the active periods of the neighbour cells configured for measurements.

For UE in RRC Connected state, we think one approach can be to base the UE behavior on what was agreed for GNSS measurement gaps for NB-IoT. The stage 2 defines the following:

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS acquisition and resumed when the GNSS acquisition is completed.

Differently from the NOTE, for the NB-IoT TDD NTN operation the neighbor cell measurement can still be performed outside the UL-DL pair of the serving cell, while the other UE AS operations such as RLM and RACH shall be suspended.

Proposal 3: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair.

At RAN2 #129bis it was agreed that when a PUR or SPS resource is in a non-U NB-IoT subframe, the UE postpones the PUR or SPS resource to the next valid UL subframe. This is a good starting point to handle the sparse uplink resource availability for the individual UE, but from the network point of view it may still happen that multiple UEs postpone their semi-static uplink resource (PUR or SPS) to the next valid uplink subframe and therefore there will be resource scarcity and in the worst-case scenario even collisions.

Observation 6: when multiple UEs postpone their PUR/SPS resource to the next valid UL subframe there may be collisions between different UEs.

This postponement issue may be addressed by network implementation, but it will put a severe restriction on how many UEs can be configured with PUR and SPS, because the bandwidth in which the UEs can be multiplexed in the first valid UL subframe is limited. It is worth noting that any UE having a PUR or SPS resource, which occurs within the time between a first UL occasion and the start of a second UL occasion, will all postpone the resource to the first valid UL subframe of the second UL occasion.

Therefore, we propose RAN2 to discuss alternative solutions to handle the PUR and SPS postponement issue. We think one useful way forward can be to define that some PUR or SPS resources are not postponed to the next valid UL subframe but further offset to another valid UL subframe.

Proposal 4: RAN2 to discuss handling of the PUR & SPS postponement including postponement to another valid UL subframe to avoid UL collision between different UEs.

At RAN1 #120bis the following was agreed:

Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

Thus the transmission of SIB1-NB will be dropped according to the RAN1 agreement, but the handling of other SIBs is still unclear. Since the SIB1-NB (and NPSS/NSSS/NPBCH) is dropped one option is also to drop the other SIBs, while alternative solutions can be to postpone either the actual SI transmission or the start of the SI window, or to define a SI repetition pattern or periodicity which is aligned with the TDD frame structure.

To align with the SIB1-NB behavior one straight forward solution is to drop other SIB-NB transmissions as well, but it may be difficult to facilitate the SI transmission if the repetition pattern is not possible to align with the valid DL subframes.

Proposal 5: RAN2 to discuss how to provide SI other than SIB1-NB, for example by use of dropping or postpone NPDSCH.

If the NPDSCH carrying a specific SIB-NB is defined to be postponed, the postponement may cause the SIB-NB to appear within a different SI-window (i.e. not the SI-window where the SI message containing the SIB-NB belongs).

In our view, this SI scheduling issue can be addressed by network implementation, because the network can make sure not to schedule NPDSCH, subject to postponement, carrying a SI message close to the SI-window end. As defined in TS 36.331 the network configures the SI message scheduling SIB1-NB.

#### 5.2.1.2a Scheduling for NB-IoT The SI messages are transmitted within periodically occurring time domain windows (referred to as SI-windows) using scheduling information provided in SystemInformationBlockType1-NB. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI is transmitted. The length of the SI-window is common for all SI messages, and is configurable. Within the SI-window, the corresponding SI message can be transmitted a number of times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS.The UE acquires the detailed time/frequency domain scheduling information and other information, e.g. used transport format for the SI messages from schedulingInfoList field in SystemInformationBlockType1-NB. The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition. SystemInformationBlockType1-NB configures the SI-window length and the transmission periodicity for all SI messages.

If needed, RAN2 can make a note to define SI messages of one SI-window do not get postponed to the next SI-window even if the NPDSCH would be subject to postponement.

Proposal 6: leave it to network implementation to ensure NPDSCH carrying a SI message is not postponed appearing in a next SI window (i.e. SI window for a different SI message) if NPDSCH carrying SIB-NB is postponed.

Regarding use of C-DRX in NB-IoT TDD NTN mode, the TS 36.321 defines the On Duration timer starts as follows:

- If the Short DRX Cycle is used and [(SFN * 10) + subframe number] modulo (shortDRX-Cycle) = (drxStartOffset) modulo (shortDRX-Cycle); or - if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

The TS 36.331 defines the values of the DRX cycle, start offset and On Duration Timer

DRX-Config-NB-r13 ::= CHOICE { release NULL, setup SEQUENCE { onDurationTimer-r13 ENUMERATED { pp1, pp2, pp3, pp4, pp8, pp16, pp32, spare}, drx-InactivityTimer-r13 ENUMERATED { pp0, pp1, pp2, pp3, pp4, pp8, pp16, pp32}, drx-RetransmissionTimer-r13 ENUMERATED { pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24, pp33, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, drx-Cycle-r13 ENUMERATED { sf256, sf512, sf1024, sf1536, sf2048, sf3072, sf4096, sf4608, sf6144, sf7680, sf8192, sf9216, spare4, spare3, spare2, spare1}, drx-StartOffset-r13 INTEGER (0..255), drx-ULRetransmissionTimer-r13 ENUMERATED { pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24, pp33, pp40, pp64, pp80, pp96, pp112, pp128, pp160, pp320} } }

As is evident from the legacy specification, the start offset (value is in number of sub-frames by step of (0…255) may be adjusted to a first set of DL subframes. However, due to the DRX cycle being a multiple of 256 subframes most subsequent On Durations will not be well aligned with the DL occasions of the TDD frame.

One possible way forward, which RAN2 can discuss, is to define one of the spare drx-Cycle values to be equal to the 90 ms TDD frame periodicity or a scaled value (x*90 ms).

Proposal 7: RAN2 to introduce a drx-Cycle value which is aligned with the 90 ms TDD frame periodicity.

# 3	Conclusion

This document has made the following observations:

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

Observation 5: RAN4 has agreed measurement requirements are applicable if the UE is provided with information on the active periods of the neighbour cells configured for the measurements.

Observation 6: when multiple UEs postpone their PUR/SPS resource to the next valid UL subframe there may be collisions between different UEs.

And proposed the following:

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Proposal 2: RAN2 to discuss how to provide assistance information on the active periods of the neighbour cells configured for measurements.

Proposal 3: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair.

Proposal 4: RAN2 to discuss handling of the PUR & SPS postponement including postponement to another valid UL subframe to avoid UL collision between different UEs.

Proposal 5: RAN2 to discuss how to provide SI other than SIB1-NB, for example by use of dropping or postpone NPDSCH.

Proposal 6: leave it to network implementation to ensure NPDSCH carrying a SI message is not postponed appearing in a next SI window (i.e. SI window for a different SI message) if NPDSCH carrying SIB-NB is postponed.

Proposal 7: RAN2 to introduce a drx-Cycle value which is aligned with the 90 ms TDD frame periodicity.

# Reference

- RP-243293, Iridum, “Revised WID on introduction of IoT-NTN TDD mode”, December 2024